# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513610

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	4.15.1

Source:	Apple

Title:	On core part maintenance for RRM phase 5

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1][2] has been agreed in RAN4#116, and the open issues have been summarized in the topic summary for [116][211] NR_RRM_Ph5_Part1 and [116][212] NR_RRM_Ph5_Part2 [3][4].

In this contribution, we discuss the core part maintenance for R19 RRM phase 5 WI.

# Discussion on CSSF enhancement

Issue 1-2-1: applicability limitation for solution 3 (3 searchers)

Agreement: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG. FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

Technically, we assume the searcher is just a resource on the buffer for measurement, and it shall not be influenced by which kind of gap-less measurement is being performing, e.g., regular intra-frequency measurement without MG or inter-frequency measurement without MG. It means that the 3 searchers can be applied to inter-frequency measurement without MG. However, inter-frequency measurement without MG is also optional feature at UE, and it could be R16 capability or R17 capability. Then, it would be very complicated that such CSSF enhancement can apply based on the combination of the capabilities of R16/R17(no gap) and R19 (3 searcher). With that, to make this feature simple and more practical, we prefer to not consider inter-frequency measurement without MG in 3 searchers case.

Proposal 1: RAN4 to not apply the enhanced CSSFoutside_gap,i scaling factor for the inter-frequency MOs without MG that are being measured outside of MG.

# Discussion on FBS

There are two open issues as captured in the WF [2]:

### Issue 1-1-4: Requirements when UE reports different BSF values for different bands Way forward: Further discuss this issue in the maintenance stage: - Option 1: - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured. ### Issue 1-1-5: Revision of the agreement on applicability requirement Agreement in Tuesday ad-hoc: - The FBS enhancement applies for the case when the target and serving carrier frequency are same. - FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different. Agreement in Thursday online: When the target and serving carrier frequency are different, - FBS requirement does not apply to inter-frequency measurement without measurement gap. - FFS inter-frequency measurement with per-FR measurement gap. - Note: inter-RAT is not in the scope.

Recall that the requirement applicability is defined as follows:

The requirements related to the support of Rel-19 L3 fast Rx beam sweeping are applicable when - For intra-frequency measurements, the carrier configured for L3 SSB measurement in the FR2-1 band is the serving carrier, and - For inter-frequency measurements, in addition to the serving carrier, at most one carrier in the FR2-1 band is configured for L3 SSB measurement, and - UE is not configured with CA or DC, and - L3 fast Rx beam sweeping operation is activated.

This means besides the serving carrier there is at most one target carrier which is not in the same band as the serving carrier. Considering the core part was completed at the last RAN4 meeting and we are now in the maintenance phase, we propose to have the following simple resolution of both open issues:

- When the target and serving carrier frequency are different, FBS applies to target carrier only.

- In the requirements, the applicable BSF is determined based on the band of the target carrier to be measured.

- Since UE is not configured with CA or DC, per-FR measurement gap can be handled in the same way as for per-UE measurement gap. In other words, no special consideration is needed.

In future releases, if the following two cases will be considered, we can revisit the agreement.

- There is more than one target carrier to be measured, and they are in different bands.

- UE is configured with CA or DC.

Proposal 2: It is proposed to resolve the open issues with the following agreements

- When the target and serving carrier frequency are different, FBS applies to target carrier only.

- In the requirements, the applicable BSF is determined based on the band of the target carrier to be measured.

- Since UE is not configured with CA or DC, per-FR measurement gap can be handled in the same way as for per-UE measurement gap. In other words, no special consideration is needed.

# Conclusion

In this contribution, we discuss the core part maintenance for R19 RRM phase 5 WI.

Proposal 1: RAN4 to not apply the enhanced CSSFoutside_gap,i scaling factor for the inter-frequency MOs without MG that are being measured outside of MG.

Proposal 2: It is proposed to resolve the open issues with the following agreements

- When the target and serving carrier frequency are different, FBS applies to target carrier only.

- In the requirements, the applicable BSF is determined based on the band of the target carrier to be measured.

- Since UE is not configured with CA or DC, per-FR measurement gap can be handled in the same way as for per-UE measurement gap. In other words, no special consideration is needed.

# References

[1] R4-2512124, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#116

[2] R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, Apple, RAN4#116

[3] R4-2509054, Topic summary for [116][211] NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#116

[3] R4-2509055, Topic summary for [116][212] NR_RRM_Ph5_Part2, Moderator (Apple), RAN4#116