3GPP TSG-RAN WG4 Meeting #116bis									            R4-2513616

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	6.2.2

Source:	Apple

Title:	On RRM performance for type 4 UE

Document for:	Discussion

# Introduction

RAN4 completed the RRM requirements for non-collocated FR1 inter-band non-collocated EN-DC and intra-band non-collocated NR-CA in RAN4#116 meeting [1]. It’s time to start discussion on RRM performance work for this feature.

This contribution presents our views on RRM test case coverage for non-collocated FR1 intra-band EN-DC/NR-CA could be addressed.

# Discussion

### 2.1 Intra-band non-collocated CA

According to the agreed CR for intra-band non-collocated CA for type 4 UE [1], the following RRM requirements are introduced in 38.133 for UE indicating support of intraBandNR-CA-non-collocated-r19 (denoted as CA type 4 UE in the following discussion). We will analyse the necessity of introducing test cases for each of the following requirements.

- MRTD/MTTD requirements

- Interruptions at SCell addition/release/activation/deactivation and interruptions during measurements on deactivated SCC.

- SCell activation delay requirements

- SCell BFD requirements

- Scheduling availability

MRTD/MTTD requirement

MRTD/MTTD requirements are only related to deployment performance rather than purely rely on UE performance. It can usually be guaranteed by proper network deployment design. Contribution of the UE performance part has been verified in other UE test cases. As a result in the current 381.33, there has never been test cases for these requirements. Follow the same logic, there is no need to test MRTD/MTTD for UE indicating intraBandNR-CA-non-collocated-r19.

Proposal 1: No need to define MRTD/MTTD test cases for CA type 4 UE.

Interruptions at SCell addition/release/activation/deactivation and interruptions during measurements on deactivated SCC.

For type 4 UE, since separate RF chain is used to handle the Rx signal for each CC, it behaves like an inter-band CA UE when receiving signals from 2CC in the same band. For type 4 UE, the legacy FR1 inter-band CA interruption requirements apply for interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC.

For test case on interruptions during measurement on deactivated NR SCC as specified in A.6.5.2.1. the test setting is generic enough and supporting to configure the NR SCC either in the same band (intra-band) or in another band (inter-band). Test requirement now only cover following 2 cases:

- NR PCell is not in the same band as the deactivated SCell

- NR PCell is in the same band as the deactivated SCell

When it comes to testing for type 4 UE, the test settings can be reused. Test requirement needs to be updated.

Observation 1: For interruptions during measurement on deactivated NR SCC, the test setting in A.6.5.2.1 can be reused. Test requirements need to be updated to cover CA type 4 UE in intra-band non-collocated CA case.

Proposal 2: it is proposed to update the test requirements in A.6.5.2.1 to cover testing for CA type 4 UE.

SCell activation delay requirements

The existing SCell activation delay requirements in clause 8.3.2 apply if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA. For SCell activation/deactivation delay requirements, several test cases have been introduced for different NR CA scenarios. The current SCell activation/deactivation delay requirements in FR1 are verified separately for known SCell and unknown SCell cases and there is no limitation on CA band combinations. Hence, there is no need to introduce additional test case for FR1 intra-band non-collocated CA scenario.

Proposal 3: No need to introduce separate test case for SCell activation delay for CA type 4 UE.

SCell BFD requirements

It has been agreed [1] that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers.

Existing test cases for BFD/CBD requirements are specified in section A.6.5.5. The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. There are some test cases configured in CA scenario. However, the test purpose is to verify the link recovery function on SCell and there is also no limitation on CA band combination. So, no new test case needs to be introduced for FR1 intra-band non-collocated CA scenario.

Observation 2: No need to consider BFD/CBD test as existing test case already tested it from functionality perspective and there is no limitation on CA combinations.

Scheduling availability

Follow the same logic as for the existing scheduling availability, there is no need to test this requirement for intra-band non-collocated CA.

Proposal 4: No need to introduce test cases for Scheduling availability for CA type 4 UE.

### 2.2 Inter-band non-collocated EN-DC with overlapping DL frequency

MRTD/MTTD requirement and interruption requirements are agreed for inter-band non-collocated EN-DC with overlapping DL frequency. Similar consideration on test coverage as that for intra-band non-collocated CA apply.

Proposal 5: Follow similar approach for testing of EN-DC type 4 UE as for CA type 4 UE.

# Requirement update

For test case on interruptions during measurement on deactivated NR SCC as specified in A.6.5.2.1. the test setting is generic enough and supporting to configure the NR SCC either in the same band (intra-band) or in another band (inter-band). Test requirement now only cover following 2 cases:

- NR PCell is not in the same band as the deactivated SCell

- NR PCell is in the same band as the deactivated SCell

When it comes to testing for type 4 UE, the test settings can be reused. Test requirement needs to be updated.

Proposal 6: For interruptions during measurement on deactivated NR SCC, the test setting in A.6.5.2.1 can be reused. Test requirements need to be updated to cover CA type 4 UE in intra-band non-collocated CA case. Below provided how to modify the requirement description.

-----------------------------------------------------Start of changed section-----------------------------------------------------------

##### A.6.5.2.1.2	Test Requirements

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5 % of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell,

- 1> the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-1.

- 1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if UE supporting requirementTypeIndication-r19 is capable of interBandMRDC-WithOverlapDL-Bands-r19 on this band pair

3> if nonCollocatedTypeMRDC-v1900 is presented with “type 4”

4>	Each interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-2.

3> else if nonCollocatedTypeMRDC-v1900 is presented with “type 1”

4> Each interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-1.

3> else if nonCollocatedTypeMRDC-v1900 is not provided

4> Each interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-1.

- 1>	when UE is configured with maxMIMO-Layers with value equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if nonCollocatedTypeMRDC-r18 is not provided

3> Each interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-2.

2> else

3> Each interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-1.

If the NR PCell is non-contiguous to the deactivated SCell in the same band and UE is capable of intraBandNRCA-NonCollocated-r18 on this FR1 band and nonCollocatedTypeNR-CA-r18 is not provided, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-1.

If the NR PCell is non-contiguous to the deactivated SCell in the same band, when UE is not capable of intraBandNRCA-NonCollocated-r18 or when UE is capable ofintraBandNRCA-NonCollocated-r18 and provided with nonCollocatedTypeNR-CA-r18, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. The interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-2.

If the NR PCell is contiguous to the deactivated SCell in the same band, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. The interruption on NR PCell shall not exceed the value defined in table A.6.5.2.1.2-2.

-----------------------------------------------------------End of changed section--------------------------------------------------------

# Conclusion

In this contribution, we provide initial discussion on performance for performance part for type 4 UE. After discussion, the following conclusions are provided:

Proposal 1: No need to define MRTD/MTTD test cases for CA type 4 UE.

Observation 1: For interruptions during measurement on deactivated NR SCC, the test setting in A.6.5.2.1 can be reused. Test requirements need to be updated to cover CA type 4 UE in intra-band non-collocated CA case.

Proposal 2: it is proposed to update the test requirements in A.6.5.2.1 to cover testing for CA type 4 UE.

Proposal 3: No need to introduce separate test case for SCell activation delay for CA type 4 UE.

Observation 2: No need to consider BFD/CBD test as existing test case already tested it from functionality

perspective and there is no limitation on CA combinations.

Proposal 4: No need to introduce test cases for Scheduling availability for CA type 4 UE.

Proposal 5: Follow similar approach for testing of EN-DC type 4 UE as for CA type 4 UE.

Proposal 6: For interruptions during measurement on deactivated NR SCC, the test setting in A.6.5.2.1 can be reused. Test requirements need to be updated to cover CA type 4 UE in intra-band non-collocated CA case. Below provided how to modify the requirement description.

# References

- R4-2512350,	Draft big CR on RRM requirements for Support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2, Huawei, RAN4#116bis

- 38.133 18.3.0