3GPP TSG-RAN WG4 Meeting #116bis												R4-2513618

Prague, Czech, October 13th -17th, 2025

Source: 	Apple

Title: 	On R20 Ambient IOT co-existence

Agenda Item:	7.11.3

Document for:	Discussion

# Introduction

SI on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices was approved at the RAN#108 meeting and further updated in RAN#109 meeting [1]. The general objective and RAN4 related objectives in the SID are copied as below:

Objectives

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]:

- Deployment scenario 4 with topology 1 with no external carrier wave.

- Traffic types DO-DTT, DT, DO-A.

- Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command).

- Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.

- FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

- Assumption of the same air interface for Device 2b and Device C.

  - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

- Positioning for outdoor scenarios is under objective 2.

RAN1-led

……

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

This contribution will give our consideration on the coexistence issue for Ambient IOT in R20.

# Discussion

## Coexistence scenarios

Per the objective in SID, Deployment scenario 4 with topology 1 with no external carrier wave will be considered, which means the Ambient IoT device directly and bidirectionally communicates with a A-IOT BS reader in outdoor assuming that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BS. Also, the objective also mentioned that the deployment considers FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. Per conclusion in [2], RAN is recommended to down-select to one or more of spectrum in-band to NR, in guard-band to LTE/NR and in standalone bands. As such, there could be both NR mode and LTE mode for in guard-band operation in such study. Considering that LTE and NR system parameters are quite similar, it is proposed to prioritize co-existence between A-IOT and NR over co-existence between A-IOT and LTE. A common channel bandwidth between LTE and NR can be used in the simulation. Companies can decide on their own whether to submit simulation results for A-IOT and LTE as well.

Proposal 1: RAN4 to prioritize co-existence between A-IOT and NR over co-existence between A-IOT and LTE.

It is proposed the simulation cases in Table 2.1-1are used for evaluation of coexistence with the following consideration

- Same set of simulation cases are considered for both device 2b and device C.

- For standalone operation, both co-located deployment and non-collocated deployment are in the scope. RAN4 could prioritize non-collocated as worst case.

- Only co-located deployment is considered for in band operation.

Since the power of A-IOT device is much lower than a NR UE, it is anticipated that case 1 will be considered as low risk from co-existence perspective.

Table 2.1-1: Co-existence simulation cases for device 2b/C (standalone& in band operation)

| Case# | Aggressor | Victim | Direction | Priority |
| --- | --- | --- | --- | --- |
| 1 | Device | NR UL | D2R | Low |
| 2 | NR UL | reader | D2R | High |
| 3 | Reader | NR DL | R2D | High (standalone only) |
| 4 | NR DL | device | R2D | High |

Proposal 2: It is proposed to consider the simulation case in table 1-1 for both standalone and in band operation for A-IOT co-existence study in R20.

## Simulation assumptions

Table 2.2-1 simulation parameters (D4T1)

| Parameter | BS/reader | UE/device |
| --- | --- | --- |
|  | Common parameter |  |
| Carrier frequency | 900 MHz |  |
| NR BS/A-IoT BS deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, the minimum 2D distance between macro-BS and device 2b/C as 50-500 m. |  |
| Inter-site distance | 750m |  |
| MCL | 70dB |  |
| Pathloss model | UMa NLOS defined in TR38.901 |  |
| Penetration loss: | For both R2D and D2R - X = {0, 20, 80} dB Note: Example use cases for the values above: - 20 dB for a Device inside a car with metallized car windows - 80 dB for a Device inside a metallic shipping container To be down selected in RAN4 further discussion. |  |
| Shadow fading margin | 6dB |  |
|  | Parameter for NR |  |
| BW and SCS for NR | 5 or 10MHz with 15KHz SCS |  |
| Power control | No power control in DL. Open loop power control as in 36.942. |  |
| Frequency reuse | 1 |  |
| UE distribution |  | Uniformly distributed in the cell - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| Tx power | 46dBm | 23 |
| Minimum output power | / | -40dBm |
| Antenna pattern | See section 2.3 | Omni-directional |
| Antenna gain | See section 2.3 | 0dBi |
| Antenna height | 25m | 1.5m |
| BS |  |  |
| Noise figure | 5 | 9 |
| ACLR | 45 | 30 |
| ACS | 46 | 33 |
|  | Parameter for A-IOT |  |
| R2D BW and SCS | 180kHz, for 15kHz SCS |  |
| D2R BW | 15kHz |  |
| Waveform | R2D: OFDM-based OOK D2R: BPSK |  |
| power control | No PC in DL. Open loop power control in UL (pending RAN1 further agreement) |  |
| Frequency reuse | 1:3 |  |
| Device distribution |  | Uniformly distributed in the cell. Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) |
| Tx power | standalone: 43 dBm in-band：38dBm | -10dBm for device 2b 5dBm for device C |
| Antenna configuration | Same as legacy NR BS | Omni-directional |
| Antenna gain | 17dBi | 0dBi per branch for both device 2b and device C |
| Noise figure | 5 | 15dB for device 2b 13dB for device C |

Proposal 3: it is proposed to adopt the simulation parameters captured in Table 2.2-1.

## BS antenna pattern

It is assumed that the NR BS point at fixed vertical direction: θtilt + 90°

Antenna pattern (horizontal) (For 3-sector cell sites s):

AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB

Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns):

AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees

Proposal 4: it is proposed to adopt the BS antenna pattern in this section (2.3).

## Evaluation methodology and performance metric

Evaluation methodology will reuse those from TR38.769 [2].

On performance metric,

- The performance metric for NR system impacted by ambient IOT is 5% throughput loss for both average and at cell edge as performance metric as legacy.

- The performance metric for ambient IOT impacted by NR system is SINR degradation for inter-system interference impact analysis.

Proposal 5: he performance metric for NR system impacted by ambient IOT is 5% throughput loss for both average and at cell edge as performance metric as legacy.

Proposal 6: The performance metric for ambient IOT impacted by NR system is SINR degradation for inter-system interference impact analysis.

# Conclusion

This contribution presented our consideration on co-existence between NR and A-IOT and the following proposals are concluded.

Proposal 1: RAN4 to prioritize co-existence between A-IOT and NR over co-existence between A-IOT and LTE.

Proposal 2: It is proposed to consider the simulation case in table 1-1 for both standalone and in band operation for A-IOT co-existence study in R20.

Proposal 3: it is proposed to adopt the simulation parameters captured in Table 2.2-1.

Proposal 4: it is proposed to adopt the BS antenna pattern in this section (2.3).

Proposal 5: he performance metric for NR system impacted by ambient IOT is 5% throughput loss for both average and at cell edge as performance metric as legacy.

Proposal 6: The performance metric for ambient IOT impacted by NR system is SINR degradation for inter-system interference impact analysis.

# Reference

[1] RP-252964, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, Moderator, RAN#108.

[2]TR38.769, Study on solutions for ambient IoT (Internet of Things).