# 3GPP TSG-RAN WG4 Meeting #116bis	                                                R4-2513624

# Prague, CZ, 13 – 17 October, 2025

Agenda Item:	6.10.3

Source: 	Apple

Title:	Discussion of RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

Performance requirements for R19 Fast SCell activation for UE supporting Rel-18 EMR was initially discussed in previous RAN4 meeting. Due to limited time, no any agreement has been made yet. In this contribution, we will continue discussion on all open issues.

# Discussion

Issue 2-2-1: Scope of test cases

Agreement in RAN4#115: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

- Proposals

  - Direct SCell activation at SCell addition for FR2

    - Consider this scenario: CATT, ZTE, vivo (Configuration 1 and 2), Huawei, Ericsson, Nokia

    - Do not consider this scenario: OPPO

  - Configuration 3

    - Consider this configuration: CATT, OPPO, CMCC, ZTE, vivo, Huawei, Ericsson, Nokia

    - Do not consider this configuration: Apple

- Recommended WF

  - Based on the majority view, define test cases for

    - Direct SCell activation at SCell addition for FR2

    - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode

We think the recommended WF from moderator is a very good starting point. We support to introduce test cases for all three scenarios and both FR1 and FR2, considering:

- the new core requirements introduced in this objective cover three scenarios, i.e. normal SCell activation, direct SCell activation and PUCCH SCell activation, and both FR1 and FR2.

- UE may only support some of the scenarios. Thus RAN4 needs to guarantee test coverage.

Regarding the configuration 3, we fail to understand why this is necessary. It is just a R16 scenario. Procedure wise RAN4 should not introduce a R19 test case to verify a R16 procedure in a R19 WI.

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode

Proposal 1: not introduce the following configuration 3, since it is just a R16 scenario and RAN4 should not introduce a R19 test case to verify a R16 procedure in a R19 WI.

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode

Issue 2-2-2: Applicability rules of test case

- Proposals

  - Option 1: (CATT)

    - No applicability rules are required if the following test case list is agreed. Otherwise, option 1 (i.e., option 2 in this issue) can be a good starting point.

| TC No. | Scenario | Comment |
| --- | --- | --- |
| 1 | Normal SCell activation + FR1 + Configuration 1 | Non-DRX |
| 2 | Normal SCell activation + FR2 + Configuration 1 | N/A |
| 3 | Normal SCell activation + FR1 + Configuration 2 | N/A |
| 4 | Normal SCell activation + FR2 + Configuration 2 | DRX |
| 5 | Normal SCell activation + FR1 + Configuration 3 | Non-DRX |
| 6 | Normal SCell activation + FR2 + Configuration 3 | N/A |
| 7 | Direct SCell activation at SCell addition + FR1 + Configuration 1 | N/A |
| 8 | Direct SCell activation at SCell addition + FR2 + Configuration 1 | DRX |
| 9 | Direct SCell activation at SCell addition + FR1 + Configuration 2 | Non-DRX |
| 10 | Direct SCell activation at SCell addition + FR2 + Configuration 2 | N/A |
| 11 | Direct SCell activation at SCell addition + FR1 + Configuration 3 | N/A |
| 12 | Direct SCell activation at SCell addition + FR2 + Configuration 3 | DRX |
| 13 | PUCCH SCell activation + FR1 + Configuration 1 | Non-DRX |
| 14 | PUCCH SCell activation + FR2 + Configuration 1 | N/A |
| 15 | PUCCH SCell activation + FR1 + Configuration 2 | N/A |
| 16 | PUCCH SCell activation + FR2 + Configuration 2 | DRX |
| 17 | PUCCH SCell activation + FR1 + Configuration 3 | N/A |
| 18 | PUCCH SCell activation + FR2 + Configuration 3 | Non-DRX |

  - Option 2: (Apple)

    - Introduce some applicability rule to reduce testing effort.

    - Agree the following testing applicability rules:

      - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

      - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

      - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

  - Option 2a: (Huawei)

    - UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

  - Option 2b: (Qualcomm)

    - If a UE supports Direct SCell Activation or PUCCH SCell Activation, the Normal SCell Activation test case can be skipped.

  - Option 3: (OPPO)

    - For normal SCell activation and PUCCH SCell activation, choose only one set of legacy configurations (e.g., non-DRX, 640ms SCell measurement cycle) to define the test case for fast SCell activation.

- Recommended WF

  - Discuss the option 1 and 2 as a starting point.

Meanwhile, to reduce testing effort, it would be great to consider some test applicability to allow UE to skip some of the test cases. The fundamental part that needs to be verified is whether UE can finish fast SCell activation based early measurement report. From this perspective, there is no huge difference among the above three scenarios. For instance, if UE can reduce SCell activation latency due to early measurement, it doesn’t really matter if the SCell is activated via RRC (direct SCell activation) or MAC (normal SCell activation). Besides, it is rational tath the new test cases for this objective can be derived based on corresponding R18 test case with valid report (A.6.6.9.4 and A.7.6.23.3). Therefore, the following testing applicability rules are proposed:

Proposal 2: introduce some applicability rule to reduce testing effort.

Proposal 3: agree the following testing applicability rules:

- Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

- Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

- Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

Issue 2-2-3: Test case design

- Proposals for principle of defining test cases

  - Option 1: (CATT, CT)

    - RAN4 to reuse the legacy test cases for Rel-18 eEMR measurement with valid result reporting, i.e., A.6.6.9/A.7.6.23, and SCell activation, i.e., A.6.5.3/A.7.5.3.

  - Option 2: (Apple)

    - Basic principle is to reuse the legacy test cases as much as possible. For instance, RAN4 can add SCell configuration and activation procedure on top of R18 eEMR test cases

  - Option 3: (ZTE, Ericsson)

    - The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle/Inactive mode fast CA/DC reporting test case.

  - Option 4: (Nokia)

    - Discuss which combinations of EMR configurations are tested with each SCell scenario.

- Proposals for details of test cases

  - Option 5: (CATT)

    - Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions:

      - For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell,  5*DRX cycles);

      - For target SCell in FR2, time gap needs to be larger than 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4.

  - Option 6: (OPPO)

    - The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively.

      - T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation.

        - For direct SCell activation, T3 and T4 can be merged into one.

    - The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline.

    - The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline.

  - Option 7: (Qualcomm)

    - RAN4 to not define test cases for DRX

    - RAN4 to consolidate test cases for EMR capability-related configurations into unified test cases based on SCell activation types (Normal SCell activation, Direct SCell activation, and PUCCH SCell activation)

      - Within each consolidated test case, adapt the test flow to accommodate the specific UE capabilities.

      - Use conditional logic or branching within the test case to handle capability-specific behaviors.

- Recommended WF

  - Discuss and polish the test procedure for normal SCell activation with Configuration 1 as a starting point:

    - Discuss whether the X1 and X2 in the test procedure are needed.

    - Discuss DRX configuration:

      - Alt 1: DRX is not configured in all test cases.

      - Alt 2: DRX is configured in some test cases.

    - Discuss whether to define a unified test case for all SCell activation types (Option 7).

    - Note: After the test procedure is concluded, it can be used as a template when drafting other test cases, while the corresponding legacy test cases can also be referred to.

Test procedure for normal SCell activation with Configuration 1: The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively as the figure below illustrates: - T1: During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2 (SCell to be activated). UE is configured with measurement reporting for Cell 2. The time point when UE receives RRC_Release message from the TE defines the starting point of T2. - T2: At the beginning of T2, Cell 2 becomes detectable. Signal level of Cell 2 and the duration of T2 are set to fixed values. - T3: At the beginning of T3, the signal level of Cell 2 is set to another value. The duration of T3 equals to measIdleValidityDuration-r18. - T4: The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report within the duration of T4. - T5: At the beginning of T5, the TE sends a MAC message for activation of the SCell (Cell 2). The point in time at which the MAC message is received at the UE antenna connector is denoted as slot #n. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3.2A.

We agree with moderator that RAN4 should reuse the legacy test cases as much as possible. A simple solution is to add SCell configuration and activation procedure based on R18 eEMR test cases with valid report.

Proposal 4: Basic principle is to reuse the legacy test cases as much as possible. For instance, RAN4 can add SCell configuration and activation procedure on top of R18 eEMR test cases.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR. After discussion, the following conclusions are provided:

Proposal 1: not introduce the following configuration 3, since it is just a R16 scenario and RAN4 should not introduce a R19 test case to verify a R16 procedure in a R19 WI.

- UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Proposal 2: introduce some applicability rule to reduce testing effort.

Proposal 3: agree the following testing applicability rules:

- Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

- Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

- Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

Proposal 4: Basic principle is to reuse the legacy test cases as much as possible. For instance, RAN4 can add SCell configuration and activation procedure on top of R18 eEMR test cases.

# Reference

- TS38.133