3GPP TSG-RAN WG4 Meeting #116bis																R4-251xxxx

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	4.26.2

Source:	MediaTek Inc.

Title:	Discussion on RRM core requirements for IoT NTN phase 3

Document for:	Discussion

# Introduction

RAN2 LS [1] states that CB‑Msg3‑EDT on the anchor carrier can include MSG3-based DL channel quality reporting for both NB‑IoT and eMTC, and we assess the impact on RRM requirements.

# DL channel quality reporting in MSG3 for contention-based Msg3 (CB-Msg3-EDT)

As indicated in RAN2 LS [1], MSG3-based channel quality reporting applies to the CB-Msg3-EDT procedure on the anchor carrier. In addition, RAN2 has the following companion change in the agreed CR (R2-2506573) for the RRC specification (TS 38.331).:

1> if the UE is a NB-IoT UE:

2> if the UE supports DL channel quality reporting in MSG3 and cqi-Reporting is present in SystemInformationBlockType2-NB:

3> except for CB-Msg3 transmission on the non-anchor carrier, set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the carrier where the random access response is received as specified in TS 36.133 [16];

However, the channel quality report requirement was defined with reference to PRACH or RAR, which do not exist in the CB-Msg3-EDT procedure and the time period T should be updated such as the period before CB-Msg3 transmission used for NRSRP measurement for CB-Msg3-EDT procedure on anchor carrier.

#### 6.6A.2.6	MSG3-based channel quality report for UE Category NB1

The requirements in this clause shall apply for UE supporting DL channel quality reporting for UE Category NB1 as defined in TS 36.331 [2] section 5.3.3.3, 5.3.3.3a, and 5.3.7.4.

The DL channel quality provides the serving eNB with information about the minimum NPDCCH repetition level to satisfy the hypothetical NPDCCH block error rate of 1% with the parameters specified in Table 6.6A.2.6-1.

Table 6.6A.2.6-1: NPDCCH transmission parameters for downlink quality reporting

| Parameters | Values |
| --- | --- |
| DCI format | Format N1 |
| Number of information bits (excluding CRC) | 23bits |
| System bandwidth | 200kHz |
| Aggregation level | 2 |
| DRX | OFF |

For channel quality reporting in the anchor carrier, the reported NPDCCH repetition level shall be derived from the channel quality measured in the period T1 or T2 in the carrier where the random access response is transmitted, where

-	T1 is the period before NPRACH transmission used for NRSRP measurement for enhanced coverage level estimation

-	T2 is the period from the beginning of the random access response to the beginning of PUSCH format 1 for DL channel quality reporting.

Proposal 1: Add new clause on	DL channel quality reporting in MSG3 for CB-Msg3-EDT, with time period T updated as follows:

-	T is the period before CB-Msg3 transmission used for NRSRP measurement for CB-Msg3-EDT procedure on anchor carrier.

# Summary

In this paper, we have the following observations and proposals:

Proposal 1: Add new clause on	DL channel quality reporting in MSG3 for CB-Msg3-EDT, with time period T updated as follows:

-	T is the period before CB-Msg3 transmission used for NRSRP measurement for CB-Msg3-EDT procedure on anchor carrier.

# References

- R2-2506299 (R4-2513012), LS on CQI reporting for CB-Msg3 EDT, Samsung, R2#131, Aug. 2025.