3GPP TSG-RAN WG4 Meeting #116bis																R4-2513638

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.20.3

Source:	MediaTek Inc.

Title:	Discussion on RRM performance requirements for IoT NTN phase 3

Document for:	Discussion

# Introduction

RAN4#116, the group agreed to define the following CB‑Msg3‑EDT test per WF [1].

- A.13.3.2.X1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT

This paper presents our view on the test definition.

# Test case for CB-msg3-EDT for IoT NTN

The test purpose is to verify the requirements in Clause 6.6A.4 (Requirements for CB-Msg3-EDT procedure) and Clause 7.20A	(UE transmit timing for NB-IoT for Satellite Access) in an AWGN model. The existing test case A.13.3.2.1 in TS 38.133	(Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage) can serve as a reference, with adaptations for the CB-Msg3-EDT procedure.

First, the CB-Msg3 EDT configuration (as defined in CB-Msg3-ConfigSIB-NB-r19) should be provided. RAN4 can focus on the parameters most relevant to performance/behavior and leave the remaining parameter details to RAN5. Second, In order to trigger CB-Msg3, conditions for initiating CB-Msg3 EDT in NTN shall be met at first as defined in TS 36.331 5.3.3.1b, e.g. 	the measured RSRP is larger than or equal to the minimum RSRP threshold configured in cb-Msg3-MinRSRP-Threshold-NB. The current threshold for NRPACH (as in Table A.13.3.2.1.1-3) can be reused for cb-Msg3-MinRSRP-Threshold-NB as baseline to ensure the RSRP is large enough, corresponding to 2 different CE level.

| cb-Msg3-MinRSRP-Threshold-NB-r19 | {39, 24} | Corresponding to {-101, -116} dBm as defined in Table 9.1.4-1 |
| --- | --- | --- |

Observation 1: In order to trigger CB-Msg3, conditions for initiating CB-Msg3 EDT in NTN should be met, as in TS 36.331, 5.3.3.1b.

Proposal 1: The CB‑Msg3‑EDT configuration, as defined in CB‑Msg3‑ConfigSIB‑NB‑r19 should be provided, where cb-Msg3-MinRSRP-Threshold-NB-r19 can be {39, 24} ({-101, -116} dBm), and let RAN5 define the parameter details.

# Summary

Our observations and proposals in this paper are summarized as below:

Observation 1: In order to trigger CB-Msg3, conditions for initiating CB-Msg3 EDT in NTN should be met, as in TS 36.331, 5.3.3.1b.

Proposal 1: The CB‑Msg3‑EDT configuration, as defined in CB‑Msg3‑ConfigSIB‑NB‑r19 should be provided, where cb-Msg3-MinRSRP-Threshold-NB-r19 can be {39, 24} ({-101, -116} dBm), and let RAN5 define the parameter details.

# References

- R4-2512148, WF on RRM requirements for IoT_NTN_Ph3, R4#116.