# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513649

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	4.19.3

Source: 	CMCC

Title: 			 Discussion on RRM core requirements for SBFD

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on RRM core requirements for SBFD, and a WF was approved [2]. This contribution provides discussion on the remaining issue.

- Discussion

Issue 2-1-2: Requirements for RACH - Proposals - Proposal 1 (Nokia, LGE, ZTE): - The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account the RO-type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols, and the RO type selection from additional RO to legacy RO, as agreed in RAN2. - Proposal 2 (HW, vivo): - RAN4 not to update RACH requirements for SBFD. - Proposal 3 (MTK): - RAN4 RRM spec should refer to the agreement on PRACH transmission re-attempt in RAN2 spec without a need to duplicate the same line in RAN4 RRM spec. - Agreements - Discuss whether and how to update RACH requirements based on RAN2 spec in maintenance phase.

According to the approved RAN2 latest CR on SBFD for MAC spec [3], the related agreements on RACH occasion switch between legacy RACH occasions and additional RACH occasions are captured. In detail, in clause 5.1.2, it is specified to determine the available set of PRACH occasions of the selected RO type. And how to select RO type is specified in clause 5.1.1 for Random Access procedure initialization and clause 5.1.4 for Random Access Response reception.

Following is the duplication of legacy RAN4 spec on RACH. It can be seen that for Random Access Resource selection procedure, it directly refer to clause 5.1.2 RAN2 spec 38.321. With this, it seems no need to update RAN4 spec to.

TS 38.133 6.2.2.2.1 Contention based random access 6.2.2.2.1.1 Correct behaviour when transmitting Random Access Preamble With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB if the association between Random Access Preambles and SSB is configured, as specified in clause 5.1.2 in TS 38.321 [7]. With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, if the association between PRACH occasions and SSBs is configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [7]. 6.2.2.2.1.2 Correct behaviour when receiving Random Access Response The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble. 6.2.2.2.1.3 Correct behaviour when not receiving Random Access Response The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [7].

Proposal 1: for RO type change, considering the related procedures are captured in RAN2 spec, no need to duplicate the procedures in RAN4 spec.

- Conclusion

This contribution provides discussion on the remaining issue for RRM core requirements for SBFD. The proposals are:

Proposal 1: for RO type change, considering the related procedures are captured in RAN2 spec, no need to duplicate the procedures in RAN4 spec.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R2-2506606, Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416919, LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2502633, Updated summary of simulation results for L1-SRS-RSRP, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting