# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513657

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.12.4

Source: 	CMCC

Title: 			 Discussion on RRM requirements for indirect measurement event prediction

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [1]. In RAN #109 meeting, the revised WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [2]. The objectives for RAN4 are duplicated as following.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any

This contribution provides discussion on RAN4 impacts for indirect measurement event prediction.

- Discussion

According to RAN2 agreements in Rel-19 study, for indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event.

Agreements in RAN2#127bis Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference.

For indirect event prediction, the requirements for the predicted event triggered reporting including, but not limited to, delay and accuracy requirements and the performance metrics are considered. In RAN4 Rel-19 study, it was agreed that in case of indirect measurement event prediction use case, if the report includes the predicted RSRPs corresponding to the predicted event occurrence time, the absolute and/or relative accuracy requirement for the predicted RSRP will be defined. This agreement is still valid for Rel-20 discussion. It is proposed to follow previous agreement to define requirement for the absolute and/or relative accuracy of predicted RSRP for indirect measurement event prediction use case.

Agreements in RAN4#115 (R4-2508268) Issue 3-1: Performance metric for event prediction use case Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

Proposal 1: for indirect measurement event prediction use case, it is proposed to follow the agreements in study phase that if the report includes the predicted RSRP, the absolute and/or relative accuracy requirement for the predicted RSRP will be defined.

According to the WID, temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction for co-located cases are considered. The definitions of indirect measurement event prediction in RAN2 is duplicated as below. Taking temporal domain case A as an example, in indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. It can be seen that for indirect measurement event prediction, the intermediate output is same as that for RRM measurement prediction. The discussion and conclusion on RRM measurement prediction can be reused.

Agreements in RAN2#128 Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

Proposal 2: for indirect measurement event prediction use case, for the requirements for the absolute and/or relative accuracy of predicted RSRP, the conclusion for RRM measurement prediction can be reused.

The final output for indirect measurement event prediction is the expected occurrence time of a certain measurement event. In legacy, there is no RAN4 requirements on the expected occurrence time of a event. It is necessary to discuss whether and how to define related RAN4 requirements.

Proposal 3: for indirect measurement event prediction, it is proposed to discuss whether and how to define RRM requirements for the final output (e.g. expected occurrence time of a event for indirect prediction).

There is discussion on impact of measurement error for event prediction use case in study phase, but there is no consensus.

Issue 3-2: Impact of measurement error for event prediction use case - Proposals and Observations: - Proposal 1 (ZTE): Measurement error should be considered into measurement event prediction, which will impact the prediction accuracy. - Proposal 2 (CMCC): Whether measurement error is considered for event prediction use case can be considered case by case. - For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused. - For direct measurement event prediction, whether to consider the impact due to measurement error depends on the model input. - If the model input does not include measured RSRP, no need to consider measurement error. Otherwise, measurement error may have impact on prediction performance. - Proposal 3 (Apple): - RAN4 to discuss the modelling of measurement errors and their impact to evaluate AI/ML event prediction benefits compared to sample-and-hold and the impact to false detection of events (higher cost) - For indirect measurement event prediction, RAN4 to discuss the modeling and impact of measurement errors to the prediction accuracy (i.e., F1 score) - RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions.

For indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. It ca be seen that the AI/ML model is only used for RRM prediction, which means for indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused.

The impact of measurement error for measurement prediction are discussed in study phase. For both intra-frequency and inter-frequency prediction, RAN4 thinks RF and baseband errors on the measurements would impact the prediction accuracy. Due to limited timeline, RAN4 do not perform the simulation to evaluate the impact. But RAN4 discussed how to model the measurement error. And conclusion is that for BB error, use link level simulation to generate L3-RSRP difference as baseband error as baseline, for RF error model, use a truncated Gaussian distribution.

During the WI phase, to reduce work load, no need to perform the discussion on impact of measurement error for both RRM measurement prediction and measurement event prediction in parallel. As discussed above, considering that AI/ML model is only used to perform RRM prediction to derive whether a measurement event at one future time instance occurs, the conclusion on how to handle the measurement error for RRM prediction mode can be reused for measurement event prediction, it is proposed that RAN4 firstly evaluate the measurement error impact for RRM measurement prediction.

Proposal 4: For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused. RAN4 firstly focus on RRM measurement prediction to  discuss the impact due to measurement error.

- Conclusion

This contribution provides discussion on RRM requirements for indirect measurement event prediction. The proposals are:

Proposal 1: for indirect measurement event prediction use case, it is proposed to follow the agreements in study phase that if the report includes the predicted RSRP, the absolute and/or relative accuracy requirement for the predicted RSRP will be defined.

Proposal 2: for indirect measurement event prediction use case, for the requirements for the absolute and/or relative accuracy of predicted RSRP, the conclusion for RRM measurement prediction can be reused.

Proposal 3: for indirect measurement event prediction, it is proposed to discuss whether and how to define RRM requirements for the final output (e.g. expected occurrence time of a event for indirect prediction).

Proposal 4: For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused. RAN4 firstly focus on RRM measurement prediction to  discuss the impact due to measurement error.

- Reference

- RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR