# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513665

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.11.5

Source: 	CMCC

Title: 			 Discussion on RRM requirements for positioning accuracy enhancement

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The related objectives are duplicated as following.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM core requirements for positioning accuracy enhancement.

- Discussion

Issue 3-1: Reporting Delay Requirements for case 1 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements Issue 3-3: Report mapping for UL SRS-TDCP measurement Agreement: Adopt the mapping for UL SRS-TDCP as proposed in R4-2510759 Issue 3-4: Report mapping for UL SRS-TDCT and UL SRS-TDCP measurements Agreement: Technically endorse CR in R4-2510760, to be later merge with the rest of positioning requirements

For case 1, it was agreed that the reporting delay includes measurement delay, inference delay and the time needed until the UE send the report. One FFS is the number of samples needed for the measurements.

For legacy measurement for positioning, the number of samples depends on UE capability and LMF request. Taking UE Rx-Tx time difference measurement as an example, part of legacy spec on number of samples is duplicated as following. Same approach are used for other positioning measurement types.

TS 38.133, Nsample is the number of UE Rx-Tx time difference measurement samples: - Nsample= 1 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] and the following conditions are met: - PRS bandwidth is within the active BWP and - Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB. - Nsample= 2 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and the LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] but the following conditions are not met: - PRS bandwidth is within the active BWP and - Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB. - Nsample= 4 otherwise.

The measurement as input for AI/ML model is smilar as legacy measurement. And the leagcy apporach can be used. In detail, the number of samples can be 1, 2, or 4 based on the UE capability and LMF request.

Proposal 1: for case 1, the number of measurement samples can be 1, 2, or 4 based on the UE capability and LMF request, similar as legacy positioning measurement.

Case 3a is NG-RAN node assisted positioning with gNB-side model (AI/ML assisted positioning). Case 3b is NG-RAN node assisted positioning with LMF-side model (direct AI/ML positioning). For case 3a/3b, the agreements reached in RAN4 previous meetings are summarized in following table.

Agreements in RAN4#112bis (R4-2417212) Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b. Agreements in RAN4#114 (R4-2502856) Issue 3-4: Report mapping for UL SRS-RSRP Agreement: - Reuse the report mapping for UL SRS-RSRP to Case 3a Agreements in RAN4#114bis (R4-2505105) Issue 3-3: Rx-Tx time difference for case 3a Agreement: - Introduce gNB Rx-Tx time difference reporting accuracy requirements - Reuse legacy accuracy requirements Issue 3-4: Report mapping for gNB Rx-Tx time difference Agreement: - Reuse the existing report mapping for gNB Rx-Tx time difference measurements Agreements in RAN4#115 (R4-2508080) Issue 3-3: Requirements for case 3b Agreement: - Reuse legacy path based Rx-Tx accuracy requirements for case 3b - Requirement applies only for the 1st path

According to current TS38.133, measurement performance requirements for NR gNB include report mapping for UL-RTOA, report mapping for gNB Rx-Tx time difference, measurement accuracy requirements for gNB Rx-Tx time difference, report mapping for UL SRS RSRP, measurement accuracy requirements for UL SRS RSRP, report mapping for UL SRS RSRPP. The situation is summarized as following Table 1.

Table 1 summary of agreements on requirements for case 3a

| Legacy meaurement requirements for gNB |  | Case 3a |
| --- | --- | --- |
| UL-RTOA | report mapping | Already agreed to reuse legacy requirements |
| gNB Rx-Tx time difference | report mapping | Already agreed to reuse legacy requirements |
|  | measurement accuracy requirements | Already agreed to reuse legacy requirements |
| UL SRS RSRP | report mapping | Already agreed to reuse legacy requirements |
|  | measurement accuracy requirements | ? |
| UL SRS RSRPP | report mapping | Already agreed to reuse legacy requirements |

It can be seen that RAN4 agreed to reuse the report mapping for UL SRS-RSRP to Case 3a, but measurement accuracy requirements for UL SRS RSRP is missing for case 3a. It is proposed that to reuse legacy UL SRS RSRP measurement accuracy requirements for case 3a.

Proposal 2: It is proposed to reuse legacy UL SRS RSRP measurement accuracy requirements for case 3a.

- Conclusion

This contribution provides discussion on RRM requirements for positioning. The proposals are:

Proposal 1: for case 1, the number of measurement samples can be 1, 2, or 4 based on the UE capability and LMF request, similar as legacy positioning measurement.

Proposal 2: It is proposed to reuse legacy UL SRS RSRP measurement accuracy requirements for case 3a.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R1-2405389, Summary #5 of specification support for positioning accuracy enhancement

- R4-2501758, On requirements for AI/ML based positioning, Ericsson