# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513669

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.4.3.2

Source: 	CMCC

Title: 			 Discussion on RRM core requirements for NR Radio Resource Management (RRM) Phase 6

Document for:	Discussion

- Introduction

In RAN #109 meeting, the new WID on NR Radio Resource Management (RRM) Phase 6 [1] was approved. According to the WID, one of the objectives is enhancement for (e)RedCap UEs, the details are duplicated as following.

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

There contribution provides discussion on relaxed RLM/BFD RRM requirements for (e)RedCap UEs.

- Discussion

RLM/BFD relaxation for legacy UE are defined in Rel-17. The relaxed requirements are defined for both SSB based radio link monitoring and CSI-RS based radio link monitoring. The relaxed requirements are applied for the UE supporting rlm-Relaxation-r17 and configured with explicit signalling goodServingCellEvaluationRLM. In detail, there are two cases: when the good serving cell quality criterion is fulfilled for the serving cell if the lowMobilityEvaluationConnected is not configured, or the UE is also configured with lowMobilityEvaluationConnected and both low mobility criterion is fulfilled for a period of TSearchDeltaP-Connected and good serving cell quality criterion is fulfilled for the serving cell. Taking SSB based RLM as an example, the relaxed requirements are duplicated as below. Scaling factor K1 and K3 are introduced the relax the delay.

Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

Table 8.1.2.4-2: Evaluation period TEvaluate_out_SSB_Relax for FR2

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K4 NOTE3, Ceil(15 K2 NOTE2 P N) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P N) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K2 = 2. NOTE 3: K4 = K2, if K2 ≤ 2; otherwise K4 = 1. |  |

For 2Rx RedCap UE, the existing requirements for SSB based RLM are shown as following. It can be seen the requirements are same as that for legacy UE. Same situation exists for CSI-RS based RLM.

Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1B.2.2-2: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR2 for 2Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P N) TSSB) | Max(100, Ceil(5 P N) TSSB) |
| DRX cycle≤320 ms | Max(200, Ceil(15 P N) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(10 P N) TDRX | Ceil(5 P N) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

To support relaxed RLM/BFD for 2RX RedCap UE, it is straightforward to reuse legacy relaxed approach including relaxation criteria and scaling factor for legacy UEs.

Proposal 1: it is proposed to reuse relaxation criteria and scaling factor for relaxed requirements for legacy UEs to define relaxed RLM/BFD requirements for 2RX RedCap UE.

For 1Rx RedCap UE, the existing requirements for SSB based RLM are shown as following. Compared with the requirements for 2RX RedCap, the out-of-sync evaluation period is doubled for 1RX RedCap UE. To support relaxed RLM/BFD requirements for 1RX RedCap UE, whether relaxation criteria and scaling factor for legacy UEs with 2RX can be reused need further discussion.

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Proposal 2: for 1RX RedCap UE, it is proposed to discuss whether relaxation criteria and scaling factor for relaxed requirements for legacy UEs with 2RX can be reused.

- Conclusion

This contribution provides discussion on relaxed RLM/BFD RRM requirements for (e)RedCap UEs. The proposals are:

Proposal 1: it is proposed to reuse relaxation criteria and scaling factor for relaxed requirements for legacy UEs to define relaxed RLM/BFD requirements for 2RX RedCap UE.

Proposal 2: for 1RX RedCap UE, it is proposed to discuss whether relaxation criteria and scaling factor for relaxed requirements for legacy UEs with 2RX can be reused.

- Reference

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6