3GPP TSG-RAN4 Meeting #116bis	R4-2513679

# Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction of ATG CA requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN2#131 meeting, the signaling for neighbour cell skipping scheme has been defined, which are not aligned with current RAN4 specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the signaling name of neighbour cell skipping scheme according to signaling design from RAN2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The R19 ATG spec will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5D, 9.2D.5, 9.5D.4, 9.8D.4, 9.10D.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The revision is based on the agreed BigCR R4-2510017 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change#1>

8.5D	Link Recovery Procedures for ATG

8.5D.1	Introduction

The requirements in clause 8.5D apply for link recovery procedure on ATG UE.

The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set q0 as specified in TS 38.213 [3] in order to detect beam failure on PCell and SCell.

The RS resource configurations in the set q0 can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set q0 on SCell shall be periodic CSI-RS. UE is not required to perform beam failure detection outside the active DL BWP. UE is not required to meet the requirements in clause 8.5D.2 and 8.5D.3 if UE does not have set q0. UE is not required to perform beam failure detection on a deactivated SCell, and also not required to perform beam failure detection on resources which is implicitly configured for a deactivated SCell. When more than 2 periodic CSI-RS resources on a CC are configured in the set q0 for current SCell or implicitly configured in the set q0 for other SCell, it is up to UE implementation to select two of CSI-RS resources in active BWP in current CC to perform beam failure detection. UE is not required to perform beam failure detection on a SCell on which q1 is not configured.

On each RS resource configuration in the set q0, the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell beams.

The threshold Qout_LR is defined as the level at which the downlink radio level link of a given resource configuration on set q0 cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. For SSB based beam failure detection, Qout_LR_SSB is derived based on the hypothetical PDCCH transmission parameters listed in table 8.5.2.1-1. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in table 8.5.3.1-1.

Upon request the UE shall deliver configuration indexes from the set q1as specified in TS 38.213 [3] , to higher layers, and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR, which is indicated by higher layer parameter rsrp-ThresholdSSB. The UE applies the Qin_LR threshold to the L1-RSRP measurement obtained from an SSB. The UE applies the Qin_LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by higher layer parameter powerControlOffsetSS. The RS resource configurations in the set q1 can be periodic CSI-RS resources or SSBs or both SSB and CSI-RS resources. UE is not required to perform candidate beam detection outside the active DL BWP. UE is not required to perform candidate beam detection on a SCell on which q1 is not configured.

8.5D.2	Requirements for SSB based beam failure detection

8.5D.2.1	Introduction

The requirements in this clause apply for each SSB resource in the set q0 configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5D.2.2.

Table 8.5D.2.1-1: Void

8.5D.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set q0 estimated over the last TEvaluate_BFD_SSB period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB period.

The value of TEvaluate_BFD_SSB is defined in table 8.5D.2.2-1 for FR1.

For FR1 ATG UE with one or multiple omni-directional antenna(s),

-	P=11−TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB.

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR1 ATG UE with the antenna array,

-	P value for an BFD-RS resource to be measured is defined as:

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	Ntotal / Navailable with Navailable > 0

-	For a window W of duration max(TL1,  MGRPmax), where MGRPmax is the maximum MGRP across all configured per-UE measurement gaps starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-		If the measured carrier is the SCC with servingcellMO configured, and neighSCellMeasSkipping is set to ‘enable’ by network

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-     the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-     the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions of other serving cell within the window W

-	otherwise,

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-    the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions for L3 measurements of the same serving cell within the window W,

-	otherwise,

-	the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target BFD-RS

-	Psharing factor = 3.

Longer evaluation period would be expected if the combination of BFD-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

Table 8.5D.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle > 320 ms | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of SSB in the set q0. TDRX is the DRX cycle length. |  |

8.5D.2.3	Measurement restriction for SSB based beam failure detection

The UE is required to be capable of measuring SSB for BFD without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following clauses.

For FR1, when the SSB for BFD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined.

8.5D.3	Requirements for CSI-RS based beam failure detection

8.5D.3.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set q0 of resource configurations for a serving cell, provided that the CSI-RS resource(s) in set q0for beam failure detection are actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5D.3.2. UE is not expected to perform beam failure detection measurements on the CSI-RS configured for BFD if the CSI-RS is not QCL-ed, with QCL-TypeD when applicable, with the RS in the active TCI state of any CORESET configured in the UE active BWP.

Table 8.5D.3.1-1: Void

8.5D.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set q0 estimated over the last TEvaluate_BFD_CSI-RS period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS period.

The value of TEvaluate_BFD_CSI-RS is defined in table 8.5D.3.2-1 for FR1.

The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For FR1 ATG UE with one or multiple omni-directional antenna(s),

-	P=11−TCSI−RSMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS.

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For FR1 ATG UE with the antenna array,

-	P value for an BFD-RS resource to be measured is defined as:

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	Ntotal / Navailable with Navailable > 0

-	For a window W of duration max(TL1,  MGRPmax), where MGRPmax is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	If the measured carrier is the SCC with servingcellMO configured, and neighSCellMeasSkipping is set to ‘enable’ by network

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-    the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-     the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions of other serving cell within the window W

-	otherwise,

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-    the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions for L3 measurements of the same serving cell within the window W,

-	otherwise,

-	the number of BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target BFD-RS

-	Psharing factor = 3.

Longer evaluation period would be expected if the combination of the BFD-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

The values of MBFD used in table 8.5D.3.2-1 are defined as:

-	MBFD = 10, if the CSI-RS resource(s) in set q0 used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PBFD used in table 8.5D.3.2-1 is defined as

For each CSI-RS resource in the set configured for a SCell

-	PBFD = Z

-	Where Z is the number of band(s) on which UE is performing beam failure detection only for SCell

Table 8.5D.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(MBFD  P PBFD)  TCSI-RS) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(1.5 × MBFD  P PBFD)  Max(TDRX, TCSI-RS)) |
| DRX cycle > 320 ms | Ceil(MBFD  P PBFD)  TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource in the set q0. TDRX is the DRX cycle length. |  |

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following clauses.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

8.5D.4	Minimum requirement for L1 indication

The requirements in clause 8.5.4 shall apply only for UE in NR SA operation mode.

8.5D.5	Requirements for SSB based candidate beam detection

8.5D.5.1	Introduction

The requirements in this clause apply for each SSB resource in the set q1 configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5D.5.2.

8.5D.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set q1 estimated over the last TEvaluate_CBD_SSB period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to annex B.2.4.1 for a corresponding band.

The UE shall monitor the configured SSB resources using the evaluation period in table 8.5D.5.2-1 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320 ms.

The value of TEvaluate_CBD_SSB is defined in table 8.5D.5.2-1 for FR1.

where,

For FR1 ATG UE with one or multiple omni-directional antenna(s),

-	P=11−TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB,

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR1 ATG UE with the antenna array,

-	 P value for an CBD-RS resource to be measured is defined as:

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	Ntotal / Navailable with Navailable > 0

-	For a window W of duration max(TL1,  MGRPmax), where MGRPmax is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-		If the measured carrier is the SCC with servingcellMO configured, and neighSCellMeasSkipping is set to ‘enable’ by network

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-    the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-    the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions of other serving cell within the window W

-	otherwise,

-		If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-    the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions for L3 measurements of the same serving cell within the window W,

-	otherwise,

-	the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CBD-RS

-	Psharing factor = 3.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

The values of PCBD used in table 8.5D.5.2-1 is defined as

For each SSB resource in the set q1 configured for a SCell

-	PCBD = Z

-	Where Z is the number of band(s) on which UE is performing candidate beam detection only for SCell

Table 8.5D.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil(3 P PCBD) TSSB) |
| DRX cycle > 320 ms | Ceil(3 P PCBD) TDRX |
| NOTE: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. |  |  |

8.5D.5.3	Measurement restriction for SSB based candidate beam detection

For FR1, when the SSB for CBD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for CBD measurement without any restrictions;

-	If SSB and CSI-RS have different SCSs,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for CBD measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for CBD measurement and CSI-RS. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

8.5D.6	Requirements for CSI-RS based candidate beam detection

8.5D.6.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set q1 configured for a serving cell, provided that the CSI-RS resources configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5D.6.2.

8.5D.6.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set q1 estimated over the last TEvaluate_CBD_CSI-RS period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS period provided CSI-RS Ês/Iot is according to annex B.2.4.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in table 8.5D.6.2-1 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320 ms.

The value of TEvaluate_CBD_CSI-RS is defined in table 8.5D.6.2-1 for FR1.

For FR1 ATG UE with one or multiple omni-directional antenna(s),

-	P=11−TCSI−RSMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For FR1 ATG UE with the antenna array,

-	 P value for an CBD-RS resource to be measured is defined as:

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	Ntotal / Navailable with Navailable > 0

-	For a window W of duration max(TL1,  MGRPmax), where MGRPmax is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	If the measured carrier is the SCC with servingcellMO configured, and neighSCellMeasSkipping is set to ‘enable’ by network

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-    the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-    the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions of other serving cell within the window W

-	otherwise,

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-   the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs resource occasions for L3 measurements of the same serving cell within the window W,

-	otherwise,

-	the number of CBD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CBD-RS

-	Psharing factor = 3.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 8.5D.6.3.

The values of MCBD used in table 8.5D.6.2-1 are defined as:

-	MCBD = 3, if the CSI-RS resource configured in the set q1 is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PCBD used in table 8.5D.6.2-1 is defined as

For each CSI-RS resource in the set q1 configured for a SCell

-	PCBD = Z

-	Where Z is the number of band(s) on which UE is performing candidate beam detection only for SCell

Table 8.5D.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil(MCBD P PCBD) TCSI-RS) |
| DRX cycle > 320 ms | Ceil(MCBD P PCBD) TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource in the set q1. TDRX is the DRX cycle length. |  |  |

# <End of change#1>

# <Start of change#2>

9.2D.5	Intra-frequency measurements without measurement gaps

9.2D.5.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra-frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2D.5.1-1 or table 9.2D.5.1-2 (CC with deactivated SCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2D.5.1-3 or table 9.2D.5.1-4 (CC with deactivated SCell).

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2D.5.2-1 or table 9.2D.5.2-2 (CC with deactivated SCell).

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1D.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps,  or according to CSSFwithin_gap,i in clause 9.1D.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in clause 9.1.8.3.

Kp = 1 when Navailable = 0.

-	Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.	If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

For UE capable of antennaArrayType-r18 on the measured carrier,

Klayer1_measurement=1,

If the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE,

Otherwise,

If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and UE support two simultaneous separate Rx beams

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of the same serving cell

Otherwise,

-     if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Otherwise, Klayer1_measurement=1.

Table 9.2D.5.1-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil(5 x Kp x N1Note 3,4 x Klayer1_measurement) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 5 x Kp x N1Note 3,4 x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp x N1 Note 3,4 x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: void. NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance information on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 4: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.1-2: Time period for PSS/SSS detection, CC with deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp x N1Note 1,2) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp x N1Note 1,2) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp x N1Note 1,2) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance information on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 2: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil(3 x Kp x N1 Note 3,4 x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (1.5 x 3 x Kp x N1 Note 3,4 x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(3 x Kp x N1 Note 3,4 x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: void NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 4: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.1-4: Time period for time index detection, CC with deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp x N1Note 1,2) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp x N1Note 1,2) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp x N1Note 1,2) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance information on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 2: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

9.2D.5.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 9.2D.5.2-1, 9.2D.5.2-2(CC with deactivated SCell).

If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

Table 9.2D.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp x N1 Note 2,3 x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp x N1 Note 2,3 x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp x N1 Note 2,3 x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 3: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

Table 9.2D.5.2-2: Measurement period for intra-frequency measurements without gaps (CC with deactivated SCell) (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp x N1 Note 2,3) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(1.5x 5 x Kp x N1 Note 2,3) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp x N1 Note 2,3) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 3: For ATG UE capable of antennaArrayType-r18, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

# <End of change#2>

# <Start of change#3>

9.5D.4	L1-RSRP measurement requirements

9.5D.4.1	SSB based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_ATG.

The value of TL1-RSRP_Measurement_Period_SSB_ATG is defined in table 9.5D.4.1-1 for FR1, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

For ATG UE with the capable of antennaArrayType-r18 on the measured carrier,

P value for SSB resource to be measured is defined as

-	Ntotal / Navailable with Navailable > 0

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	if the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE

-	if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements of other serving cell within the window W

-	otherwise,

-	if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements of same serving cell within the window W

-	otherwise,

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements within the window W

-	TL1 is periodicity of the target SSB

-	Psharing factor = 3.

Otherwise, for UE with one or multiple omni-directional antennas

For a UE supporting concurrentMeasGap-r17 and when concurrent gaps are configured,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	TL1 is periodicity of the target SSB.

Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured,

-	P=11−TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency or inter-frequency, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

Where:

-	TSSB = ssb-periodicityServingCell of the serving cell

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and GAP configurations does not meet previous conditions.

Table 9.5D.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB_ATG for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_ATG (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

9.5D.4.2	CSI-RS based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS_ATG.

-	For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	For aperiodic CSI-RS resources M=1

For ATG UE capable of antennaArrayType-r18 on the measured carrier,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Navailable with Navailable > 0

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	if the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE,

-	if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements of other serving cell within the window W

-	otherwise,

-	if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements of same serving cell within the window W

-	otherwise,

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements within the window W

-	TL1 is periodicity of the target CSI-RS

-	Psharing factor = 3.

Otherwise, for UE with one or multiple omni-directional antenna(s)

For a UE supporting concurrentMeasGap-r17 and when concurrent gaps are configured,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

TL1 is periodicity of the target CSI-RS.

Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured,

-	P=11−TCSI−RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

Where:

TCSI-RS = the periodicity of CSI-RS configured for L1-RSRP measurement

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

Table 9.5D.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS_ATG for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS_ATG (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

# <End of change#3>

# <Start of change#4>

9.8D.4	L1-SINR measurement requirements

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the CSI-RS configured as CMR and no dedicated resource configured as IMR for L1-SINR computation, and the UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only_ATG.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only_ATG is defined in table 9.8D.4.1-1 for FR1, where

For the value of M,

-	For periodic and semi-persistent CSI-RS resources as CMR, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise;

-	For aperiodic CSI-RS resources as CMR, M=1.

For ATG UE capable of antennaArrayType-r18 on the measured carrier,

P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Navailable with Navailable > 0

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	If the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE,

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams,

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W.

-	Otherwise,

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements of other serving cell within the window W.

-	Otherwise,

-	If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements of same serving cell within the window W.

-	Otherwise,

-	the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RS resource occasions for L3 measurements within the window W.

-	TL1 is periodicity of the target CSI-RS

-	Psharing factor = 3.

Otherwise, for UE with one or multiple omni-directional antenna(s)

For a UE supporting concurrentMeasGap-r17 and when concurrent gaps are configured,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	TL1 is periodicity of the target CSI-RS.

Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured,

-	P=11−TCSI−RSxGP, when in the monitored cell there are [measurement gaps] configured for intra-frequency or inter-frequency, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

Where:

-	TCSI-RS = the periodicity of CSI-RS configured for L1-SINR measurement

-	a CSI-RS is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-SINR measurement and SMTC means that CSI-RS for L1-SINR measurement is within the SMTC window duration.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and GAP configurations does not meet previous conditions.

Table 9.8D.4.1-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only_ATG for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only_ATG (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

# <End of change#4>

# <Start of change#5>

9.10D.2.5	Intra-frequency measurements without measurement gaps

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the CSI-RS based measurement shall include PSS/SSS detection time of associatedSSB, the time period used to acquire the SFN information and CSI-RS based measurement period without gap.

-	PSS/SSS detection time of associatedSSB is the intra-frequency TPSS/SSS_sync_intra in clause 9.2D.5.1.

-	The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra as shown in table 9.10D.2.5-3 for FR1.

-	If the associatedSSB, which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2D.5.1, becomes undetectable for a period ≤ 5 seconds and then the associatedSSB becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than  3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3], PSS/SSS detection time and time period used to acquire the SFN information are equal to 0.

The measurement period for CSI- RS based intra-frequency measurements without gaps is as shown in table 9.10D.2.5-1.

Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, or if CSI-RS is configured with associated SSB but not QCL-ed to the associated SSB, the UE is not required to monitor the corresponding CSI-RS resource.

Table 9.10D.2.5-1: Measurement period for intra-frequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | T CSI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x CSI-RS period) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x max(CSI-RS period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp_CSI-RS x N1Note 3 x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. D is frequency domain density for the 1-port CSI-RS for L3 mobility defined in clause 7.4.1 of TS 38.211 [6]. NOTE 2: Kp_CSI-RS is applicable for a UE supporting concurrent gaps NOTE 3: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 4: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1D.5.

For a UE not supporting concurrentMeasGap-r17 or for a UE is supporting concurrentMeasGap-r17 but not configured with concurrent measurement gaps,

-	if the intra-frequency CSI-RS resource does not overlap with any measurement gaps, Kp_CSI-RS=1;

-	if some occasions of the intra-frequency CSI-RS resource is overlapped with measurement gaps, Kp_CSI-RS = 1/(1- (CSI-RS resource period /MGRP)) , where CSI-RS resource period < MGRP, and the MGRP is the periodicity of the measurement gap.

-	Otherwise, when UE supports concurrent measurement gaps, and concurrent measurement gaps are configured, Kp_CSI-RS is the scaling factor for a CSI-RS frequency layer to be measured outside gap which is defined as Kp_CSI-RS = Ntotal / Navailable

For a window W of duration max(CSI-RS period,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG, and starting at the beginning of any gap occasions covering the CSI-RS resources:

Ntotal is the total number of CSI-RS resources within the window, including those overlapped with other MG occasions within the window, and

Navailable is the number of CSI-RS resources that are not overlapped with any other non-dropped MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule.

Kp_CSI-RS = 1 when Navailable = 0

For UE supporting antennaArrayType-r18 on the measured carrier,

Klayer1_measurement=1,

If the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE,

Otherwise,

If inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE support two simultaneous separate Rx beams

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs occasions of same serving cell

Otherwise,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs occasions

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

For UE not supporting antennaArrayType-r18 on the measured carrier, Klayer1_measurement=1.

Table 9.10D.2.5-2: Void

Table 9.10D.2.5-3: Time period for SFN acquisition for intra-frequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | TCSI-RS_SFN_intra |
| --- | --- |
| No DRX | max(200 ms, ceil(5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(2000 ms, ceil (1.5 x 5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(5 x Kp_CSI-RS x N1Note 2 x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For ATG UE capable of antennaArrayType-r18, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. NOTE 3: For ATG UE capable of antennaArrayType-r18, for the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE. Otherwise, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided. |  |

# <End of change#5>