3GPP TSG-RAN WG4 Meeting #116bis			R4-2513682

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	4.26.2

Source: 	CMCC

Title: 			   Discussion on the RRM requirement maintenance for IoT NTN phase3

Document for:	Discussion

- Introduction

In last meeting, the core part of IOT NTN phase 3 has been finished[1], but the requirements are not fully completed which are waiting for RAN2’s progress.

Besides, a LS was received from RAN2#131 meeting[2], which inform RAN4 about additional agreements on CQI reporting in CB-Msg3 EDT procedure. RAN4 need to take them into account when define the RRM requirements.

In this contribution, we further discuss the maintenance issues.

- Maintenance for CB-Msg3 EDT

## Correct behaviour when receiving CB-Msg4

When receiving the CB-Msg4, further UE behavior is defined by RAN2 in two more sub-cases about whether C-RNTI and RRC message are included in the CB-Msg4. We copy them for the information.

Agreement from RAN2#131 meeting: Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4 without an RRC message, the UE considers UL transmission as successful but the UE does not consider the CB-Msg3-EDT procedure has ended and continues monitoring PDCCH. Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including both an RRC message and a C-RNTI, the UE behaves as if it received an empty EarlyDataComplete message, terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE.

We think above UE behavior shall be also included in RAN4 spec.

Proposal 1: Include following UE behavior when receiving the CB-Msg4 in the Clause “Correct behaviour when receiving CB-Msg4” according to RAN2’s agreements:

- Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring PDCCH.

- Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE.

## Correct behaviour when not receiving CB-Msg4

When not receiving CB-Msg4, several related agreements were achieved by RAN2 in last meeting:

Agreement from RAN2#131 meeting: When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, the MAC indicates failure of CB-Msg3 transmission to the upper layer (i.e. RRC). When the RRC layer receives a CB-Msg3 EDT failure indication from the MAC layer the RRC procedure is re-initiated (how to avoid data loss is up to UE implementation). Which procedure (e.g. EDT, 4-step RACH, CB-Msg3-EDT) is initiated is up to UE implementation If the PDCCH is successfully decoded before CB-Msg3ResponseTimer expires, but the corresponding PDSCH is successfully decoded after the timer has expired, the MAC PDU is treated the same as the one successfully decoded before the timer expires.

Based on these agreements, UE behavior when the CB-Msg3ResponseTimer expires and the UE has reached maximum number of transmissions can be specified.

Proposal 2: For “Correct behaviour when not receiving CB-Msg4”, include following UE behavior when the CB-Msg3ResponseTimer expires and the UE has reached maximum number of transmissions according to RAN2’s agreements:

- When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, and the UE has not received a CB-Msg4, or the UE doesn’t successfully decode the corresponding PDSCH after the CB-Msg3ResponseTimer has expired, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation.

## RAN2 LS on CQI reporting for CB-Msg3 EDT

LS R2-2506299 In Release 19 IoT NTN, RAN2 is specifying support for Contention-Based Msg3 Early Data Transmission (CB-Msg3 EDT). In this scheme the UE can perform EDT without transmitting Msg1 and receiving Msg2, in other words directly transmitting Msg3 in a contention-based manner. As part of this, RAN2 has discussed whether CQI reporting in CB-Msg3 EDT procedure shall be supported or not. RAN2 has come to the following agreement, which applies to both NB-IoT and eMTC: We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4)

For NB-IoT UE, the legacy MSG3-based CQI report requirement in Clause 6.6A.2.6 can be taken as the reference, with removing the non-anchor carrier part. Besides, the period to derive reported NPDCCH repetition level shall also re-defined. The period T shall be the period before CB-Msg3 transmission used for NRSRP measurement for enhanced coverage level estimation.

Proposal 3: For NB-IoT UE, take the legacy MSG3-based CQI report requirement in Clause 6.6A.2.6 as the reference, with removing the non-anchor carrier part, and re-define the period to derive reported NPDCCH repetition level as the period before CB-Msg3 transmission used for NRSRP measurement for enhanced coverage level estimation.

For eMTC UE, the legacy MSG3-based CQI report requirement is defined in Clause 4.7A.3. Due to RSRP measurement before PRACH transmission is mandatory for CE-ModeB UE in enhanced coverage and optional for eMTC UE in normal coverage, the period to derive reported NPDCCH repetition level only contains the period from the beginning of the random access response to the beginning of PUSCH for DL channel quality reporting, which can not directly reused for CB-Msg3 based CQI report.

With above background, since CB-Msg3 EDT only applies for CE-ModeA UE which operates in normal coverage, with non mandatory RSRP measurement before PRACH transmission, and without preceding Msg1 and Msg2 procedure, how and when does the UE measure the channel quality for deriving the reported MPDCCH repetition level or aggregation level shall be further studied. Currently, we fail to see the feasibility unless RAN2 introduce additional measurement procedure. If RAN4 achieve such agreements, LS to RAN2 is needed.

Proposal 4: For eMTC CE-ModeA UE, further study how and when does the UE measure the channel quality for deriving the reported MPDCCH repetition level or aggregation level.

Proposal 5: If RAN4 fail to see the feasibility of introducing the CQI reporting for CB-Msg3 EDT for eMTC UE, LS is needed to inform RAN2.

We also bring a companion CR based on our above proposals.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: Include following UE behavior when receiving the CB-Msg4 in the Clause “Correct behaviour when receiving CB-Msg4” according to RAN2’s agreements:

- Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring PDCCH.

- Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE.

Proposal 2: For “Correct behaviour when not receiving CB-Msg4”, include following UE behavior when the CB-Msg3ResponseTimer expires and the UE has reached maximum number of transmissions according to RAN2’s agreements:

- When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, and the UE has not received a CB-Msg4, or the UE doesn’t successfully decode the corresponding PDSCH after the CB-Msg3ResponseTimer has expired, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation.

Proposal 3: For NB-IoT UE, take the legacy MSG3-based CQI report requirement in Clause 6.6A.2.6 as the reference, with removing the non-anchor carrier part, and re-define the period to derive reported NPDCCH repetition level as the period before CB-Msg3 transmission used for NRSRP measurement for enhanced coverage level estimation.

Proposal 4: For eMTC CE-ModeA UE, further study how and when does the UE measure the channel quality for deriving the reported MPDCCH repetition level or aggregation level.

Proposal 5: If RAN4 fail to see the feasibility of introducing the CQI reporting for CB-Msg3 EDT for eMTC UE, LS is needed to inform RAN2.

- Reference

- R4-2512148, WF on RRM requirements for IoT_NTN_Ph3

- R2-2506299, LS on CQI reporting for CB-Msg3 EDT, RAN2