3GPP TSG-RAN4 Meeting #116bis	R4-2513683

# Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | DraftCR | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction of CB-msg3 based random access requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#131 meeting, some further agreements was achieved which have impact on RAN4 spec, including - When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, the MAC indicates failure of CB-Msg3 transmission to the upper layer (i.e. RRC). - If the PDCCH is successfully decoded before CB-Msg3ResponseTimer expires, but the corresponding PDSCH is successfully decoded after the timer has expired, the MAC PDU is treated the same as the one successfully decoded before the timer expires. - When the RRC layer receives a CB-Msg3 EDT failure indication from the MAC layer the RRC procedure is re-initiated (how to avoid data loss is up to UE implementation). Which procedure (e.g. EDT, 4-step RACH, CB-Msg3-EDT) is initiated is up to UE implementation - Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4 without an RRC message, the UE considers UL transmission as successful but the UE does not consider the CB-Msg3-EDT procedure has ended and continues monitoring PDCCH - Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including both an RRC message and a C-RNTI, the UE behaves as if it received an empty EarlyDataComplete message, terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE - We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4) Some other mistakes shall be revised |  |  |  |  |  |  |  |  |
| Summary of change: |  | Supplement incomplete content for UE correct behavior when or when not receiving CB-Msg4. Supplement CB-MSG3-based CQI requirement requirement Revision on some misalignment part |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec for IOT-NTN phase3 will not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A, 6.6A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This draftCR is based on agreed BigCR R4-2512899 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change#1>

6.2.4A	Random Access Requirements for Cat-M1 UEs with CB-Msg3 EDT for Satellite Access

The requirement in this clause applies for Cat-M1 CE Mode A UE for satellite access when CB-Msg3-EDT is configured by CB-Msg3-ConfigSIB-r19 and the UE supports ntn-MO-CB-Msg3-EDT-UP-r19. The UE in IDLE mode is allowed to initiate a contention-based PUSCH transmission.

The UE shall have the capability to calculate the transmission power for the initial CB-Msg3 transmission and the subsequent CB-Msg3 replica transmissions according to the formula defined in TS 36.213.

6.2.4A.1	Correct behaviour when transmitting CB-Msg3

UE shall also execute the CB-Msg3-EDT procedure defined in clause 5.1x in TS 36.321 [17] using the PUSCH configuration contained in CB-Msg3-ConfigSIB-r19 in TS 36.331 [2]. The UE shall transmit CB-Msg3 replicas within the CB-Msg3 window, according to network configuration.

6.2.4A.2	Correct behaviour when receiving CB-Msg4

The UE shall send HARQ-ACK if the Contention Resolution is successful if HARQ-ACK resource is provided.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring PDCCH.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE.

If the Msg3ResponseTimer expires, the maximum attempt number of CB-Msg3 transmissions is not achieved, and the UE has received a PDCCH message that includes a UE Contention Resolution Identity MAC control element but the UE Contention Resolution Identity included in the MAC control element doesn’t match the CCCH SDU transmitted in the uplink message, the UE shall re-attempt a CB-Msg3 transmission in a new CB-Msg3 window with the calculated PUSCH transmission power.

6.2.4A.3	Correct behaviour when not receiving CB-Msg4

The UE shall re-attempt a CB-MSG3 transmission in a new CB-MSG3 window if the contention resolution failure is detected and the UE has not reached the maximum number of transmissions.

When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, and the UE has not received a CB-Msg4, or the UE doesn’t successfully decode the corresponding PDSCH after the CB-Msg3ResponseTimer has expired, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation.

# <End of change#1>

# <Start of change#2>

6.6A.4	Requirements for CB-Msg3-EDT procedure

The requirement in this clause applies for for UE category NB-IoT for satellite access when CB-Msg3-EDT is configured by CB-Msg3-ConfigSIB-NB-r19 and the UE supports ntn-MO-CB-Msg3-EDT-UP-r19. The UE in IDLE mode is allowed to initiate a contention-based NPUSCH transmission.

The UE shall have the capability to calculate the transmission power for the initial CB-Msg3 transmission and the subsequent CB-Msg3 replica transmissions according to the formula defined in TS 36.213.

6.6A.4.1 Correct behaviour when transmitting CB-Msg3

UE shall also execute the CB-Msg3-EDT procedure defined in clause 5.1x in TS 36.321 [17] using the NPUSCH configuration contained in CB-Msg3-ConfigSIB-NB-r19 in TS 36.331 [2].

The UE shall transmit CB-Msg3 replicas within the CB-Msg3 window, according to network configuration.

6.6A.4.2	Correct behaviour when receiving a CB-Msg4

The UE shall send HARQ-ACK if the Contention Resolution is successful if HARQ-ACK resource is provided.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring NPDCCH.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE.

If the Msg3ResponseTimer expires, the maximum attempt number of CB-Msg3 transmissions is not achieved, and the UE has received a NPDSCH message that includes a UE Contention Resolution Identity MAC control element but the UE Contention Resolution Identity included in the MAC control element doesn’t match the CCCH SDU transmitted in the uplink message, the UE shall re-attempt a CB-Msg3 transmission in a new CB-Msg3 window with the calculated NPUSCH transmission power.

6.6A.4.3	Correct behaviour when detecting CB-Msg3-EDT failure

The UE shall re-attempt a CB-MSG3 transmission in a new CB-MSG3 window if the contention resolution failure is detected and the UE has not reached the maximum number of transmissions.

When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, and the UE has not received a CB-Msg4, or the UE doesn’t successfully decode the corresponding NPDSCH after the CB-Msg3ResponseTimer has expired, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation.

6.6A.4.4	CB-MSG3-based channel quality report for UE Category NB1

The requirements in this clause shall apply for UE supporting DL channel quality reporting for UE Category NB1 as defined in TS 36.331 [2] section 5.3.3.3, 5.3.3.3a, and 5.3.7.4.

The DL channel quality provides the serving eNB with information about the minimum NPDCCH repetition level to satisfy the hypothetical NPDCCH block error rate of 1% with the parameters specified in Table 6.6A.4.4-1.

Table 6.6A.4.4-1: NPDCCH transmission parameters for downlink quality reporting

| Parameters | Values |
| --- | --- |
| DCI format | Format N1 |
| Number of information bits (excluding CRC) | 23bits |
| System bandwidth | 200kHz |
| Aggregation level | 2 |
| DRX | OFF |

For channel quality reporting in the anchor carrier, the reported NPDCCH repetition level shall be derived from the channel quality measured in the period T in the carrier where the CB-Msg3 is going to be transmitted, where

-	T is the period before CB-Msg3 transmission used for NRSRP measurement for enhanced coverage level estimation.

The NPDCCH repetition level for CQI-NPDCCH-NB and CQI-NPDCCH-Short-NB is chosen from the supported NPDCCH repetition levels [3]. The report mapping is defined in 9.1.22.15.

The UE shall satisfy the downlink channel quality measurement accuracy requirements as specified in 9.1.22.16.

# <End of change#2>