3GPP TSG-RAN WG4 Meeting # 116-bis						R4-2513694

Prague, 13th – 17th October 2025

Agenda item:			7.11.3

Source:	Qualcomm

Title:	Discussion on Ambient IoT Coexistence in outdoor scenario

Document for:	Approval

# 1  Introduction

Through Rel-18 and Rel-19 a set of SIs and WIs were completed across 3GPP’s WGs, leading to the specification of Device 1 (having ~1 μW power consumption, and with D2R transmission by backscattering) for use in Deployment Scenario 1 with Topology 1, i.e., by direct connection between a BS reader and the A-IoT device.

In Rel-20, the follow-up SI by supporting outdoor scenarios was approved and RAN4 will conduct the coexistence study between Device 2b/Device C and NR/LTE in the outdoor scenarios using band n8 as an example band [1].

This paper provides our considerations on the coexistence scenarios and system assumptions for ambient IoT in outdoor scenario.

# 2  Discussion for co-existence scenario

## 2.1 Spectrum usage plan

The studied device type is device 2b and device C. The device’s D2R transmission is generated internally by the device and not relying on the backscattered power. Therefore, the R2D link is transmitted in the DL spectrum and D2R link is transmitted in the UL spectrum. Similar with Rel-19 study, there will have standalone mode and in-band mode for ambient IoT.

Observation 1: R2D link is transmitted in the DL spectrum and D2R link is transmitted in the UL spectrum. Both standalone mode and in-band mode for ambient IoT are in the scope.

## 2.2 Outdoor topology

A shown in Figure 1, the typical coordinated operation is proposed to be used for the outdoor topology, 19 sites 57sectors with wrap around. The BS and reader are co-located.

Figure 1. Outdoor Topology

Proposal 1: The typical coordinated topology in Figure 1 is proposed to be used for the outdoor topology, 19sites 57sectors with wrap around. The BS and reader are co-located.

## 2.3 Coexistence case between NR and ambient IoT

The coexistence cases are proposed to study as below Table 1. For the inter-system coexistence, the ACLR and ACS would be used and for intra-system coexistence, IBE/IBB values are needed to study.

Table 1. Coexistence Cases

| No. | Aggressor | Victim | ACLR/ACS for standalone, IBE/IBB for in-band |  |
| --- | --- | --- | --- | --- |
| 1 | Ambient IoT R2D | NR DL | BS ACLR | UE ACS |
| 2 | Ambient IoT D2R | NR UL | TAG ACLR | BS ACS |
| 3 | NR DL | Ambient IoT R2D | BS ACLR | TAG ACS |
| 4 | NR UL | Ambient IoT D2R | UE ACLR | BS ACS |

Proposal 2: The coexistence cases listed in Table 1 are proposed to be studied.

## 2.4 Simulation assumptions

The system parameters for coexistence study are proposed as Table 2.

Table 2. Simulation Parameter Assumptions

| System Parameter | Typical deployment | Ambient IoT |  |
| --- | --- | --- | --- |
| NR BS deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |  |
| Frequency and bandwidth | 900MHz, 10MHz with 15kHz SCS | 900MHz, R2D 180KHz with 15kHz SCS; D2R 15KHz |  |
| NR BS Inter-site distance | 500m for UMa refer to TR 38.901 | 500m for UMa refer to TR 38.901 |  |
| Minimum NR BS – NR UE distance (2D) | 35 m | 35m |  |
| UE dropping | uniformly distributed, 100% outdoor | Uniformly distributed, 100% outdoor as baseline, TBD% in cargo trucks with penetration loss |  |
| UE number | DL active UE: 1 UE per cell | 1 active A-IoT device under one reader at the same time; TBD for FDM |  |
|  | UL active UE: 3 UE per cell | 1 active A-IoT device |  |
| BS Parameter | Typical deployment | Reader Parameter |  |
| BS Tx power (dBm) | 46 | 43 for stand alone, 38/33 for in-band |  |
| Height of NR BS (m) | 25 | 25 |  |
| BS Noise Figure(dB) | 5 | 5 |  |
| BS antenna gain (dBi) and antenna pattern | Directional antenna | same with typical BS |  |
| UL control Parameter | TR 36.942 Section 9.1 | TBD on power control mechanism under discussion in RAN1 |  |
| NR UE Parameter | UE | Device 2b | Device C |
| UE TX power in dBm | -40 to 23 | -10 dBm | 5 dBm |
| NR UE Antenna gain (dBi) | 0 | 0 | 0 |
| Height of UE (m) | 1.5 | 1.5 | 1.5 |
| NR UE Noise Figure (dB） | 9 | 15 | 13 |
| SINR mapping curve | -10 dB | depending on RAN1 discussion | depending on RAN1 discussion |
| Antenna configuration | Omni direction antenna | Omni direction antenna | Omni direction antenna |

Table 3. BS Antenna Pattern

| NR Macro-BS Parameter | Values for evaluation |
| --- | --- |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} |
| Antenna gain (dBi) | 15 (including feeder loss) |

Proposal 3: The system assumptions listed in Table 2 and BS antenna pattern in Table 3 are proposed to be used in the coexistence study.

Observation 2: The reader Tx power for in-band mode and stand alone mode is different considering the allowable max PSD for BS. Reader Tx power is 43dBm for standalone mode and 38dBm or 33dBm for in-band mode. Considering the RF parameters used would also be different for in-band and standalone, both inter-system coexistence and in-band coexistence would be needed to study to verify the interference impact.

Proposal 4: Both out-of-band coexistence and in-band coexistence would be needed to study to verify the interference impact.  For the pathloss model used in the system, UMa scenario is proposed to be used as baseline. The penetration loss can be 0dB for outdoor, 20dB for in car scenario and 80dB in the metal shipping container.

Proposal 5: UMa (ISD=500m) pathloss model is proposed to be used in the coexistence.

Proposal 6: RAN4 to take 100% outdoor device as the baseline and to discuss whether to consider the ratio of device in cargo trucks. The penetration loss for in-cargo device can be set as 20dB for in car and 80dB for metal shipping containers.

## 2.5 Coexistence criteria

The coexistence criteria between NR and ambient IOT is proposed to reuse the criteria in TR 38.769.

Proposal 7: The coexistence criteria are proposed as below:

- For NR system, use 5% throughput loss as performance metric as legacy.

- For inter-system interference (between AIOT and NR): If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible. For SINR degradation, SINR refers to the 5% and 50% CDF SINR.

# 3  Conclusion

In summary, we have following observations and proposals.

Observation 1: R2D link is transmitted in the DL spectrum and D2R link is transmitted in the UL spectrum. Both standalone mode and in-band mode for ambient IoT are in the scope.

Proposal 1: The typical coordinated topology in Figure 1 is proposed to be used for the outdoor topology, 19sites 57sectors with wrap around. The BS and reader are co-located.   Proposal 2: The coexistence cases listed in Table 1 are proposed to be studied.

Proposal 3: The system assumptions listed in Table 2 and BS antenna pattern in Table 3 are proposed to be used in the coexistence study.

Observation 2: The reader Tx power for in-band mode and stand alone mode are different considering the allowable max PSD for BS.  Reader Tx power is 43dBm for standalone mode and 38dBm or 33dBm for in-band mode.  Considering the RF parameters used would also be different for in-band and standalone, both inter-system coexistence and in-band coexistence would be needed to study to verify the interference impact.

Proposal 4: Both out-of-band coexistence and in-band coexistence would be needed to study to verify the interference impact.

Proposal 5: UMa (ISD=500m) pathloss model is proposed to be used in the coexistence.

Proposal 6: RAN4 to take 100% outdoor device as the baseline and to discuss whether to consider the ratio of device in cargo trucks. The penetration loss for in cargo device can be set as 20dB for in car and 80dB for metal shipping containers.

Proposal 7: The coexistence criteria are proposed as below

- For NR system, use 5% throughput loss as performance metric as legacy.

- For inter-system interference (between AIOT and NR): If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible. For SINR degradation, SINR refers to the 5% and 50% CDF SINR.

# 4  References

[1] RP-251884, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for

active devices