3GPP TSG-RAN WG4 Meeting #116bis	R4- 2513697

# Prague, Czech Republic, Oct. 13 – 17, 2025

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for RRM measurement prediction

Agenda item:	7.12.3

Document for:	Discussion

Introduction

In this contribution, we provide our high-level views for measurement event prediction use case on AI/ML mobility based on revised WID [1] in RANP#109 meeting and WF[2].

Discussion

This RAN4#116bis meeting in October is the first meeting for AI/ML Mobility WI. Although RAN4 discussed AI/ML Mobility SI during Rel-19 period, some issues agreed to discuss further in WI phase. So, We provide our views on that issues based on WF[2], and high-level views also based on the WID[1].

The objectives of RAN4 part are captured below:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

According to the WID we can categorize the issues as below three parts.

- Core requirements for RRM measurement prediction and indirect measurement event prediction

- Investigation whether the core requirements for RRM measurement prediction can be resued for indirect measurement event prediction

- LCM-related core requirements to support the LCM functionality management

- Category #1: Core requirements for RRM measurement prediction

According to the WID[1], RAN4 need to consider below use cases and scenarios.

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported.

The temporal domain prediction case A is continuos measurement results in PW are predicted by continuous historical measurement results in OW. The OW and PW can slide forward with either sampling period (with sliding L1/L3 filtering option which described in figure 5.1-1 at TR38.744) or measurement period (with non-sliding L1/L3 filtering option which described in figure 5.1-2 at TR38.744).

Below figure 5.2.1.1-1 is captured from TR38.744 which show an example for temporal domain case A scenario. Although the complexity may increase in case A scenario, the handover performance is expected to improve.

The temporal domain prediction case B is similar to case A except that the measurement results in previous PW are skipped during window sliding as below examples which captured from TR38.744.

Both case A and case B involve the variables OW and PW, and RAN4 need to define the lengths of these variables. The AI/ML model can depends on the size of input and output. So, the definition of length of OW and PW should be sample unit not time unit. E.g. 10, 20 or 40 samples.

Proposal 1: RAN4 need to define the lengths of OW and PW. Regarding the unit of length, our preference is using number of samples instead of length of time for easy mapping to AI/ML model.

Issue 2-1: Impacts of measurement error on prediction accuracy

In the last RAN4#116 meeting, RAN4 discussed regarding the impact of measurement error on prediction accuracy and made the conclusion as below:

Agreement: RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy: - As a baseline: - For BB error, use link level simulation to generate L3-RSRP difference as baseband error. - For RF error model, use a truncated Gaussian distribution. - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

For RF error model, RAN4 decided to use a truncated Gaussian distribution and further discuss either the same or different RF error value at least for different frequency layers and/or different RF chains. We think the different frequency layers and/or different RF chains should have independent each other. So, our preference is different RF error value should be considered for different frequency layers and/or different RF chains.

Proposal 2: Different RF error value should be considered for different frequency layers and/or different RF chains.

Issue 2-2: Relative RSRP accuracy definition

Agreement: Relative RSRP accuracy definition - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - Note: It is not precluded to update the definition based on further RAN2 progress in WI phase.

According to the WID[1], For UE sided model, the L3 beam level prediction is not supported. So, the relative RSRP accuracy for beam level measurements is not necessary.

Proposal 3: The relative RSRP accuracy for beam level measurements is not necessary.

- Category #2: Investigation whether the core requirements for RRM measurement prediction can be reused for indirect measurement event prediction

This category can be discussed in indirect measurement event prediction agenda.

- Category #3: LCM-related core requirements to support the LCM functionality management

To support the LCM functionality management, the post deployment testing can be necessary. Based on the results of post deployment testing, the AI/ML model can be fallback to legacy or update to new AI/ML model. For checking the AI/ML model in the field, the case A scenario seems to be more suitable. So, periodically or by event-triggered, the case A scenario can be operated for checking the AI/ML model.

Proposal 4: RAN4 need to discuss how to check the AI/ML model in the field for RRM measurement prediction use case.

Proposal 5: RAN4 need to discuss how to trigger the checking operation of the AI/ML model in the field for RRM measurement prediction use case.

Conclusion

In this contribution, we provide our high-level views for measurement event prediction use case on AI/ML mobility based on revised WID [1] in RANP#109 meeting and WF[2].

Proposal 1: RAN4 need to define the lengths of OW and PW. Regarding the unit of length, our preference is using number of samples instead of length of time for easy mapping to AI/ML model.

Proposal 2: Different RF error value should be considered for different frequency layers and/or different RF chains.

Proposal 3: The relative RSRP accuracy for beam level measurements is not necessary.

Proposal 4: RAN4 need to discuss how to check the AI/ML model in the field for RRM measurement prediction use case.

Proposal 5: RAN4 need to discuss how to trigger the checking operation of the AI/ML model in the field for RRM measurement prediction use case.

Reference

[1] RP-252899, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR,” OPPO, Interdigital

[2] R4-2512127, “WF on AI/ML Mobility Part 1,” Moderator (Nokia)