3GPP TSG-RAN4 Meeting #116bis	R4-2513701

Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on core requirements maintenance for Rel-19 MIMO asymmetric TRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The changes are based on the Big CR R4-2509238. For the case “UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s)”, based on RAN1 LS reply R1-2502956, single DL reference timing does not apply to the case. Thus, current description about “same for both TAGs” is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the sentece “and is the same for both TAGs” for the case UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state or when transmitting PUSCH on CG resources for SDT in RRC_INACTIVE. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available during at least one discovery burst transmission window, at the UE due to DL CCA failures at gNB during the last 1280 ms; otherwise the reference cell on the carrier frequency subject to CCA is considered as available at the UE.

For UE supporting rach-EarlyTA-Measurement-r18 for LTM, and if the candidate cell is a neighbour cell or a secondary serving cell without uplink carrier,

-	UE shall have capability to follow the frame timing of the reference cell. The PRACH transmission for early TA acquisition takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE shall use this candidate cell as the reference cell for deriving transmit timing. UE initial transmit timing accuracy is defined in the following requirements.

For multi-DCI based multi-TRP operation with two TAs and for asymmetric DL sTRP/UL mTRP deployment with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point.

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal.

For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is configured with PL offset in joint/UL TCI state(s) and UE does not expect to receive SSB from UL TRP(s),  the reference point for PRACH/PUCCH/PUSCH/SRS is the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DL or Joint TCI state.

For  asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or -UL TCI states TS 38.331 [2] having the same TAG as the uplink signal.

<End of Change 1>