3GPP TSG-RAN WG4 Meeting #116bis	R4-2513708

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 OD-SIB1

Agenda Item:	6.15.2.3

Document for:	Discussion

- Introduction

The core part of Rel-19 NES was completed in last meeting with agreements captured in [1]. In this contribution, we provide our analysis on performance part for OD-SIB1.

- Discussion

The performance part has been initially discussed in last meeting with following agreements.

Issue 3-2-1: Test case - Background In RAN4 #115 meeting, Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.. - Proposals - Option 1: RAN4 not to define OD-SIB1 RRM performance test case. - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

In core part discussion, companies have different views on whether and how to define paging interruption requirements for OD-SIB1. Based on following RAN1 agreements, UE is allowed to try to monitor PDCCH for SIB1 before UL-WUS transmission. Thus, it is hard to define a concrete interruption length, but it is agree to define TC with longer TSI-NR.

RAN1 Agreements in RAN1#120bis Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=123 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

Based on above, RAN4 can introduce on test case to verify OD-SIB1 specific procedure. In details, the test case is based on existing Cell re-selection TC with necessary modifications. Taking A.6.1.1.1 as example, the test requirements are shown as follows, the Cell re-selection delay is defined as Tdetect, NR_Intra + TSI-NR, where TSI-NR is assumed as 1280ms in the test. In the OD-SIB1 test cases, Cell 2 is a NES cell without SIB1 transmission. UE shall transmit UL WUS to NES Cell, which can be taken as the endpoint of the test cases. Since it is allowed to try to search for the PDCCH before UL WUS transmission, existing value of TSI-NR is a reasonable value for UE to try to monitor SIB1, and UE shall be able to transmit UL WUS no later than the first RO after TSI-NR.

- A.6.1.1.1.3 Test Requirements The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2. The cell re-selection delay to a newly detectable cell shall be less than 34 s. The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 1. The cell re-selection delay to an already detected cell shall be less than 8 s. The rate of correct cell reselections observed during repeated tests shall be at least 90 %. NOTE: The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR, Where: Tdetect, NR_Intra See table 4.2.2.3-1 in clause 4.2.2.3 Tevaluate, NR_ intra See table 4.2.2.3-1 in clause 4.2.2.3 TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case. This gives a total of 33.28 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.68 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

Proposal 1: RAN4 to define one Cell re-selection test case for OD-SIB1. The details are proposed as follows:

- Target Cell (Cell2) is a NES Cell.

- The ending point of the TC is UL WUS transmission.

- UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

- Conclusion

Proposal 1: RAN4 to define one Cell re-selection test case for OD-SIB1. The details are proposed as follows:

- Target Cell (Cell2) is a NES Cell.

- The ending point of the TC is UL WUS transmission.

- UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

References

[1] R4-2512327	WF on RRM requirements for Netw_Energy_NR_enh_Part1