3GPP TSG-RAN WG4 Meeting #116bis	R4-2513709

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 AIOT

Agenda Item:	6.14.3

Document for:	Discussion

- Introduction

The core part of Rel-19 AIoT was completed in last meeting with agreements captured in [1]. In this contribution, we provide our initial analysis on perform requirements for AIoT device.

- Discussion

Based on core requirements discussion, RAN4 define Random access and D2R timing requirements for A-Iot device 1. For performance part, RAN4 initiate the discussion in last meeting and achieve following agreements.

Issue 2-1-1: Test scenario Agreement: - The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test. Issue 2-1-2: Test configuration - Proposals from companies: - Option 1 (CATT): - RAN4 to discuss the following test parameters as a starting point: - CW transmitting power: [33] dBm. - CW2D distance: [5, 10, 20] m. - R2D SCS = 15 kHz only. - R2D transmission bandwidth configuration: [1 PRB, 2 PRB, 3 PRB, 4 PRB]. - Option 2 (OPPO): - Set X=1 for MSG 1 transmission in the test case. - Consider the multiple Toffset1 values as different configuration in the test case: - Configuration 1: Toffset1 = 1333.4us, reference D2R chip length is 133.33us - Configuration 2: Toffset1 = 666.6us, reference D2R chip length is 33.33us - Configuration 3: Toffset1 = 132.32us, reference D2R chip length is 4.17us - Configuration 4: Toffset1 = 33.33us, reference D2R chip length is 2.08us, R2D chip length is 11.11us - Option 3 (Xiaomi): - The common testing configuration for Rel19 AIoT RRM test cases can be defined as: - SCS = 15KHz - BW =[180kHz] - Device 1 only - Option 4 (CMCC): - For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 /Toffset2/Toffset3/Toffset4 and timing error due to SFO.

Regarding whether and how to define test cases for AIoT device, it should be aware that AIoT device is extremely low case device, and each added functionality and test case will increase the cost of device implementation, which is harmful to the ecosystem. One direction is to only verify necessary and essential requirements, and try to combined the test cases for demod and RF as possible.

Based on RF requirements and test method discussion, the RF requirements are verified via CRA procedure and correct timing relation is considered as the criteria for a success response. The relevant details could be found in TS 38.191 which is shown as follows:

TS 38.191 ## B.1 General Unless stated otherwise, the transmitter and receiver performances are measured through the contention-free access (CFA) procedure, consisting of one R2D message and one D2R message. - Confirm that the DUT can send correct response in D2R channel within correct timing relationship and the test equipment is able to decode the responses by measuring the power received in both θ-polarization and ϕ-polarization either simultaneously or sequentially.

Thus, there is no need to have duplicated test cases which may increase the test burden of A-IoT device. From our understanding, the unique RRM characteristic is timing error for D2R transmission. Thus, if RAN4 agreed to define TC for AIoT device, one test case for CBRA is sufficient and the timing error requirements is to be verified in the same test cases.

Proposal 1: RAN4 to define one test case for CBRA and the D2R timing error requirements are verified in the same test case.

Regarding the details test setup, it should be guaranteed that the test is conducted under the conditions that the D2R link and R2D link is reliable. Thus, based on the test method and RF requirements as defined in TS 38.191, it is proposed to test RRM test case is performed base on the test procedures for sensitivity, and the RRM test is only performed at peak direction.

Proposal 2: RRM test case is performed based on the test procedures for sensitivity, and the RRM test is only performed at peak direction.

Besides, the test procedure and setup, the details test requirements are proposed as follows. The motivation is to avoid unnecessary behaviours to be verified (e.g., Msg2 is no received, transmission falure), which maybe verified in RAN5 signalling test.

Proposal 3: Take following test procedure and test requirements as baseline.

- TE sends paging A-IoT Paging message with access type as CBRA, with Paging ID absence.

  - Number of Access Occasions is configured as 2 or 4 to limit the test time.

  - Time Resource Indication and Frequency Resource Indication is configured as 1.

- Device shall select one access occasion accordingly and transmit Access Random ID and the timing error shall be verified at the same time.

- TE monitors occasions whether there is Access Random ID reflected by device, and transmit Access trigger message is there is no Access Random ID detected, until and Access Random ID is detected or the all-access occasions has passed.

- If an Access Random ID is received, TE transmits Random ID Response which is identical to Access Random ID.

- Device shall transmit D2R Upper Layer Data Transfer and the timing error shall be verified at the same time.

- Conclusion

Proposal 1: RAN4 to define one test case for CBRA and the D2R timing error requirements are verified in the same test case.

Proposal 2: RRM test case is performed based on the test procedures for sensitivity, and the RRM test is only performed at peak direction.

Proposal 3: Take following test procedure and test requirements as baseline.

- TE sends paging A-IoT Paging message with access type as CBRA, with Paging ID absence.

  - Number of Access Occasions is configured as 2 or 4 to limit the test time.

  - Time Resource Indication and Frequency Resource Indication is configured as 1.

- Device shall select one access occasion accordingly and transmit Access Random ID and the timing error shall be verified at the same time.

- TE monitors occasions whether there is Access Random ID reflected by device, and transmit Access trigger message is there is no Access Random ID detected, until and Access Random ID is detected or the all-access occasions has passed.

- If an Access Random ID is received, TE transmits Random ID Response which is identical to Access Random ID.

- Device shall transmit D2R Upper Layer Data Transfer and the timing error shall be verified at the same time.

References

- R4-2512134, WF on RRM requirements for Ambient_IoT_Solutions