3GPP TSG-RAN WG4 Meeting #116bis	R4-2513710

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on RRM requirements for Rel-20 MIMO

Agenda Item:	7.10.3

Document for:	Discussion

- Introduction

The Rel-20 MIMO WI was approved in [1], with following objectives. RAN4 RRM work is scheduled to start from this meeting, and we provide initial analysis on RRM impacts for each objective.

RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

- Discussion

  - UL capacity and coverage enhancement

For UL capacity and coverage enhancement, there is potential impacts on SRS AS switching requirements considering the Cross-slot SRS enhancement. In Rel-17, RAN4 defined SRS AS interruption requirements based on per SRS resource, and it is assumed that SRS symbols are allocated in the last 6 symbols of a slot. And the interruption length is rounded up to number of symbols/slots. For cross-slot SRS, RAN4 needs to re-evaluate the case that one SRS resource could occupy two adjacent slots, and the current SRS AS requirements can not apply anymore.

Observation 1: Existing SRS AS requirements applies for the case the SRS resource is allocated in the last 6 symbols within a slot, which can not apply to the cross-slot SRS resource.

Proposal 1: RAN4 to define SRS AS requirements for the case for cross-slot SRS resource.

RAN1 has initiated the discuss in last meeting with agreement shown as follows:

Agreement: For a P/SP cross-slot SRS resource, the slot offset configured to the SRS resource refers to the first of the two slots spanned by the SRS resource. Agreement: When there is a cross-slot SRS resource in the end of a S slot and the beginning of a U slot in a serving cell, support transmitting PUSCH with a priority index 0 and corresponding DMRS after this cross-slot SRS in the U slot in the serving cell. Agreement: Support at least the following scenarios for cross-slot SRS transmission: - Scenario 1: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S slot and a second consecutive U slot. - FFS whether Scenario 2 below is supported. - Scenario 2: an aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted in a first S slot, and at least one another SRS resource with time-domain resource transmitted in a second consecutive U slot. Agreement: For a given AP-SRS resource set in case of SRS transmission across two adjacent S+U slots, study the following alternatives and down-select one to determine “available slot” in the next meeting: - Alt-0 (per SRS resource set): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-1 (per SRS resource): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-2 (per slot): An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resource(s) with same slot offset in the SRS resource set, and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. Agreement: To determine the time-domain location of cross-slot SRS, - One set of time-domain resource allocation related parameters (i.e., startPosition, nrofSymbols, and repetitionFactor) is configured for the SRS resource without restriction on “within a slot”. - For the index of each SRS symbol l', it is the same as legacy spec., i.e., l'=0, 1, …, NsymbSRS-1. - The offset loffset∈0,1,…,13 counts symbols backwards from the end of the starting slot of the resource

Based on RAN 1 progress, at least Scenario 1 is supported that at least one SRS resource is crossing two slots (S+U). For other scenarios, they are still under discussion in RAN4.

Proposal 2: RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

  - DL CSI acquisition enhancement

For DL CSI acquisition enhancement, there are following three cases considered in the WID:

- Case1: UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4

- Case2: SCell activation in CONNECTED

- Case3: switching out of SCell dormancy in CONNECTED

And based on WID descriptions, the scenarios are further restricted as: FR1 only, and based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy.

Based on the first RAN1 meeting, the progress is summarized as follows:

Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support at least aperiodic SRS-AS transmission triggered via MSG4 of 4-Step RACH. Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support aperiodic CSI reporting triggered via MSG4 of 4-Step RACH based on the followings: - The aperiodic CSI reporting is transmitted on PUSCH. - Support at least aperiodic CSI-RS for CSI associated with the aperiodic CSI reporting - Support PMI-based reporting with wideband PMI based on Rel-15 Type-I SP codebook and wideband CQI - FFS: Which report quantity(s) can be configured - Support PMI-free reporting with wideband CQI - FFS: Which report quantity can be configured Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering. - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3. - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting. Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering. Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately. Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of SRS-AS/CSI-RS/CSI when UE transition from IDLE to CONNECTED mode, study the following at least three options for Step-1 and Step-2: - Option-1: NW can provide the resource/report configuration in SIBx based on only one UE capability assumption, and UE can report through MSG3 whether the resource/report configuration received in SIBx is supported. - Option-2: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 which resource/report configuration(s) received in SIBx is/are supported. - Option-3: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 the supported capability(s) of early SRS/CSI/CSI-RS triggering (e.g., whether to support this feature, max number of CSI-RS ports, xTyR for SRS-AS, max bandwidth of the CSI-RS/SRS-AS etc.). Note: The term “capability” or “UE capability” above does not mean legacy RRC based UE capability.

For case 1, it could be observed that AP SRS/AP CSI-RS reception/AP CSI reporting is supported by RAN1 so far. For case 2, similarly, AP SRS/AP CSI-RS reception/AP CSI reporting is supported. For case 3, there is no much progress in RAN1 so far. From our understanding, case 2 needs more RAN4 discussion than other two cases. It is also agreed in RAN1 that the requirements of the timeline of the aperiodic SRS-AS/CSI-RS/CSI reporting. For SCell activation, there are many SCell activation to be considered (e.g., known/unknown, with/without PUCCH), for different cases, the timeline that UE is able to transmit AP SRS and receive AP CSI-RS are complicated. For case 1 and case 3, the situation is much simpler. For case 1, the AP SRS/CSI-RS happens after Msg4 which means the UE is already well synchronized. Similar, for case 3, when UE is in dormant BWP, UE is continuously performing P/SP CSI measuring.

Proposal 3: For early SRS/CSI/CSI-RS triggering, RAN4 to discuss whether to define RRM requirements for following three cases:

- Case1: UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4

- Case2: SCell activation in CONNECTED

- Case3: switching out of SCell dormancy in CONNECTED

Observation 2: The timeline of case 2 (SCell activation) is much more complicated than other cases.

Proposal 4: RAN4 to discuss the requirements for timeline for early SRS/CSI/CSI-RS triggering for SCell activation.

Proposal 5: RAN4 to discuss the applicable SCell activation cases for early SRS/CSI/CSI-RS triggering.

For the other objective (i.e., 2.b), which is to reduce the density for large CSI-RS port number, from our understanding, there is no RRM impacts.

Proposal 6: No RRM impacts for CSI-RS density reduction.

- Conclusion

Observation 1: Existing SRS AS requirements applies for the case the SRS resource is allocated in the last 6 symbols within a slot, which can not apply to the cross-slot SRS resource.

Proposal 1: RAN4 to define SRS AS requirements for the case for cross-slot SRS resource.

Proposal 2: RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

Proposal 3: For early SRS/CSI/CSI-RS triggering, RAN4 to discuss whether to define RRM requirements for following three cases:

- Case1: UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4

- Case2: SCell activation in CONNECTED

- Case3: switching out of SCell dormancy in CONNECTED

Observation 2: The timeline of case 2 (SCell activation) is much more complicated than other cases.

Proposal 4: RAN4 to discuss the requirements for timeline for early SRS/CSI/CSI-RS triggering for SCell activation.

Proposal 5: RAN4 to discuss the applicable SCell activation cases for early SRS/CSI/CSI-RS triggering.

Proposal 6: No RRM impacts for CSI-RS density reduction.

References

- RP-251856, New WID: NR MIMO Phase 6