3GPP TSG-RAN WG4 Meeting #116bis	R4-2513731

# Prague, Czech, Oct. 13-17, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for enhanced Low Band CA operation

Agenda item:	7.8.4

Document for:	Discussion

Introduction

In the last RAN#109, new WI for enhancement of low NR band carrier aggregation via switching [1] was approved, and the objective is

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this contribution, we provide our initial views on RRM issues for the enhanced LB CA via switching based on WID

Discussion

Based on objective of the WI, the target scenario is the low band CA via switching for the dual downlink between FDD and SDL

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

The deployment constraints are the same as Rel-19 LB CA assumption in addition to no in-gap interference between DL carrier

  - The carrier frequency for all cases is <1GHz.

  - Co-located and synchronized network deployment for both carriers.

  - Both carriers are in a single TAG.

  - SCS 15Khz on both carriers.

  - No in-gap interference between DL carriers is expected

Also, Rel-19 semi-static switching mechanisms based on RRC configuration is reused.

According to the above scenario, RAN4 can resue most principle considered in Rel-19 LB CA WI phase.

Switching pattern applicability for RRM requirement

For Rel-20 LB CA via switching, the semi-static switching mechanism based on RRC configuration defined in Rel-19 is resued. Therefore, the same switching pattern applicability for RRM requirement could be aaplied for Rel-20 LB CA via switching with dual DL.

  - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133),

    - The switching pattern for SDL SCell is applied.

  - For the SCell which is in activation procedure,

    - The switching pattern for SDL SCell is applied.

  - For the SCell which is deactivated,

    - The switching pattern for SDL SCell is not applied.

- Proposal 1: Switching pattern applicability for RRM requirements for Rel-19 LB CA can be reused

  - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133),

    - The switching pattern for SDL SCell is applied.

  - For the SCell which is in activation procedure,

    - The switching pattern for SDL SCell is applied.

  - For the SCell which is deactivated,

    - The switching pattern for SDL SCell is not applied.

RRM requirement applicability

In Rel-19, RRM requirement applicability has been agreed

  - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC:

    - a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

In Rel-20, durting UE staty SDL carrier, UE can receive signal from both FDD DL and SDL carriers simultaneously. Therefore, the RRM requirement applicability for Rel-20 enhanced LB CA should be applied to only SDL carriers.

  - For gap-less based L3 measurement and L1 measurement of activated SDL SCC:

    - a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Figure 1 LB CA via switching with dual DL between FDD and SDL band

- Proposal 2: RRM requirement applicability for Rel-20 enhanced LB CA via switching

  - For gap-less based L3 measurement and L1 measurement of activated SDL SCC:

    - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Based on the RRM requirements applicability for Rel-20 LB CA,

- Proposal 3: for Rel-20 enhanced LB CA SDL SCC, RAN4 to reuse following LB CA requirements defined in Rel-19

  - SCell activation / deactivation delay requirements

  - Link recovery requirements

  - TCI state switch delay requirements

  - Intra-frequency measurement without measurement gap

  - L1-RSRP measurements for reporting

  - L1-SINR measurements for reporting

Interruption or scheduling restriction requirement

In Rel-20 LB CA, the semi-static switching mechanisms based on RRC configuration is reused. Therefore, switching gap could be reused for enhanced LB CA via switching. Since no interruption and scheduling restriction deu to switching was required in Rel-19 LB CA via switching, it is also necessary to introduce interruption and scheduling restriction in Rel-20.

- Proposal 4: For Rel‑20 LB CA via switching, it is proposed to maintain the same approach in Rel-19, i.e., no interruption or scheduling restriction shall be considered due to switching (between case 1 and case 3).

Interruption requirement for deactivated SDL SCell measurement

RAN4 has defined the missed ACK/NACK rate due to deactivated SDL SCell measurement in Rel-19 below

  - Interruptions on PCell due to the measurement on deactivated SDL SCell are allowed with up to Y % probability of missed ACK/NACK. The value of Y is defined as 2*Length_intmeasCycleSCell ;where, Length_int is equal to  (Tuning time from PCell to SCell  + TSMTC_duation + Retuning time from SCell to PCell)

Since enhanced LB CA via switching consider dual DL between FDD DL and SDL, RAN4 needs to discuss new interruption requirements for Rel-20 LB CA via switching with dual DL. It might depend on RF architecture discussion.

- Proposal 5: RAN4 to discuss new interruption requirement due to deactivated SDL Scell measurement considering dual DL, and it might depend on RF architecture discussion in RF session.

SSB assumptions for Rel-20 enhanced LB CA via switching

In Rel-19 LB CA via switching, RAN4 only considered that SDL SCell is transmitting SSB, i.e., no requirements for SSB-less scenarios. The deploymenet of LB CA via switching is co-located and synchronized network for both FDD and SDL carriers, so SSB-less scenario could be useful. As propoased above, most RRM requirements for Rel-19 LB CA via switching could be resued for Rel-20 enhanced LB CA via switching, RAN4 needs to discuss and introduce SSB-less based requirements.

- Proposal 6: RAN4 to deiscuss and introduce RRM requirements for SSB-less SCell in enhanced LB CA via switching.

Conclusion

In this contribution, we provide our initial views on Rel-20 enhanced low band CA via switching with dual DL, and we propose

- Proposal 1: Switching pattern applicability for RRM requirements for Rel-19 LB CA can be reused

  - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133),

    - The switching pattern for SDL SCell is applied.

  - For the SCell which is in activation procedure,

    - The switching pattern for SDL SCell is applied.

  - For the SCell which is deactivated,

    - The switching pattern for SDL SCell is not applied.

- Proposal 2: RRM requirement applicability for Rel-20 enhanced LB CA via switching

  - For gap-less based L3 measurement and L1 measurement of activated SDL SCC:

    - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed.

    - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

    - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

- Proposal 3: for Rel-20 enhanced LB CA SDL SCC, RAN4 to reuse following LB CA requirements defined in Rel-19

  - SCell activation / deactivation delay requirements

  - Link recovery requirements

  - TCI state switch delay requirements

  - Intra-frequency measurement without measurement gap

  - L1-RSRP measurements for reporting

  - L1-SINR measurements for reporting

- Proposal 4: For Rel‑20 LB CA via switching, it is proposed to maintain the same approach in Rel-19, i.e., no interruption or scheduling restriction shall be considered due to switching (between case 1 and case 3).

- Proposal 5: RAN4 to discuss new interruption requirement due to deactivated SDL Scell measurement considering dual DL, and it might depend on RF architecture discussion in RF session.

- Proposal 6: RAN4 to deiscuss and introduce RRM requirements for SSB-less SCell in enhanced LB CA via switching.

Reference

[1] RP-252953, “New WID on enhancement of low NR band carrier aggregation via switching” Moderator (Huawei)