# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513736

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	4.15.1

Source: 	CMCC

Title: 			Discussion on RRM core requirements for FR2-1 SSB based L3 measurement delay reduction

Document for:	Discussion

- Introduction

In RAN #103 meeting, the WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- Objective of core part - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Objective of performance part - Specify RRM performance requirements and test cases for the Core part enhancements.

There was discussion on RRM core requirements of FR2-1 SSB based L3 measurement delay reduction , and a WF was approved [2]. This contribution provides discussion on the remaining issues for RRM core requirements for FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF outside gap.

- Discussion

Issue 1-2-1: applicability limitation for solution 3 Agreement: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG. FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

Solution 3 is to perform CSSF enhancement based on 3 searchers. For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing. As for the applied scenario for 3 searchers, it was agreed to support FR1 CA, FR1+FR2 scenario where PCell is FR1, and NR-DC. Furthermore, it was agreed that enhanced CSSF apply to all the 3 aspects including intra-frequency measurement, inter-frequency measurement and inter-RAT measurement.

Inter-frequency measurement without gap includes multiple cases:

- SSB is completely contained in the active BWP of the UE

- UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for inter-frequency measurement

- UE indicates ‘no-gap’ via NeedForGapsInfoNR and the UE indicates ‘no-gap-no-interruption’ or no-gap-with-interruption via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement

We do not see the necessity to exclude inter-frequency measurement without gap for solution 3, especially for the case that SSB is completely contained in the active BWP of the UE. On the other hand, enhanced CSSFoutside_gap,i scaling factor not apply for the inter-frequency MOs without MG will limited the applied scenario of this solution.

Proposal 1:  for solution 3, it is not preferred to exclude inter-frequency measurement without gap, i.e. enhanced CSSFoutside_gap,i scaling factor apply for the inter-frequency MOs without MG that are being measured outside of MG.

- Conclusion

This contribution provides discussion on the remaining issues for RRM core requirements for FR2-1 SSB based L3 measurement delay reduction. The proposals are:

Proposal 1:  for solution 3, it is not preferred to exclude inter-frequency measurement without gap, i.e. enhanced CSSFoutside_gap,i scaling factor apply for the inter-frequency MOs without MG that are being measured outside of MG.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2512124, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements