# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513775

# Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Huawei, HiSilicon

Title: 	OTA OOB blocking test case reduction

Agenda Item:	6.8.4.2.1

Document for:	Approval

1 Introduction

New WID on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at RAN#103 meeting. The work item concentrates on 2 work areas where the current BS specification may be optimised, i.e. Expected EIRP mask for upper 6GHz and OTA test enhancement.

The considerations on test case reduction for TX IM test reduction was discussed in previous meetings and the latest WF is captured in [7]. In this contribution we provide further discussion and proposals for open issue.

2 Out of band blocking

For RX test case reduction, it is agreed to focus on out of band blocking and focus on the measurement step size for Rx OOBB requirement. Two option are for further decision.

Issue 1-2-1: Out of band blocking

- Agreement on step size of OOBB requirement, keep the following options for further discussion:

    - Option 1: 1/3 of the minimum supported bandwidth

      - Option 1a:

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

- Option 2: use 2/3 of the test signal configuration channel bandwidths

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 3 |
|  | 10 | 6 |
|  | 20 | 13 |

RAN4 had similar discussion for FR2 in Rel-15 and finally agreed to reduce the amount of time by increasing the interferer step size. The less than 1/3 BW approach is adopted when define the measurement step for FR2-1.

Table 7.6.4.2.3-1: Interferer signal step size for FR2-1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

For FR1 similar approach can be considered, e.g. option 1. We propose to set the measurement step size according to the supported BS channel bandwidth declared by the manufacture.

Table 3-1: Interferer signal step size for FR1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | ≤5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. The existing FR2 approach considers that the harmonic number m of the interferer signal equals 4 or more are usually not significant hence 1/3 BW approach is adopted. If further test time reduction is considered, we think at least low order IMD interference should be considered, i.e. IM2 or IM3. In such case n is less or equal to 2. Hence 1/2 BW approach can be adopted if we agree only low order IMD is considered for out of band blocking.

Proposal: It is proposed to further discuss and decide on the following 2 options.

    - Option 1: 1/3 of the minimum supported channel bandwidth

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

    - Option 2: 1/2 of the minimum supported channel bandwidth

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 2 |
|  | 10 | 6 |
|  | 20 | 12 |

4	Summary

Proposal: It is proposed to further discuss and decide on the following 2 options.

    - Option 1: 1/3 of the minimum supported channel bandwidth

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

    - Option 2: 1/2 of the minimum supported channel bandwidth

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 2 |
|  | 10 | 6 |
|  | 20 | 12 |

3	Reference

[1] R4-2419243, Test case reduction for TX IM, Huawei

[2] R4-2419828, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation

[3] R4-2501854, OTA test case reduction, Huawei

[4] R4-2502375, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation

[5] R4-2504740, Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP

[6] R4-2508658, Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP

[7] R4-2512580, Way Forward for [116][304] NR_BS_RF_Part3_OTA_TRP