3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513788

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	On BS RF requirements for SBFD-capable BS

Agenda item:			4.19.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In RAN4#116, CR was agreed to TS38.104 to introduce BS RF requirement for SBFD-capable BS, which has been approved in the CR package [RP‑252418] in RAN#109. In this contribution, we would like to provide viewpoints to solve the remaining issues of SBFD-capable BS RF requirement for maintenance purpose.

# 2. Discussion

## 2.1 In-channel adjacent subband leakage

Based on RAN4#116 discussion, the following agreement is achieved, which is the basis to define the requirement of in-channel adjacent subband leakage:

##### Issue 4-2-2: In-channel adjacent subband leakage for FR1 WA/MR BS, for DU/UD case Agreement: - Absolute limit for in-channel adjacent subband leakage for DU/UD case - Reuse legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1) for bands: Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit BS category / BS class; ACLR absolute basic limit. Category A Wide Area BS; -13 dBm/MHz. Category B Wide Area BS; -15 dBm/MHz. Medium Range BS; -25 dBm/MHz. Local Area BS; -32 dBm/MHz. - The necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104 shall be studied in maintenance of Rel-19 SBFD WI

Based on the above agreement, the necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104 shall be studied in maintenance of Rel-19 SBFD WI.  For band n104 ACLR requirement, the relative ACLR requirement is 7dB relaxed compared to normal bands, and gNB vendors can fulfill the existing ACLR requirement by applying a relaxed relative ACLR requirement rather the absolute requirement. Taking FR1 SBFD DU case as example in which 80MHz (DL) – 20MHz (UL), 50dBm as BS TX power and 38dBc as relative in-channel adjacent subband leakage (which is already 7dB tightened than original n104 relative ACLR requirement), the power density is around -7.03dBm, which is around 6dB higher than the ICASL absolute basic limit. By considering advanced DPD techniques can be applied, which can help to further mitigate the ICASL by 3dB, the existing requirement is still very demanding. Therefore, RAN4 shall confirm the necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104, and corresponding n104 dedicated requirement can be applied.

Proposal 1: RAN4 shall introduce the additional 3dB relaxation of ACLR absolute basic limit for band n104.

Accordingly, the following text proposal can be adopted.

Proposal 2: RAN4 shall adopt the following text proposal for SBFD Base station ICASL absolute basic limit for band n104.

------------------------------------------- Text Proposal Start -----------------------------------------------

### 12.2.9	In-channel adjacent subband leakage for SBFD

#### 12.2.9.1	General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of SBFD UL subband within NR carrier.

The requirement shall apply during the transmitter ON period.

#### 12.2.9.2	Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit for bands other than band n104 | ICASL absolute basic limit for band n104 |
| --- | --- | --- |
| Category A Wide Area BS | -13 dBm/MHz | -10 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz | -12 dBm/MHz |
| Medium Range BS | -25 dBm/MHz | -22 dBm/MHz |
| Local Area BS | -32 dBm/MHz | -29 dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

------------------------------------------- Text Proposal End -----------------------------------------------

# 3. Conclusion

In this contribution, we have provided we would like to provide viewpoints to solve the remaining issues on BS RF requirements for maintenance purpose:

Proposal 1: RAN4 shall introduce the additional 3dB relaxation of ACLR absolute basic limit for band n104.

Accordingly, the following text proposal can be adopted.

Proposal 2: RAN4 shall adopt the following text proposal for SBFD Base station ICASL absolute basic limit for band n104.

------------------------------------------- Text Proposal Start -----------------------------------------------

### 12.2.9	In-channel adjacent subband leakage for SBFD

#### 12.2.9.1	General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of SBFD UL subband within NR carrier.

The requirement shall apply during the transmitter ON period.

#### 12.2.9.2	Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit for bands other than band n104 | ICASL absolute basic limit for band n104 |
| --- | --- | --- |
| Category A Wide Area BS | -13 dBm/MHz | -10 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz | -12 dBm/MHz |
| Medium Range BS | -25 dBm/MHz | -22 dBm/MHz |
| Local Area BS | -32 dBm/MHz | -29 dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

------------------------------------------- Text Proposal End -----------------------------------------------

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2512665, “Way Forward for [116][306] NR_duplex_evo_General”, Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE, RAN#116.

[4] R4-2512680, “Way Forward for [116][307] NR_duplex_evo_BSRF”, Huawei, HiSilicon, Samsung, RAN#116.

[5] R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung, RAN#116.