3GPP TSG-RAN WG4 Meeting #116bis                                  R4-2513791

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	On SBFD BS to SBFD BS Adjacent Channel Coexistence

Agenda item:			7.2.4

Document for:	Approval

# Introduction

In Rel-18, Sub-Band non-overlapping Full Duplex (SBFD) was adopted as a new study item (SI) to enable the simultaneous transmission and reception capability of NR base stations (BSs) and to enhance the capacity of NR system. Since then, comprehensive studies have been conducted on the SBFD topics and conclusions have been made and documented in technical report [1].

To explore the potential deployment of SBFD BSs, the adjacent channel co-existence evaluation has been proposed in Rel-19, and the TDD-SBFD co-existence problem have been investigated. However, due to the work load and time limitation, RAN4 agreed not to introduce specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19 [2].

- Issue 1-1: Whether SBFD-SBFD co-ex case should be in Rel-19 scope - Way forward - RAN4 do not introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19. SBFD-SBFD co-ex case can be considered for Rel-20.

In RAN#108, it is agreed to continue investigating the SBFD BS to SBFD BS adjacent channel coexistence, but focusing on the more realistic deployment scenarios.

6 BS RF centric evolution 6.1 Agreements: The high-level scope for WI on BS RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - Enhancement of co-location requirements - It is FFS on the applicable frequency ranges, e.g. >3GHz requirements, for OTA - R19 outcome should be considered - Coexistence requirement enhancement can be further discussed in RAN#109 - Down select from the following four options in RAN#109 - Additional Hybrid requirement sets for BS type 1-H/large antenna array - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - It is limited to RAN4 only solution - Neither - Both - No RAN1/2 impact is expected - Target 0.5RF TU in Q4’25 and 1 TU allocation from Q1’26.

In RAN#109, the new work item (WI) entitled “NR base station (BS) RF requirement evolution for FR1 and testing phase 2” has been approved to further investigate the co-location requirements enhancement and SBFD BS to SBFD BS adjacent co-existence problems in Rel-20 [3].

SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

In this contribution, we discuss the SBFD BS to SBFD BS (SBFD-SBFD) adjacent channel co-existence problem and corresponding evaluation requirements.

# Discussion

## 2.1 Background Information of SBFD-SBFD

The TDD-SBFD adjacent channel co-existence scenario have been extensive investigated in Rel-19. It is reported that the co-existence of SBFD BS and conventional TDD BS is only feasible for the networks with high grid-shift (e.g., 100% grid-shift) in Urban Macro to Urban Macro scenarios. The SBFD UL performance would see a significant downgrade from both cell edge throughput and average throughput perspectives, because the limited ACIR enhancement of TDD BS can not support higher co-existence performance [1].

Table 1: Case 3 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 and Scenario 6 | FR1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput, and minor degradation but acceptable to some companies for average throughput. With other assumptions (higher BS Tx power and lower grid-shifts), the degradation is increased for cell edge throughput and average throughput. |
|  | FR2-1 | Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power and lower grid-shifts. |

However, the high grid-shift assumption is too ideal to be true for the practical Urban macro BS deployment environment. Therefore, it is necessary to study other feasible co-existence scenarios in support of future SBFD deployment.

Different from the SBFD-TDD co-existence scenario, the adjacent channel co-existence condition between an SBFD BS and another SBFD BS is less stringent due to the flexibility of frequency band allocation of SBFD. In such adjacent channel co-existence scenario, the better adjacent channel interference ratio (ACIR) can be achieved by carefully organize the DL and UL sub-band channel for different SBFD BS and imposing additional RF requirements for both transmitter and receiver sides. As shown in Fig. 1, the DL sub-band of the adjacent SBFD operator can be used as guard band to its corresponding UL channel by adopting an additional sub-band Tx filter. In this way, the Tx unwanted emission can be significantly controlled and the in-band blocking receiver (for SBFD UL sub-band) performance can also be improved, which can help to enable SBFD-SBFD adjacent channel deployment with less deployment limitation. In addition, it is also worth noting that such adjacent channel co-existence scheme will not bring about much implementation difficulty since the guard band is sufficiently large [6].

Fig. 1 An example for SBFD-SBFD co-existence scenario

In the following part, the SBFD-SBFD adjacent channel co-existence simulation configurations will be discussed.

## 2.2 SBFD-SBFD Co-existence Simulation Planning

### 2.2.1 Adjacent Channel Co-existence Scenarios

Based on the scenarios defined in Table 11.1-1 in TR38.858 [1], Scenario 1 and 6 are related to the wide area coverage co-existence and Scenarios 1-5 are related to the co-existence simulation with FR1. According to the agreement in RAN#109 [3], the SBFD-SBFD co-existence study will focus on FR1 WA. Thus, only Scenario 1 should be adopted for the SBFD-SBFD co-existence simulation.

Table 11.1-1: Adjacent channel co-existence scenarios [1]

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation.

### 2.2.2 Adjacent Channel Co-existence Cases

According to TR38.858, there are four co-existence cases defined in the technical report, which are SBFD-TDD DL, SBFD-TDD UL, TDD DL-SBFD, TDD UL-SBFD [1]. However, these co-existence cases are not suitable for the SBFD-SBFD co-existence simulation study. It is recommended that RAN4 introduce new co-existence case definition for SBFD-SBFD co-existence study.

RAN4 should define new co-existence cases for SBFD-SBFD adjacent channel co-existence simulation.

Note that in TR38.858, the SBFD configuration case {D, U, D} has not been taken into consideration, because its performance is comparable to the selection SBFD configuration case {D, U} based on the RAN4 model and parameters related to leakage. However, for the SBFD-SBFD co-existence study, the different sub-band configurations, e.g. {U, D},{D, U} and {D, U, D}, could lead to different sub-band filter performance, and thus result in different co-existence behaviors. In this case, the different SBFD configurations have to be taken account in the SBFD-SBFD co-existence studies.

RAN4 should consider SBFD configurations {U, D} and {D, U, D} in the SBFD-SBFD co-existence cases.

Based on the two SBFD configuration types, we propose to define three additional adjacent channel co-existence cases for the SBFD-SBFD co-existence study, which are 1) SBFD UD – SBFD DU case, 2) SBFD UD – SBFD DUD case, 3) SBFD DUD – SBFD DUD case. The detailed information is provided in the updated adjacent channel co-existence case table below.

Table 11.1-2: (Updated) Adjacent channel co-existence cases

| Case | Aggressor | Aggressor baseline | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL | TDD DL |  |
| 2 | SBFD | TDD UL | TDD UL |  |
| 3 | TDD DL | No aggressor network | SBFD |  |
| 4 | TDD UL | No aggressor network | SBFD |  |
| 5 | SBFD UD | No aggressor network | SBFD DU |  |
| 6 | SBFD UD | No aggressor network | SBFD DUD |  |
| 7 | SBFD DUD | No aggressor network | SBFD DUD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |  |

From the perspective of size of the SBFD guard band, the SBFD UD – SBFD DU case possesses largest guard band size, which naturally has the largest flexibility and also the largest redundancy of sub-band filter design. On the contrary, the SBFD DUD – SBFD DUD case has the smallest guard band size, which would add difficulties on the sub-band filter design and may affect the Tx unwanted emission suppression performance. Concerning the work load and time limitation, the SBFD-SBFD co-existence cases can be down-selected. It is recommended RAN4 first consider Case 5 (SBFD UD – SBFD DU), i.e., the most generally adopted case, and the Case 7 (SBFD DUD – SBFD DUD), i.e., the worst performance case in the SBFD-SBFD co-existence simulation.

RAN4 could adopt Case 5 (SBFD UD – SBFD DU) and Case 7 (SBFD DUD – SBFD DUD) in SBFD-SBFD adjacent channel co-existence simulation for simplification.

### 2.2.3 Deployment Parameters, Traffic Parameters, BS & RF characteristics

In TR38.858, the deployment parameters, traffic parameters, as well as BS & RF characteristics are defined in Annex E [7]. For the SBFD-SBFD co-existence simulation, the detailed simulation parameters can be discussed based on companies’ observations. However, in general, it is recommended that RAN4 follows simulation parameters in TR38.858 Annex E at the beginning stage of the SBFD-SBFD co-existence simulation.

RAN4 could follow simulation parameters in TR38.858 Annex E at the beginning stage of the SBFD-SBFD co-existence simulation.

### 2.2.4 Co-location and Non-co-location Deployment Requirement

In CR [8], it is noted that the in-band blocking requirements are derived from the non-co-location assumption for SBFD-TDD co-existence study.

12.3.4.2 In-band blocking for SBFD 12.3.4.2.1 General The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for BS type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking. NOTE 1: For FR1 WA BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements.

For the SBFD-SBFD co-existence study, since SBFD BS can have better Tx unwanted emission control, the in-band blocking requirement assumption can be less ideal, and both co-location and non-co-location BS deployment assumptions can be adopted.

RAN4 to adopt both co-location and non-co-location BS deployment assumption for the SBFD-SBFD co-existence study.

## 2.3 Discussion on Future SBFD-SBFD Evolution

For the SBFD co-existence study in NR, the carrier frequency of the SBFD BS is assumed to be 4GHz under FR1 deployment scenarios [7], which is documented in Table E.2.1-1 TR 38.858 Annex E. The following SBFD-TDD and SBFD-SBFD co-existence studies have been conducted under this carrier frequency requirement. However, concerning the future evolutions of full-duplex techniques as well as radio-frequency technologies, other potential carrier frequencies should be taken into consideration. Based on the outcomes of IMT parameter studies in TR 38.922 [9], it is recommended that RAN4 considers 7GHz as an example band in the future co-existence study.

RAN4 to consider 7GHz as one of example bands in the future co-existence study.

At the same time, the array antenna model and the corresponding antenna parameters for 7125 - 8400 MHz frequency range have been provided in TR 38.922 [9]. For the co-existence study for 7GHz in RAN4, it is reasonable to consider the BS antenna assumption for 7GHz in TR 38.922 as starting point.

RAN4 to consider the BS antenna assumption for 7GHz in TR 38.922 as starting point.

# Conclusion

- Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation.

RAN4 should define new co-existence cases for SBFD-SBFD adjacent channel co-existence simulation.

RAN4 should consider SBFD configurations {U, D} and {D, U, D} in the SBFD-SBFD co-existence cases.

RAN4 could adopt Case 5 (SBFD UD – SBFD DU) and Case 7 (SBFD DUD – SBFD DUD) in SBFD-SBFD adjacent channel co-existence simulation for simplification.

RAN4 could follow simulation parameters in TR38.858 Annex E at the beginning stage of the SBFD-SBFD co-existence simulation.

RAN4 to adopt both co-location and non-co-location BS deployment assumption for the SBFD-SBFD co-existence study.

RAN4 to consider 7GHz as one of example bands in the future co-existence study.

RAN4 to consider the BS antenna assumption for 7GHz in TR 38.922 as starting point.

# References

TR 38.858, “Study on Evolution of NR Duplex Operation”

R4-2508713, “Way Forward for [115][307] NR_duplex_evo_BSRF”, Huawei, HiSilicon, 3GPP TSG RAN4#115, Malta, MT, May 19-23, 2025.

RP-251879, “WF for RAN4 led REL-20 topics”, RAN4 Chair (Apple), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, Jun. 9-13, 2025

RP-252905 “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, Moderator (Huawei), 3GPP TSG RAN Meeting #109, Beijing, China, Sep. 15-18, 2025

RP-252067, “Views on 5G A Rel 20 BS RF topics”, Samsung, 3GPP TSG RAN Meeting #109, Beijing, China, Sep. 15-18, 2025

RP-252739, “Proposals for RAN4-led Rel-20 BS RF topics”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #109, Beijing, China, Sep. 15-18, 2025

TR 38.858, “Study on Evolution of NR Duplex Operation: Annex E”

R4-2511255, “CR to TS38.104 to Introduce BS RF Requirement for SBFD-Capable BS”, Samsung, 3GPP TSG RAN4#116, Bengaluru, India, Aug. 25-29, 2025

Tr 38.922 “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz;”