3GPP TSG-RAN WG4 Meeting #116bis		R4-2513801

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:		6.6.3.2

Source:		Samsung

Title:	Discussion on RRM impacts by using NGSO in the test

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#106 meeting. The objectives of performance part of NTN testing for NGSO are described in [1] as:

The objectives of performance part for enhanced requirements and test methodology for NR and IoT NTN include: NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In last RAN4#116 meeting, there are open issues for NTN tests by using NGSO in performance part. In this contribution, we provide our view of the RRM performance part of NTN testing for NGSO.

- Discussion

In last RAN4#116 meeting and RAN#109 meeting, there was a discussion on the applicability of NTN testing by using NGSO. There are two options:

One option is that the NTN testing by using NGSO channel, it is applied for NTN uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands according to WID.

The other option is that the NTN testing by using NGSO channel, it is applied for all NTN test cases.

By our understanding, the motivation of the WID is to:

  - Introduce new NGSO channel mode.

  - UE timing tests are key test cases to verity the UE performance for varying doppler and delay shifts.

  - If UE can pass such timing test, which means UE has no issues to handle NGSO and varying doppler and delay shifts.

  - If extend the evaluation of all RRM tests, whether to add margin, what is the value of margin, there are workload.

To avoid such workload, that is why the current WID only mentioned for RRM uplink timing test cases. In demod discussion, there are similar situation. They picked up some of demod requirements but not all of them to discuss whether to add margin due to timing/frequency drift.

In last RAN4#116 meeting, some companies think in RAN4, we only change the channel from AWGN to NGSO. The evaluation is left to RAN5. This is not the same understanding from our side. We think typical RAN4/RAN5 working procedure is that RAN4 specified the test configuration and test requirements. RAN5 will only add MU and TT on top of RAN4 test cases. If 3GPP will apply the NGSO channel to all NTN tests, RAN4 shall evaluate whether current test requirements are still valid.

Proposal 1: The applicability of NGSO channel for NTN tests is for UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands.

In the existing test cases, for NR in 38.133 Annex, there are two series of TCs for NR-NTN:

#### A.14.3.1.1 NR UE Transmit Timing Test for FR1 ### A.14.3.2 Timing advance for satellite access #### A.14.3.2.1 SA FR1 timing advance adjustment accuracy

For the first test case, it is to test the UE transmit timing.

For the second test case, it is to test the timing advance adjustment.

In both tests, AWGN channel was used in the tests. For the first step, the NGSO channel model should be replaced from Rel-19.

| Propagation condition |  | 1,2 | AWGN |
| --- | --- | --- | --- |

| Propagation condition | - | AWGN |
| --- | --- | --- |

Proposal 2: First, NGSO channel model should be added in these TCs in the table. The corresponding configuration of NGSO channel should be added.

- During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

For the Transmit timing test, the test equipment verified the timing based on UE estimated value.

For the second timing advance adjustment, in the test, it verified that the UE adjust the timing of its transmissions with the relative accuracy shall be better than or equal to the UE timing advance adjustment accuracy. In this relative accuracy, there is no timing drifting from T0 to T1. The relative accuracy is at the point when UE adjusts the timing of its transmission.

Proposal 3: For NR-NTN

- For transmit timing test, no impacts for the test procedure and test requirements. The NTA,adjUE in the test requirements is calculated by UE itself.

- For the timing advance test, no impacts for the test requirements.

For Iot-NTN tests in 36.133, five TCs are specified:

## A.13.4 Timing and signalling characteristics for satellite access ### A.13.4.1 UE transmit timing for satellite access A.13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access #### A.13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access #### A.13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access ### A.13.4.2 UE timing advance for satellite access A.13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access A.13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

Proposal 4: For five TCs for IoT-NTN, use the same principle as NR-NTN.

- Conclusion

In this contribution, we analyze the potential impact on RRM performance requirements for NTN testing for NGSO and provide our proposals:

Proposal 1: The applicability of NGSO channel for NTN tests is for UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands.

Proposal 2: First, NGSO channel model should be added in these TCs in the table. The corresponding configuration of NGSO channel should be added.

- During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

Proposal 3: For NR-NTN

- For transmit timing test, no impacts for the test procedure and test requirements. The NTA,adjUE in the test requirements is calculated by UE itself.

- For the timing advance test, no impacts for the test requirements.

Proposal 4: For five TCs for IoT-NTN, use the same principle as NR-NTN.

- Reference

[1] RP-242901, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

[2] R4-2512342, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi