3GPP TSG-RAN WG4 Meeting #116bis		     R4-2513808

Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item:	7.9.3

Source: 	OPPO

Title: 	Inter-vendor training collaboration and interoperability for two-sided AIML models

Document for:	Approval

Introduction

In R20 WID, following objectives are approved for the inter-vendor training collaboration for two-sided AI/ML models

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, issues regarding the development of RAN4 standardized models and standardized datasets are analysed. Additionally, performance evaluations under different model approaches are conducted based on the backbone provided by RAN1 in R19.

Discussion

As guided in the end of last RAN4 meeting, at least following issues should be clarified first, that may be helpful to get an aligned understanding among companies.

Assumptions on the dataset

In R19, a mixed dataset based on the UMa channel assumption was developed using inputs from five companies. Based on this mixed dataset, RAN4 evaluated the performance under different model assumptions, the compatibility between various encoders and a fixed decoder, and the impact of different encoder complexities on the performance of CSI compression use cases.

In R20, to fully specify a reference model in RAN4, this mixed dataset could be reused initially. On one hand, no significant issues have been identified with the dataset after its use in R19. On the other hand, constructing a new standardized dataset would require more discussion, evaluation, and development. Therefore, for the fully defined/specified reference model ("Direction C"), in the absence of a clearly defined new standardized dataset, the existing mixed dataset could serve as a baseline for simulation and evaluation by companies, e.g., assessing the performance of candidate standardized models like transformer backbones and ensuring performance alignment.

The existing mixed dataset could serve as a baseline for simulation and evaluation by companies, e.g., assessing the performance of candidate standardized models like transformer backbones and ensuring performance alignment.

For R20, compared to R19, scalability needs to be considered as an additional factor in the standardization of models. This change will introduce new requirements for datasets. In R19, the mixed datasets used had fixed interface parameters, such as the number of sub-bands = 13 and the number of antenna ports = 32, and the output size of the CSI encoder was fixed at 32 floating-point numbers or 64 bits. For 6G, to address scalability considerations, a new dataset with different assumptions on the number of sub-bands, the number of antenna ports, and the output size of the CSI encoder should be considered.

For 6G, to address scalability considerations, a new dataset with different assumptions on the number of sub-bands, the number of antenna ports, and the output size of the CSI encoder should be considered.

Furthermore, during the R19, RAN4 created a mixed dataset comprising 500,000 samples. For 6G, more companies are encouraged to provide their respective datasets to help create a mixed dataset with broader sources. However, it is important to note that the datasets contributed by each company should be calibrated, e.g., using the RAN4 R19 CNN based CSI encoder/decoder to validate the performance on their respective datasets, to ensure the overall quality of the RAN4 standardized dataset.

The datasets contributed by each company should be calibrated, e.g., using the RAN4 R19 CNN based CSI encoder/decoder to validate the performance on their respective datasets, to ensure the overall quality of the RAN4 standardized dataset.

Assumptions on the test decoder and reference encoder structures

In R19, a CNN-based decoder was proposed and agreed to verify whether model performance alignment could be achieved, as shown in Appendix 1. Additionally, a CNN-based CSI encoder, along with simplified transformer-based, simplified CNN-based, and MLP-based encoders, were utilized in conjunction with the proposed CNN-based decoder, as shown in Appendix 2.

In R20, further research on model standardization is required, with the following aspects needing consideration

- UE may differ in their AI capabilities, including supported models, architectures, and computational complexities. When designing models and defining minimum performance requirements, RAN4 should consider potential limitations arising from these differing UE capabilities.

    - Option1: Standardized models with varying complexities need to be introduced,

    - Option2: Ensuring that constraints on UE deployment(e.g., available computing resource, memory size, power consumption) are fully addressed during standardized model design.

- RAN1 provided one example structure for study purposes, and RAN4 evaluations can begin with this structure. However, the structure given by RAN1 lacks detailed parameter specifications. For RAN4 evaluations, parameter designs of varying complexities can be assessed in Q4.

In Section 3, we give 2 different description based on the transformer backbone and also provide performance analyses of the two models with different complexity levels.

- Besides the transformer backbone, the CNN-based CSI decoder model adopted in the R19 phase, along with the accompanying simplified transformer-based, simplified CNN-based, and MLP-based CSI encoders, could also serve as candidate test decoders and reference encoder structures for evaluation. The key difference from the R19 is the need to incorporate scalability evaluation.

UE may differ in their AI capabilities, including supported models, architectures, and computational complexities.

RAN4 should consider potential limitations arising from these differing UE capabilities.

    - Option1: More than one standardized models with varying complexities need to be introduced,

    - Option2: Ensuring that constraints on UE deployment(e.g., available computing resource, memory size, power consumption) are fully addressed during standardized model design.

Based on the transformer backbone that provided by RAN1, RAN4 evaluations can begin with this structure, detailed designs of varying complexities can be assessed in Q4.

The CNN-based CSI decoder model adopted in the R19, along with the accompanying simplified transformer-based, simplified CNN-based, and MLP-based CSI encoders, could also serve as candidate test decoders and reference encoder structures for evaluation

Assumptions on the scalability

Regarding the scalability issue, it is necessary to incorporate corresponding evaluations during the model design. However, we would like to point out that under the current CSI feedback requirements, scalability challenges exist in multiple dimensions, including 1) the number of sub-bands, 2) the number of antenna ports, and 3) CSI payload sizes.

For instance, in terms of the number of sub-bands, beyond the currently adopted 13 sub-bands, various other sub-band configurations exist, which are directly related to the frequency domain bandwidth, e.g., 5MHz, 10MHz, 20MHz. Similarly, for the number of antenna ports, configurations such as 4, 8, 16, and 32 ports all need to be considered. As for CSI payload sizes, the protocol already supports a range of bit sizes from tens to hundreds of bits. Designing a single model to handle all these scalability aspects would pose significant challenges to both model design and performance.

Scalability challenges exist in multiple dimensions, including

    - Number of sub-bands (e.g., corresponding to 5MHz, 10 MHz, 20 MHz, with 4 RBs per sub-band)

    - Number of antenna ports (e.g., 16 and 32)

    - CSI payload sizes (e.g., 64 bits, 128 bits)

In terms of specific solutions for designing model architectures to address scalability, the following approaches should be considered:

- Pre/post-processing with truncation/padding: For example, when the input to the CSI compression model has 32 antenna ports, no additional processing is needed. However, when the input has only 16 antenna ports, 16 zeros may be padded to match the model's expected input dimensions. Similarly, for the output of the CSI recovery model, when 32 antenna ports are required, no adjustment is needed, but for 16 antenna ports, a subset may be truncated to produce the corresponding output.

- Adaptation layers in the AI/ML model: Introduce an Adaptation layers in the AI/ML model, to accommodate different input dimensions of the CSI encoder, CSI payload size (output dimensions of the CSI encoder, input dimensions of the CSI decoder), and output dimensions of the CSI decoder.

- Concatenating multiple models: If higher input or output dimensions are required in practical applications, perform inference by concatenating multiple models. For example, for a 10 MHz bandwidth, use two 5 MHz models separately.

- Above approaches can be combined.

- Additionally, when considering scalability issues, it is also necessary to evaluate whether to adopt a "single model structure + single set of model parameters" approach or a "single model structure + multiple model parameters" approach to address scalability challenges.

In terms of specific solutions for designing model architectures to address scalability, the following approaches should be considered

    - Pre/post-processing with truncation/padding

    - Adaptation layers in the AI/ML model

    - Concatenating multiple models

RAN4 to evaluate whether to adopt "single model structure + single set of model parameters" or “single model structure + multiple model parameters" to address scalability challenges

Whether the decoder/encoder for RAN4 are the same as for direction C

For direction C, RAN1 may aim to define a reference/ model that meets commercial deployment requirements (minimum performance requirements for deployment), which imposes relatively high demands on model performance.

In contrast, R20 RAN4 still focuses on specifying a test model that fulfills testing requirements (minimum performance requirements for testing). Thus, the key question is: which of the following standardization approaches should RAN4 adopt.

- Option1: RAN4 develops a standardized model based on RAN4’s testing requirements, addressing the scalability issue in R20.

- Option2: RAN4 develops a standardized model that fulfils both RAN4’s testing requirements and RAN1’s commercial deployment requirements.

- Option3: RAN4 develops two separate standardized models, one to meet RAN4’s testing needs and another one to fulfill RAN1’s commercial deployment requirements.

For option2 and option3 mentioned above, RAN1’s requirements should be clarified and let RAN4 know.

In our view, in R20, we should focus on RAN4 testing first and develop the standardized models under assumptions that account for scalability issues. Throughout this process, additional RAN1 input(if any) may serve as a factor influencing the design and evaluation of standardization.

In R20, RAN4 should focus on the testing issue and develop the standardized models under assumptions that account for scalability.

How to capture model structures and model parameters in 3GPP

Regarding the model structure, it could be captured in 3GPP through a description of the architecture. However, directly capturing the model parameters in 3GPP, such as explicitly listing each parameter value, would be a challenging task. For model parameters, an alternative approach could be the parameters are stored on 3GPP server, while the storage location of these model parameters would be referenced in a relevant 3GPP TS or TR documents, thereby serving as a means to capture the model parameters.

Regarding the model structure, it could be captured in 3GPP by a description of the architecture.

Regarding the model parameter(or model with parameter), it could be stored on 3GPP server, while the storage location of these model parameters would be referenced in a relevant 3GPP TS or TR documents.

Test metric and Channel model for the requirements

For CSI prediction, Channel model for CSI prediction tests has been agreed in RAN4 #113

Agreement (RAN4 #113): For channel model for CSI prediction tests, TDL channel models will be used.

For CSI prediction, test metric has been agreed in RAN4#114bis

Agreement (RAN4 #113): Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

Regarding the test metric and channel model used for CSI compression, the same assumptions as those for the CSI prediction could be reused. This is because both CSI compression and CSI prediction essentially evaluate the performance of PMI reporting in testing, only difference is through different methodologies.

RAN4 could further assess whether the performance requirement for AI based CSI compression differ from those of traditional PMI reporting. However, for the channel model and the test metric themselves, there is no need to introduce new assumptions.

Regarding the test metric and channel model used for CSI compression, the same assumptions as those for the CSI prediction could be reused.

Simulation results for R20 AI/ML CSI compression

Model structure

In RAN1 R19, for studying the standardized model structure, following model structure is proposed as one example structure for study purpose.

Figure 1  CSI encoder and CSI decoder that proposed in RAN1 R19

Regarding hyperparameters for the CSI encoder and decoder, the following need to be specified:

| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice |
| --- | --- |
| The decoder has a mirroring design as the encoder |  |

Based on the above description, we propose two approaches as outlined below:

- Case A: Utilizing the hyperparameters defined for the lower-complexity Encoder2 in RAN4 R19 (with consistent hyperparameters maintained between the encoder and decoder, shown below)

| Model type | Transformer |
| --- | --- |
| d_model | 64 |
| d_feedforward | 256 |
| N_head | 8 |
| d_head | 64 |
| N_TF | 4 |

For case A, the FLOPs, number of trainable parameters and model size are summarized as below

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 3.6M | 3.7M | 0.16M | 0.17M | 1.6MB | 1.6MB |

- Case B: The model complexity has been increased to achieve better performance (with consistent hyperparameters maintained between the encoder and decoder, shown below).

| Model type | Transformer |
| --- | --- |
| d_model | 256 |
| d_feedforward | 1024 |
| N_head | 4 |
| d_head | 256 |
| N_TF | 6 |

For case B, the FLOPs, number of trainable parameters and model size are summarized as below

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 248.1M | 248.4M | 9.7M | 9.6M | 42MB | 42MB |

Simulation results

For both Case A and Case B, we train/test the encoder and decoder using the RAN4 R19 mixed dataset. Simulations are shown below

| Model | SGCS |
| --- | --- |
| Case A | 0.697 |
| Case B | 0.744 |

It can be observed that the introduction of the more complex transformer model yields higher SGCS gain. However, it is also important to note that in Case B, the model complexity reaches 248 MFLOPs on the UE side, with a model size of 42MB, which poses significant challenges for UE deployment. In contrast, the model corresponding to Case A achieves lower SGCS performance but has a complexity of only 3.6 MFLOPs and a model size of 1.6MB.

Therefore, in Release 20, RAN4 needs to clarify: the expected performance level, along with the acceptable model complexity, parameter number, and model size. RAN4 needs to establish a set of constraints, otherwise it will be difficult to conduct effective standardization discussions or perform meaningful model performance comparisons and selections.

In R20, RAN4 needs to clarify the expected performance level, along with the acceptable model complexity, parameter number, and model size.

Conclusion

In this contribution, issues regarding the development of RAN4 standardized models and standardized datasets are analysed and got the following proposals:

Proposal 1: 	The existing mixed dataset could serve as a baseline for simulation and evaluation by companies, e.g., assessing the performance of candidate standardized models like transformer backbones and ensuring performance alignment.

Proposal 2: 	For 6G, to address scalability considerations, a new dataset with different assumptions on the number of sub-bands, the number of antenna ports, and the output size of the CSI encoder should be considered.

Proposal 3: 	The datasets contributed by each company should be calibrated, e.g., using the RAN4 R19 CNN based CSI encoder/decoder to validate the performance on their respective datasets, to ensure the overall quality of the RAN4 standardized dataset.

Observation 1: 	UE may differ in their AI capabilities, including supported models, architectures, and computational complexities.

Proposal 4: 	RAN4 should consider potential limitations arising from these differing UE capabilities.

    - Option1: More than one standardized models with varying complexities need to be introduced,

    - Option2: Ensuring that constraints on UE deployment(e.g., available computing resource, memory size, power consumption) are fully addressed during standardized model design.

Proposal 5: 	Based on the transformer backbone that provided by RAN1, RAN4 evaluations can begin with this structure, detailed designs of varying complexities can be assessed in Q4.

Proposal 6: 	The CNN-based CSI decoder model adopted in the R19, along with the accompanying simplified transformer-based, simplified CNN-based, and MLP-based CSI encoders, could also serve as candidate test decoders and reference encoder structures for evaluation

Proposal 7: 	Scalability challenges exist in multiple dimensions, including

    - Number of sub-bands (e.g., corresponding to 5MHz, 10 MHz, 20 MHz, with 4RB per sub-band)

    - Number of antenna ports (e.g., 16 and 32)

    - CSI payload sizes (e.g., 64 bits, 128 bits)

Proposal 8: 	In terms of specific solutions for designing model architectures to address scalability, the following approaches should be considered

    - Pre/post-processing with truncation/padding

    - Adaptation layers in the AI/ML model

    - Concatenating multiple models

Proposal 9: 	RAN4 to evaluate whether to adopt "single model structure + single set of model parameters" or “single model structure + multiple model parameters" to address scalability challenges

Proposal 10: 	In R20, RAN4 should focus on the testing issue and develop the standardized models under assumptions that account for scalability.

Proposal 11: 	Regarding the model structure, it could be captured in 3GPP by a description of the architecture.

Proposal 12: 	Regarding the model parameter(or model with parameter), it could be stored on 3GPP server, while the storage location of these model parameters would be referenced in a relevant 3GPP TS or TR documents.

Proposal 13: 	Regarding the test metric and channel model used for CSI compression, the same assumptions as those for the CSI prediction could be reused.

Proposal 14: 	In R20, RAN4 needs to clarify the expected performance level, along with the acceptable model complexity, parameter count, and model size.

Reference

RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

Appendix

Appendix 1 – Simulation results from RAN4 R19

CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion are shown as below:

CNN based CSI encoder 								CNN based CSI decoder

Figure 2  CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion

FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Based on the CNN based CSI encoder and CNN based CSI decoder proposed by RAN4#111, performance evaluation results are shown as below(train by UMa channel data, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

Note：Baseline etype2 SGCS is 0.71

However, after adopting the same model structure and the same simulation assumptions (based on RAN4 conclusion), different companies still obtained different simulation results(e.g., in SGCS), as summarized in RAN4#112 meeting.

In RAN4#112 meeting, companies agreed to perform a second round of alignment and refined more simulation parameters as captured in R4-2414447. Simulation results from our side are as follows:

| CSI feedback bits | Quantization of encoder output | SGCS (UMa channel) |
| --- | --- | --- |
| 64 bits | 2 bits quantization for CSI report | 0.717 |
| 32 floats | without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report | 0.771 |

In addition to the agreed CNN-based CSI encoder, three different low-complexity encoders will be evaluated. Their corresponding FLOPs, model size, and backbone descriptions are summarized as below, with detailed descriptions provided in Appendix-2.

|  | FLOPs | Model size | Description |
| --- | --- | --- | --- |
| Encoder-1 | 94.1M | 0.13Mbyte | CNN-based encoder |
| Encoder-2 | 3.63M | 1.6Mbyte | Transformer-based low-complexity encoder |
| Encoder-3 | 4.14M | 92Kbyte | CNN-based low-complexity encoder |
| Encoder-4 | 0.22M | 0.11Mbyte | MLP- based low-complexity encoder |

As discussed in 3GPP RAN4 Meeting #115, the verification procedures for these low-complexity encoders are outlined as below

- SGCS-0: Based on the agreed CSI Encoder-1 structure and the agreed decoder structure, trained on the mixed dataset to obtain SGCS-0.

- SGCS-1: Compared to SGCS-0, the CSI decoder is frozen. Encoder 1 is trained on the mixed dataset to obtain SGCS-1.

- SGCS-2: Compared to SGCS-1, multiple encoders are evaluated, including Encoder-2 (Transformer-based), Encoder-3 (CNN-based), and Encoder-4 (MLP-based). The decoder remains the Samsung-provided frozen decoder.

- SGCS-3: Compared to SGCS-2, the decoder is no longer frozen but is constrained to the agreed CSI decoder structure and participates in training. A dedicated decoder is joint-trained with each encoder using the mixed dataset, to obtain corresponding SGCS-3 per encoder.

- SGCS-4: Compared to SGCS-3, one decoder is trained to match different encoders, instead of training a dedicated decoder per encoder.

The updated simulation results for low-complexity encoders with different backbones are shown as below

|  | （1-to-1） Joint training | New encoder training with frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- |
| Encoder-1 | SGCS-0 0.7054 | SGCS-1 0.7040 | SGCS-4 (step-1) 0.6871 | SGCS-4 (step-2) 0.6935 |
| Encoder-2 | SGCS-3 0.7175 | SGCS-2 0.6993 | SGCS-4 (step-1) 0.6926 | SGCS-4 (step-2) 0.7095 |
| Encoder-3 | SGCS-3 0.6988 | SGCS-2 0.6991 | SGCS-4 (step-1) 0.6851 | SGCS-4 (step-2) 0.6899 |
| Encoder-4 | SGCS-3 0.6775 | SGCS-2 0.6381 | SGCS-4 (step-1) 0.6698 | SGCS-4 (step-2) 0.6709 |

Appendix 2 – Model structure assumptions for encoder with lower complexity

## (a)

## (b)                                                                                                        (c)

Figure 3  Model assumptions for encoder with lower complexity

(a) Transformer based lower-complexity encoder (Encoder 2)

(b) CNN based lower-complexity encoder (Encoder 3)

(c) MLP based lower-complexity encoder (Encoder 4)