3GPP TSG-RAN5 Meeting #116bis	R4-2513841

# Prague, Czech Republic, 13th Oct 2025 – 17th Oct 2025

Agenda item:	4.2.8

Source: 	Viasat, Space 42, Aalyria

Title: 	Missing ETSI emission requirements for IoT NTN bands 253 and 255 and NR NTN band n255

Document for:	Approval

## 1.	Introduction

During the RAN4#116 meeting, as part of the combined L-Band WI [1], it was agreed to capture the two sets of emissions to address the existing as well as ongoing work on the ETSI harmonized standards [2].

This paper discusses implications for other previously specified NTN bands (n255 and bands 253/255) for which the same ETSI harmonized standards are applicable and proposes to specify the two sets of requirements for the legacy n255 and b253/b255.

## 2.	Discussion

In RAN4#114, as part of discussions related to the introduction of the combined L-Band, it was agreed that there exist ETSI requirements for unwanted emissions that the UE must comply with for the UL frequency ranges being considered as part of the combined L-bands.

In RAN4#116, an LS was received from ETSI TC SES [3], providing progress update on further refinement of the harmonised standards related to the mobile earth station operation in the L-band. As a result, during RAN4#116, for n250, n251 and n253, it was agreed to include two sets of emissions requirements and corresponding NS flags and A-MPR values [2]. It was also agreed that the requirements in ETSI EN 301 681 [4] are still in force and are applicable to 3GPP spectrum bands specified in the L-band spectrum ranges listed in [1]. Furthermore, it was also agreed that once the new ETSI standards are ratified, the emission requirements in the 3GPP specifications may be updated to reflect that the newer ETSI standard what would be in force.

Observation 1: Two sets of requirements have been specified for NR NTN bands n250, n251 and n253 (Annex B) – set 1 based on current ETSI requirements in EN 301 681 [4, Annex A] and set 2 based on requirements likely to be included in the new ETSI standard inferred from [3].

Observation 2: RAN4 has agreed that given that the new ETSI standard has not been completed, the emissions requirements and the NS flags and A-MPR values associated with set 1 requirements (current ETSI requirements in EN 301 681) are in force.

The uplink frequency range of the existing L-bands (NR NTN band - n255, IoT NTN bands - 253/255) specified as part of 3GPP releases 17 and 18 are identical to the new combined L-bands n250 and n251. Therefore, the same ETSI emission requirements are also applicable to those IoT and NR NTN 3GPP bands.

| IoT/NR NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| b255/ n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| b253/n253 | 1668 MHz – 1675 MHz | 1518 MHz – 1525 MHz | FDD |
| b251/n251 | 1626.5 MHz – 1660.5 MHz | 1518 MHz – 1559 MHz | FDD |
| b250/n250 | 1668 MHz – 1675 MHz | 1518 MHz – 1559 MHz | FDD |

Observation 3: The ETSI emissions requirements specified for bands n251 also apply to NR NTN band n255 but have not been incorporated in TS 38.101-5.

Observation 4: The ETSI emissions requirements specified for bands n251 and n253 also apply to IoT NTN bands 255 and 253 respectively but have not been incorporated in TS 36.102.

Furthermore, as part of the HPUE NTN UE Rel-19 work, RAN4 has specified PC2 NR NTN UE for n255 and PC2/1 for band 255. However, since the ETSI emissions requirements were not captured and the NS flags associated with those requirements not specified, A-MPR evaluations for these additional requirements were also not performed.

Observation 5: The A-MPR assessment for PC2 n255 NR NTN UE was performed without considering the ETSI requirements.

Observation 6: The A-MPR assessment for PC2 and PC1 band 255 IoT NTN UEs was performed without considering the ETSI requirements.

Given that ETSI requirements and associated power back-offs required to meet those requirements have not been specified for n255 and b253/b255, it is unclear how an NTN operator authorized to deploy the L-Band spectrum in a country/region can signal to n255/b253/b255 NTN UEs that it is required to meet additional emission requirements to comply with the ETSI requirements.

Furthermore, it is unclear how the UE manufacturers will be able to pass the certification without the implementation of proprietary back-offs. Without knowing the proprietary back-off being implemented by the UE manufacturers, it is unlikely that NTN operators will be able to plan for and ensure that the deployment can enable connectivity across all UEs.

Observation 7: If the ETSI requirements are not specified for n255/b253/b255 operators (like Viasat, Space42) will be unable to signal that the UE needs to comply with additional emission requirements in Region 1 and any other country where ETSI emissions requirements are required to be met.

Observation 8: If proprietary power back-off implementations are implemented to pass regulatory certification, operators will be unable to plan its deployment such that the UE can close the link using the allotted/assigned RBs – this will likely result in severe degradation and/or connectivity failure for its customers.

There are currently no UEs in the market that support NR NTN band n255. Therefore, it is proposed that the emission requirements in TS 38.101-5 be updated from Release-17 specification onwards. Draft CRs have been provided for information in R4-2513842, R4-2513843 and R4-2513844 at this meeting.

Proposal 1: Update the n255 specifications in TS 38.101-5 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 NR NTN UE from Rel-17 specifications onwards.

Proposal 2: Update the Rel-19 n255 specifications to specify the A-MPR for PC2 NR NTN UE for the flags associated with the two sets of requirements.

With respect to IoT NTN bands 253 and 255, it is unclear if there are commercially available 3GPP compliant IoT NTN UEs. Therefore, it is proposed that emission requirements in TS 36.102 be updated from Release-18 specification onwards. Draft CRs have been provided for information in R4-2513858 and R4-2513859 at this meeting.

Proposal 3: Update the band 253 and 255 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 IoT NTN UE from Rel-18 specifications onwards.

Proposal 4: Update the Rel-19 specifications for b255 to specify the A-MPR for PC2 and PC1 IoT NTN UEs for the flags associated with the two sets of requirements.

## 3.	Conclusion

Observation 1: Two sets of requirements have been specified for NR NTN bands n250, n251 and n253 (Annex B) – set 1 based on current ETSI requirements in EN 301 681 [4, Annex A] and set 2 based on requirements likely to be included in the new ETSI standard inferred from [3].

Observation 2: RAN4 has agreed that given that the new ETSI standard has not been completed, the emissions requirements and the NS flags and A-MPR values associated with set 1 requirements (current ETSI requirements in EN 301 681) are in force.

Observation 3: The ETSI emissions requirements specified for bands n251 also apply to NR NTN band n255 but have not been incorporated in TS 38.101-5.

Observation 4: The ETSI emissions requirements specified for bands n251 and n253 also apply to IoT NTN bands 255 and 253 respectively but have not been incorporated in TS 36.102.

Observation 5: The A-MPR assessment for PC2 n255 NR NTN UE was performed without considering the ETSI requirements.

Observation 6: The A-MPR assessment for PC2 and PC1 band 255 IoT NTN UEs was performed without considering the ETSI requirements.

Observation 7: If the ETSI requirements are not specified for n255/b253/b255 operators (like Viasat, Space42) will be unable to signal that the UE needs to comply with additional emission requirements in Region 1 and any other country where ETSI emissions requirements are required to be met.

Observation 8: If proprietary power back-off implementations are implemented to pass regulatory certification, operators will be unable to plan its deployment such that the UE can close the link using the allotted/assigned RBs – this will likely result in severe degradation and/or connectivity failure for its customers.

Proposal 1: Update the n255 specifications in TS 38.101-5 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 NR NTN UE from Rel-17 specifications onwards.

Proposal 2: Update the Rel-19 n255 specifications to specify the A-MPR for PC2 NR NTN UE for the flags associated with the two sets of requirements.

Proposal 3: Update the band 253 and 255 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 IoT NTN UE from Rel-18 specifications onwards.

Proposal 4: Update the Rel-19 specifications for b255 to specify the A-MPR for PC2 and PC1 IoT NTN UEs for the flags associated with the two sets of requirements.

## 4.		References

[1]	RP-241689, "Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges ", Inmarsat, Viasat

[2]	R4-2511861, CR to TS 38.101-5: Introduction of combined L-bands

[3]	R4-2509023, LS on Harmonised Standard for NTN capable UE, ETSI TC SES

[4]	ETSI EN 301 681, Harmonised Standard for Mobile Earth Stations (MES) ofGeostationary mobile satellite systems, including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) under the Mobile Satellite Service (MSS), operating in the 1,5 GHz and 1,6 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

[5]	TS 38.101-5, ver 19.2.0

- Annex A: Current ETSI requirements for L-bands [4]

Table A-1: In-band emission requirements for a NTN UE with UL operation in 1626.5-1660.5MHz frequency range

Table A-2: In-band emission requirements for a NTN with UL operation in 1668-1675MHz frequency range

Table A-3: Out-of-band emission requirements for a NTN UE with UL operation in 1626.5-1660.5MHz frequency range

Table A-4: Out-of-band emission requirements for a NTN UE with UL operation in 1668-1675MHz frequency range

- Annex B: RAN4 specifications for n250 and n251 [5]

## B-1: Two NS Values for the two sets of emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz (NS_09N, NS_13N) and 1668 to 1675 MHz (NS_10N, NS_14N)

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_09N | 6.5.2.3.3 6.5.3.3.8 | n250, n253 | 5 |  | Clause 6.2.3.7 |
| NS_10N | 6.5.2.3.3 6.5.3.3.9 | n251 | 5, 10, 15, 20 |  | Clause 6.2.3.8 |
| NS_13N5 | 6.5.3.3.10 |  | 5 |  |  |
| NS_14N5 | 6.5.3.3.10 |  | 5, 10, 15, 20 |  | Clause 6.2.3.9 |
| NOTE 1: This NS can be signalled for NR satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz of the band is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_01 label with the field additionalPmax [8] absent is default for all NR satellite bands. NOTE 4: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation NOTE 5: NS_09N (n253, n250), NS_10N (n251) are currently in force. NS_13N (n253, n250), NS_14N (n251) may supersede NS_09N, NS_10N when the new ETSI requirements get ratified. |  |  |  |  |  |

## B-2: Emission requirements for two L-band uplink frequency ranges: 1626.5 to 1660.5 MHz and 1668 to 1675 MHz

##### 6.5.2.3.3	Requirements for network signalling value “NS_09N" and “NS_10N"

When "NS_09N" or "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3-1 for any channel bandwidth configured within the corresponding band.

Table 6.5.2.3.3-1: Additional requirements for "NS_09N" and "NS_10N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5.3.3.8	Requirement for network signalling value "NS_09N"

When "NS_09N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.8-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.8-1: Additional out-of-band requirements for "NS_09N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1666.5 | -25 | 30 kHz | Average |
| 1666.5 to 1667.575 | -20 to -35 | 3 kHz | Average |
| 1667.575 to 1667.875 | -20 | 3 kHz | Average |
| 1667.875 to 1667.975 | 15 to - 20 | 3 kHz | Average |
| 1667.975 to 1668 | 30 to 15 | 3 kHz | Average |
| 1675 to 1675.025 | 30 to 15 | 3 kHz | Average |
| 1675.025 to 1675.125 | 15 to - 20 | 3 kHz | Average |
| 1675.125 to 1675.425 | -20 | 3 kHz | Average |
| 1675.425 to 1676.5 | -20 to -35 | 3 kHz | Average |
| 1676.5 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5.3.3.9	Requirement for network signalling value "NS_10N"

When "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.9-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.9-1: Additional out-of-band requirements for "NS_10N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1662.5 | -25 | 30 kHz | Average |
| 1662.5 to 1665.5 | -30 | 30 kHz | Average |
| 1665.5 to 1670.5 | -30 | 100 kHz | Average |
| 1670.5 to 1680.5 | -30 | 300 kHz | Average |
| 1680.5 to 1690.5 | -30 | 1 MHz | Average |
| 1690.5 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5.3.3.10	Requirement for network signalling value "NS_14N"

When "NS_14" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.10-1: Additional out-of-band requirements for "NS_14N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |