3GPP RAN WG4 Meeting #116bis		R4-2513853

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	7.4.3.1

Source:	Apple

Title:	On RedCap UE 3MHz channel bandwidth REFSENS

WI/SI:	NR_RRM_Ph6-Core

Release:	Rel-20

Document for:	Approval

# 1	Introduction

3MHz channel bandwidth was first introduced for NR in Rel-18 [1] where the support has been limited to non-RedCap UE only ever since. Considering the 3MHz channel bandwidth support is well aligned with the nature of RedCap UE from the lower complexity and lower power consumption perspective, there has been strong demand from operators to extend the 3MHz channel bandwidth support to RedCap UE. With this motivation, the objective of introducing less than 5MHz channel bandwidth (or specifically 3MHz channel bandwidth) for (e)RedCap UE is hence included in the Rel-20 RAN4 RRM phase 6 WID [2] which was approved in RAN #109 meeting. Under the RRM phase 6 WID, there are UE RF requirements corresponding to the support of less than 5MHz channel bandwidth to be developed for bands n100, n106, n26, n28, and n85 which are primarily the REFSENS requirements for 2Rx with both FD-FDD and HD-FDD and 1Rx with both FD-FDD and HD-FDD configurations.

In this contribution, we share our inputs on the RedCap UE 3MHz channel bandwidth REFSENS requirements for 2Rx FD-FDD, 2Rx HD-FDD, 1Rx FD-FDD, and 1Rx HD-FDD configurations for the intended NR bands.

# Discussion

## 2.1	2Rx FD-FDD

The 3MHz channel bandwidth REFSENS for 2Rx FD-FDD configuration have already been specified for non-RedCap UE in 38.101-1 [3] Table 7.3.2-1a which are also applicable for RedCap UE. Table 2.1-1 summarizes the requirements for the intended NR bands.

Observation 1: The 3MHz channel bandwidth REFSENS for 2Rx FD-FDD configuration have already been specified for non-RedCap UE which are also applicable for RedCap UE.

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -99.7 | 15 |
| n28 | 15 | -100.2 | 15 |
| n85 | 15 | -99.2 | 15 |
| n100 | 15 | -102.2 | 15 |
| n106 | 15 | -99.2 | 15 |

Table 2.1-1 RedCap UE 3MHz channel bandwidth REFSENS requirements for 2Rx FD-FDD

## 2.2	2Rx HD-FDD

For 2Rx HD-FDD configuration, since there is no Tx self-interference, the 3MHz channel bandwidth REFSENS requirements can be derived based on downlink NRB scaling from 5MHz channel bandwidth REFSENS which have been specified in 38.101-1 [3] Table 7.3I.2-2. For bands n100 and n106, since the 5MHz REFSENS requirements have not been specified for RedCap UE, we can use the same way for deriving 2Rx HD-FDD REFSENS from 2Rx FD-FDD REFSENS for RedCap UE as proposed in [4] where for n100, the difference is 0 dB and for n106, the difference is 0.8 dB. Table 2.2-1 summarizes the requirements for the intended NR bands.

Observation 2: For 2Rx HD-FDD configuration, the 3MHz channel bandwidth REFSENS requirements can be derived based on the following formula:

2Rx HD-FDD REFSENS_5MHz – 10log10(25/15) dB = 2Rx HD-FDD REFSENS_5MHz – 2.2 dB

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -100.5 | 15 |
| n28 | 15 | -101.5 | 15 |
| n85 | 15 | -100.0 | 15 |
| n100 | 15 | -102.2 | 15 |
| n106 | 15 | -100.0 | 15 |

Table 2.2-1 RedCap UE 3MHz channel bandwidth REFSENS requirements for 2Rx HD-FDD

## 2.3	1Rx FD-FDD

For 1Rx FD-FDD configuration, since Tx self-interference is negligible for 3MHz channel bandwidth, REFSENS can be derived from 2Rx FD-FDD configuration REFSENS by adding 2.5dB relaxation to account for the loss of Rx diversity gain, similar to what has been specified for 5MHz channel bandwidth for RedCap UE. Table 2.3-1 summarizes the requirements for the intended NR bands.

Observation 3: For 1Rx FD-FDD configuration, the REFSENS can be derived from 2Rx FD-FDD configuration REFSENS by adding 2.5dB relaxation to account for the loss of Rx diversity gain.

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -97.2 | 15 |
| n28 | 15 | -97.7 | 15 |
| n85 | 15 | -96.7 | 15 |
| n100 | 15 | -99.7 | 15 |
| n106 | 15 | -96.7 | 15 |

Table 2.3-1 RedCap UE 3MHz channel bandwidth REFSENS requirements for 1Rx FD-FDD

## 2.4	1Rx HD-FDD

For 1Rx HD-FDD configuration, since there is no Tx self-interference, the REFSENS can be derived from 2Rx HD-FDD configuration REFSENS by adding 2.5dB relaxation to account for the loss of Rx diversity gain, similar to what has been specified for 5MHz channel bandwidth for RedCap UE. Table 2.4-1 summarizes the requirements for the intended NR bands.

Observation 4: For 1Rx HD-FDD configuration, the REFSENS can be derived from 2Rx HD-FDD configuration REFSENS by adding 2.5dB relaxation to account for the loss of Rx diversity gain.

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -98.0 | 15 |
| n28 | 15 | -99.0 | 15 |
| n85 | 15 | -97.5 | 15 |
| n100 | 15 | -99.7 | 15 |
| n106 | 15 | -97.5 | 15 |

Table 2.4-1 RedCap UE 3MHz channel bandwidth REFSENS requirements for 1Rx HD-FDD

# 3	Conclusion

In this contribution, we share our inputs on the RedCap UE 3MHz channel bandwidth REFSENS requirements for 2Rx FD-FDD, 2Rx HD-FDD, 1Rx FD-FDD, and 1Rx HD-FDD configurations for the intended NR bands.

Proposal: RAN4 to consider the 3MHz channel bandwidth REFSENS requirements in the tables below for RedCap UE under 2Rx FD-FDD, 2Rx HD-FDD, 1Rx FD-FDD, and 1Rx HD-FDD configurations respectively.

2Rx FD-FDD

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -99.7 | 15 |
| n28 | 15 | -100.2 | 15 |
| n85 | 15 | -99.2 | 15 |
| n100 | 15 | -102.2 | 15 |
| n106 | 15 | -99.2 | 15 |

2Rx HD-FDD

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -100.5 | 15 |
| n28 | 15 | -101.5 | 15 |
| n85 | 15 | -100.0 | 15 |
| n100 | 15 | -102.2 | 15 |
| n106 | 15 | -100.0 | 15 |

1Rx FD-FDD

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -97.2 | 15 |
| n28 | 15 | -97.7 | 15 |
| n85 | 15 | -96.7 | 15 |
| n100 | 15 | -99.7 | 15 |
| n106 | 15 | -96.7 | 15 |

1Rx HD-FDD

| Band | SCS (kHz) | REFSENS (dBm) | UL Config (RB) |
| --- | --- | --- | --- |
| n26 | 15 | -98.0 | 15 |
| n28 | 15 | -99.0 | 15 |
| n85 | 15 | -97.5 | 15 |
| n100 | 15 | -99.7 | 15 |
| n106 | 15 | -97.5 | 15 |

# 4	References

RP-220401 “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 7th – 13th, 2022

RP-252955 “WID: NR Radio Resource Management (RRM) Phase 6”, Apple, 3GPP TSG RAN Meeting #109, Beijing, China, September 15th – 18th, 2025

3GPP TS 38.101-1 V19.3.0 (2025-09)

R4-2117993 “RedCap UE REFSENS requirements”, Apple, 3GPP RAN WG4 Meeting #101-e, Online, November 1st – 12th, 2021