3GPP RAN WG4 Meeting #116bis		R4-2513856

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	11

Source:	Apple

Title:	On band combination database improvements

WI/SI:

Release:	Rel-20

Document for:	Decision

# 1	Introduction

The agreement to transition from a table-based format capturing thousands of band combinations to a database in Rel-20 time frame[1] is a major leap forward for RAN4. During Rel-20 time frame a database will be finalised containing all CA and DC combinations from TS 38.101-1/2/3. This will allow simplified handling and data processing. The database consists of individual JSON files storing the data in a dedicated structured format.

This contribution discusses certain challenges and issues discovered when processing the test files currently available for CA combinations.

Additional, a proposal is made to consider a dedicated database for A-MPR requirements.

# 2	Improvements to JSON file format and notation

While working on the existing JSON schema it has been found that certain modification could be needed for accuracy against RAN4 agreements and for processing purposes.

##### 2.1 Carrier Aggregation Notation

When RAN4 discussed the band combination handling and notation in 2018 it was agreed that band combination notations follow a specific format. This notation system is specified in TR 38.817-01 [2]. In clause 4.1 the technical report defines the standardized representation methods for NR-only configurations, EN-DC and SUL combinations. It establishes the fundamental notation principles, using Arabic numerals for LTE bands and "n"-prefixed numerals for NR bands, along with specific formatting conventions to distinguish between intra-band/inter-band and contiguous/non-contiguous deployments. The specification includes detailed examples and bandwidth class representations for different combination types, providing fundamental reference for notation of band combinations.

Part of the essential notation format is the use of band combination prefixes such as ‘CA’, ‘DC’ and ‘SUL’. Those are necessary to differentiate between various combination types (i.e. NR-CA, NR-DC, EN-DC etc.). It is a fundamental part of the notation which is repeated and expended upon in TR 38.846 clause 6.1 [3] and will also be part of the new PRD document. The TR defines the prefix notation for band combinations which are "CA_", "DC_", "SUL_", "V2X_" and makes them mandatory. Below is an excerpt from clause 6.1B listing the first four rules for the notation of band combinations. The focus lies on the first bullet which makes it mandatory for a configuration to start with either “CA_”, “DC_” or any other prefix.

Removing band combination prefixes from JSON files creates issues going beyond simple reader confusion. The prefixes serve as essential semantic identifiers providing the type of the band combination. Dropping the prefix in the database breaks the notation rules set up in TR 38.817-01 [2] and TR 38.846 [3]. Furthermore, the removal creates overhead by requiring custom parsing logic and exception handling since the source repository needs to be included as “part” of the band combination notation. The reliance on external context to complete the context of a band combination (e.g. whether it is a NR-CA or NR-DC combo) is not a good approach and RAN4 should not take this direction for 5G or 6G specification. Instead, the band combination prefix should be included in the database according to the specified notation rules.

Observation 1: RAN4 established mandatory band combination prefixes (CA, DC, SUL, V2X) in TR 38.817-01 and TR 38.846 to create a standardized notation system that differentiates between various combination types using clear semantic identifiers. This will be part of the new PRD document as well. Removing these prefixes from JSON files violates the agreed specification rule. This approach forces reliance on external context (e.g. source repository) to determine combination types. The reliance on external context to complete the information on a band combination (e.g. whether it is a NR-CA or NR-DC combo) is not a good approach and RAN4 should not take this direction for 5G or 6G specification.

Proposal 1: We propose that the band combination prefixes (CA, DC, SUL, V2X) should be included in the database according to the notation rules from TR 38.817-01 and TR 38.846.

##### 2.2 Wi-code

When writing CRs it is mandatory to provide the work item codes for each band combination which has been modified/corrected. It’s a time-consuming task to find all the work items and check which WI the combination is defined in. It would be a considerable quality of live improvement if the WI-code would be captured in the JSON files. The WI-code field should be mandatory for band combinations.

Proposal 2: To improve CR writing efficiency and quality of life, we propose adding mandatory WI-codes to JSON files for band combinations.

##### 2.3 CR title page

Even though it is not a big practical issue, it is not clear whether the CR title page should provide any indication on whether the JSON file is attached. Our view is that it would be nice to have some indication on the CR cover page so that other companies, and a secretary, know that the JSON attachment exists. And if so agreed, it would be also nice to clarify which field on the CR cover page should contain that information.

Proposal 3: We propose adding a clear indication on the CR title page to show when a JSON file is attached to improve visibility.

##### 2.4 Format of changes

One of the first issues which we faced is in which format we should be submitting JSON file modifications. The simplest approach is to attach the original JSON file with the modifications. While it is doable and is perfectly fine, this would be somewhat against the principles of "software development" approach which we try to adopt as it may result in the same issues with implementation of changes if several companies bring changes to the same band combination. In this case each company may submit a modified JSON file, not necessarily with the same changes, which will require somebody to figure out what the actual changes are and to merge them accordingly. Thus, a more elegant approach would be to submit a "diff" file containing actual changes (or both modified JSON file and a diff file). Having "diff" files firstly simplifies and automates the process of applying changes coming from different sources. Furthermore, if the changes are conflicting, the corresponding tools can immediately indicate it thus eliminating human errors if the same changes would be applied manually.

Proposal 4: We propose submitting JSON file modifications as "diff" files rather than complete modified files to enable automated merging, prevent conflicts when multiple companies modify the same band combinations, and to increase visibility of changes made to the original file.

##### 2.5 Database for A-MPR

The JSON based database offers improved machine readability allowing more efficient and faster data crunching. Analog to band combinations the A-MPR tables (regions and values) need to be transferred from word-based specification. Lots of effort is needed to guarantee error free conversion from word tables into internal databases. Major improvements would be expected if A-MPR requirements would also be provided as database.

Observation 2: Analog to band combinations the A-MPR tables (regions and values) need to be transferred from word-based specification. Lots of effort is needed to guarantee error free conversion from word tables into internal databases. Major improvements would be expected if A-MPR requirements would also be provided as database.

Proposal 5: RAN4 should discuss whether A-MPR requirements could be moved into a database.

# 3	Conclusion

This contribution discusses certain challenges and shortcomings of the current JSON database implementation and suggests improvements. As final suggestion it is proposed to consider a database approach for A-MPR requirements. The following observations and proposal are made:

Observation 1: RAN4 established mandatory band combination prefixes (CA, DC, SUL, V2X) in TR 38.817-01 and TR 38.846 to create a standardized notation system that differentiates between various combination types using clear semantic identifiers. This will be part of the new PRD document as well. Removing these prefixes from JSON files violates the agreed specification rule. This approach forces reliance on external context (e.g. source repository) to determine combination types. The reliance on external context to complete the information on a band combination (e.g. whether it is a NR-CA or NR-DC combo) is not a good approach and RAN4 should not take this direction for 5G or 6G specification.

Proposal 1: We propose that the band combination prefixes (CA, DC, SUL, V2X) should be included in the database according to the notation rules from TR 38.817-01 and TR 38.846.

Proposal 2: To improve CR writing efficiency and quality of life, we propose adding mandatory WI-codes to JSON files for band combinations.

Proposal 3: We propose adding a clear indication on the CR title page to show when a JSON file is attached to improve visibility.

Proposal 4: We propose submitting JSON file modifications as "diff" files rather than complete modified files to enable automated merging, prevent conflicts when multiple companies modify the same band combinations, and to increase visibility of changes made to the original file.

Observation 2: Analog to band combinations the A-MPR tables (regions and values) need to be transferred from word-based specification. Lots of effort is needed to guarantee error free conversion from word tables into internal databases. Major improvements would be expected if A-MPR requirements would also be provided as database.

Proposal 5: RAN4 should discuss whether A-MPR requirements could be moved into a database.

# 4	References

- R4-2511804, “WF for the ETSI MCC band combination database”, Nokia, 3GPP TSG-RAN WG4 Meeting #116

- TR 38.817-01, “General aspects for User Equipment (UE) Radio Frequency (RF) for NR”, v16.4.0, 3GPP

- TR 38.846, “Study on simplification of band combination specification for NR and LTE”, v18.1.0, 3GPP