# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513866

# Prague, Czech, Oct. 13-17, 2025

Agenda Item:	7.6.4.1

Source: 	Sharp

Title: 	UE RF requirements to support HD-FDD for Ku/Ka bands with FR1 numerologies

Document for:	Discussion and decision

# Introduction

A new Rel-20 work item “Enhanced requirements for NR NTN and IoT NTN Phase 2” in RP-252962 was approved at RAN#109, and the following objective is included:

HD-FDD for NTN Core Part

Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies

Phase 1: Ku band with FR1 numerology

Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

Phase 2:  Ku band and Ka band with FR2 numerology

Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

Phase 1 work will commence immediately.  Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability

Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

This contribution provides our views on phase 1 of this matter.

# Discussions

## Confirmation of the RAN4 work target for this objective

The objective for phase 1 (i.e. Ku band with FR1 numerologies) includes two parts: (part 1) Introduction of HD-FDD operation without small antennas, and (part 2) Introduction of HD-FDD operation with small antennas.

For part 1 of phase 1, the benefit of the introduction of HD-FDD is to achieve the smaller (i.e. half) antenna size thanks to the avoidance of separate antenna panels for UL and DL while the antenna gain is kept. With this understanding, we don’t think the requirements on (min peak) EIPR and EIS need to be updated compared with the legacy VSAT types (2, 3, 5 and 6). The necessary update of the specs from UE RF perspective with respect to part 1 of phase 1 is to introduce HD-FDD capability for such VSAT types.

For part 2 of phase 1, only a single new VSAT type can be introduced to enable small size antenna (e.g. 20 x 20cm) because of the concerns to specify many VSAT types into the specifications. We share this concern, and the introduction of new VSAT type(s) should be avoided as much as possible to keep the specifications simple. Thus, our preference is to add notes applied to the existing VSAT type(s) which is to specify the new requirements for HD-FDD operation with small antenna, rather than introducing a new VSAT type. However, if a new VSAT type is introduced, the WID description is not clear enough to identify which existing VSAT type could be the baseline for the new single VSAT type. We would like to propose to have VSAT type 6 as a baseline, if a new VSAT type with small antenna is necessary. Meanwhile, the introduction of small antenna will affect the min peak EIRP and EIS requirements and RAN4 needs to discuss how much relaxation we can accept. The relaxed value will reach 6-9 dB according to our initial assessment, and this should be further discussed in RAN4.

Proposal 1: RAN4 to clarify the scope of phase 1 discussion as follows:

- For HD-FDD operation that is not using small antenna (20 x 20 cm), there is no need to update EIRP and EIS requirement for VSAT types of 2, 3, 5 and 6. Other spec impact (e.g. UE capability) can be further discussed.

- For HD-FDD operation that is using small antenna (20 x 20 cm), at least VSAT type 6 should be the target. The goal is to update min peak EIRP and EIS requirements aiming at the relaxation of [around 6-9 – FFS in RAN4] dB. RAN4 should further discuss if a new VSAT is really needed or not when updated requirements are captured in the specification(s).

## Specs affected by the introduction of HD-FDD

Based on the latest TS 38.101-5 capturing the spec support Ku and Ka bands, some part of the UE RF requirements, i.e. EIRP, EIS, off-axis spurious, are dependent on the VSAT UE type. We propose that RAN4 should firstly identify which part of the RAN4 UE requirements to further discuss to support the HD-FDD operation with small antenna as in the objective.

Proposal 2: RAN4 should discuss which parts in TS 38.101-5 will be affected by the introduction of HD-FDD VSAT with small antenna (20 x 20 cm):

- Table 9.2.1.0-1 to introduce a new mobile VSAT types for HD-FDD VSATs with small antenna, if necessary

- Table 9.2.1.2-1 to update Min peak EIRP for small antenna

- Table 9.2.1.2-2 to update Max TRP and Max EIRP for small antenna to add a new VSAT type, if necessary

- Table 9.5.3.2.3.2-1 and 9.5.3.2.3.2-2 to update on-axis spurious limit in “Carrier-on” state to add a new VSAT type, if necessary

- Table 10.3.2-2 to update the EIS for small antenna

- Table 10.4.2-1 to update the maximum input level for small antenna

# Conclusion

This contribution provided our views on UE RF requirements to support HD-FDD for Ku band with FR1. Our proposals are summarized as follows:

Proposal 1: RAN4 to clarify the scope of phase 1 discussion as follows:

- For HD-FDD operation that is not using small antenna (20 x 20 cm), there is no need to update EIRP and EIS requirement for VSAT types of 2, 3, 5 and 6. Other spec impact (e.g. UE capability) can be further discussed.

- For HD-FDD operation that is using small antenna (20 x 20 cm), at least VSAT type 6 should be the target. The goal is to update min peak EIRP and EIS requirements aiming at the relaxation of [around 6-9 – FFS in RAN4] dB. RAN4 should further discuss if a new VSAT is really needed or not when updated requirements are captured in the specification(s).

Proposal 2: RAN4 should discuss which parts in TS 38.101-5 will be affected by the introduction of HD-FDD VSAT with small antenna (20 x 20 cm):

- Table 9.2.1.0-1 to introduce a new mobile VSAT types for HD-FDD VSATs with small antenna, if necessary

- Table 9.2.1.2-1 to update Min peak EIRP for small antenna

- Table 9.2.1.2-2 to update Max TRP and Max EIRP for small antenna to add a new VSAT type, if necessary

- Table 9.5.3.2.3.2-1 and 9.5.3.2.3.2-2 to update on-axis spurious limit in “Carrier-on” state to add a new VSAT type, if necessary

- Table 10.3.2-2 to update the EIS for small antenna

- Table 10.4.2-1 to update the maximum input level for small antenna