3GPP TSG-RAN WG4 Meeting #116bis	R4-2513875

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	7.5.4

Source:	Samsung

Title:	Discussion on Rel-20 performance requirements for 6Rx UE with interference

Document for:	Discussion

- Introduction

In the RAN#109 meeting, a new work item for “NR demodulation performance: Phase 6” has been approved [1], in which additional scenarios with interference for 6Rx UE are important objectives for the enhancement of UE demodulation performance parts. The detailed objective descriptions of 6Rx UE with interference are as below:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

Generally, the enhancement focuses on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario. In this contribution, we provide our analysis and views on the test scopes and simulation assumptions for 6Rx with interference.

- Discussion

  - General Scope

Since this is the first meeting for Rel-20 demodulation performance requirements for 6Rx UE with interference, the scope should be clarified firstly, such as duplex mode, number of Tx antennas, number of Rx antennas, SCS and Bandwidth, TDD UL-DL configuration, DRMS configuration, etc.

Duplex mode

In the latest WID for 6Rx [2], the expected bands for supporting 6Rx as below

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104

According the defination in 38.101-1 as below, these expected bands are all FR1 TDD bands, and RAN4 agreed to define 6Rx demodulation performance requirements without interference for FR1 TDD only in RAN4#114bis meeting [3]. Hence we prefer to define 6Rx demodulation performance requirements with interference only for FR1 TDD.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n7712 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n7917 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n10417,18 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |

Observation 1: Expected bands for 6Rx are all FR1 TDD bands, and RAN4 agreed to define 6Rx demodulation performance requirements without interference for FR1 TDD only in RAN4#114bis meeting.

Proposal 1: Prefer only to define 6Rx demodulation performance requirements with interference for FR1 TDD according the expected bands for 6Rx.

SCS and Bandwidth

For TDD case, we think use typical configuration SCS as 30kHz and Bandwidth as 40MHz is sufficient.

Proposal 2: Typical configuration SCS as 30kHz and Bandwidth as 40MHz is sufficient.

TDD UL-DL configuration

For the TDD UL-DL configuration, 7DS2U with S: 6D+4G+4U configuration defined in FR1.30-1 is a good selection.

Proposal 3: For the TDD UL-DL configuration, use 7DS2U with S: 6D+4G+4U configuration in FR1.30-1.

PDSCH mapping type

For the PDSCH mapping type for 6Rx with interference, prefer to reuse the same configuration for Rel-17 2/4Rx and Rel-19 8Rx UE with interference, i.e., PDSCH mapping type A only.

Proposal 4: Reuse the same PDSCH mapping type configuration for Rel-17 2/4Rx and Rel-19 8Rx UE with interference, i.e., PDSCH mapping type A only.

Number of Tx antennas and Rx antennas

Given RAN4 agreed to define 4 layer demodulation requirements for 6Rx UE without interference with 4Tx and 6Rx antennas [4], and the objective descriptions mentioned ”Up to 4 MIMO layers for target cell” for inter-cell interference scenario and ” Up to 2 MIMO layers for each user” for intra-cell inter-user interference scenario, it is a reasonable selection to reuse 4Tx and 6Rx antenna configurations.

Proposal 5: Set the number of antennas as 4Tx and 6Rx for Rel-20 6Rx demodulation requirements with interference.

  - PDSCH demodulation with inter-cell interference

Interference modelling and power levels

As the same interference modelling and power levels are used by Rel-17 2/4Rx demodulation requirements with inter-cell interference and Rel-19 8Rx demodulation requirements with inter-cell interference, it’s a good solution to reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx.

Proposal 6: For Interference modelling and power levels, reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx.

Propagation condition

Since WID mentioned “Use the Rel-17 interference model for 2/4Rx UE as a baseline”, take the propagation condition for 2/4Rx UE as baseline, which corresponding to TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell, it’s a good selection to reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell.

Proposal 7: For propagation condition of 6Rx UE with inter-cell interference, reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell.

MIMO Rank of serving cell

For the Rank of serving cell, rank 1 is selected for Rel-17 2/4Rx demodulation requirements with inter-cell interference, while rank 2 and rank 4 are agreed for Rel-19 8Rx demodulation requirements with inter-cell interference. The objective description for Rel-20 demodulation performance with inter-cell interference mentioned “Up to 4 MIMO layers for target cell”, which means maximum value of MIMO rank should be 4. However, considering this is the rank discussion for inter-cell interference with MMSE-IRC receiver scenario, which means UE is supposed to be distributed at the cell edge where usually not suitable for high rank. It’s no doubt that rank 2 could be supported by 6Rx UE even with inter-cell interference. But for rank 4, it’s better to evaluate the demodulation performance with inter-cell interference for rank 4 firstly, and decide whether to introduce demodulation requirements based on simulation results.

Proposal 8: It’s no doubt MIMO rank of severing cell could be 2 for inter-cell interference scenario, and whether to introduce demodulation requirements for 4 MIMO layers of serving cell based on simulation results.

MCS values

According the experience on Rel-17 2/4Rx demodulation requirements with inter-cell interference and Rel-19 8Rx demodulation requirements with inter-cell interference, we would like to propose MCS17 (Table 1) for serving cell Rank 2 case, and MCS13 (Table 1) for serving cell Rank 4 case.

Proposal 9: For the serving cell MCS values of 6Rx UE with inter-cell interference, prefer to use MCS17 (Table 1) for serving cell Rank 2 case, and MCS13 (Table 1) for serving cell Rank 4 case.

MIMO Correlation

For the MIMO correlation, ULA Low are used for Rel-17 2/4Rx demodulation requirements with inter-cell interference and Rel-19 8Rx demodulation requirements with inter-cell interference. however, considering different correlation matrixes may be needed for 6Rx FWA and Handheld UE, the MIMO correlation are still on discussion for 6Rx UE without interference scenario as below [5]. Therefore, it is reasonable to follow the same conclusion for MIMO correlation when there is conclusion for 6Rx FWA and Handheld UE.

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Proposals - Option 1: One single test case that is applicable for both FWA and Handheld UE - Option 1a: α = 0 and β = 0.07179 - Option 1b: α = 0 and β = 0 - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

Proposal 10: for the MIMO correlation of 6Rx, it is reasonable to follow the same conclusion when there is conclusion for 6Rx FWA and Handheld UE in Rel-19 6Rx discussion.

Other parameters

For other parameters of 6Rx UE test assumptions with inter-cell interference, reuse the same configuration for Rel-17 2/4Rx as the starting point.

Proposal 11: For other parameters of 6Rx UE test assumptions with inter-cell interference, reuse the same configuration for Rel-17 2/4Rx as the starting point.

  - PDSCH demodulation with intra-cell inter-user interference

For the intra-cell interference with MMSE-IRC receiver scenario, PDSCH demodulation test requirements has been standardized for 2Rx and 4Rx UE antennas with multiple UE pairing, which are using 2x2 and 4x4 antenna configurations seperately.

For 6Rx UE antennas, it’s a good starting point to use current test assumptions for MU-MIMO PDSCH demodulation requirements.

Interference modelling

For the MU-MIMO beamforming model, we suppose it’s still workable to reuse the same MU-MIMO beamforming model for Rel-17 in 38.101-4 B.4.2. For the number of co-UE, it seems 1-co-UE is enough to verify the performance of 6Rx intra-cell inter-user interference scenario.

Proposal 12: For 6Rx intra-cell inter-user interference scenario, reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4, and consider 1 target UE + 1 co-UE configuration.

Propagation condition

For the propagation condition, reuse the same configuration as Rel-17 is a suitable selection.

Proposal 13: For 6Rx intra-cell inter-user interference scenario, reuse the same propagation condition as Rel-17 is a suitable selection, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Precoding method for the co-scheduled UE

Considering the MU-MIMO pairing scheduling strategy in the network is highly related with the long-term beam weights but not with the short-term weight i.e., PMIs, it seems it is too idealization to use orthogonal PMI selection for co-scheduled UE with the target UE. Thus, it is better to use the randomized precoder which is not equal to any column of the precoder matrix of target UE. However, due to Rel-17 MU-MIMO requirements are based on both random and orthogonal PMI selection, we think it’s an acceptable way to proceed.

Proposal 14: For 6Rx intra-cell inter-user interference scenario, reuse the same precoding method as Rel-17 MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

Rank (target UE + co-scheduled UEs) configuration

For the ranks in 2Rx antennas deployment, 1+ 1 for the target UE and co-scheduled UE, while for the ranks in 4Rx antennas deployment, 2+ 2 for the target UE and co-scheduled UE. Hence, for 6Rx with intra-cell inter-user interference scenario, evaluate the demodulation performance for 1+1 and 2+2 cases are recommended.

Proposal 15: For 6Rx intra-cell inter-user interference scenario, the rank assumptions could be Rank 1+1 and Rank 2+2 for the target UE and co-scheduled UE separately.

MIMO Correlation

For the MIMO correlation, ULA Low are used for Rel-17 2/4Rx demodulation requirements with intra-cell inter-user interference and Rel-19 8Rx demodulation requirements with intra-cell inter-user interference. however, considering different correlation matrixes may be needed for 6Rx FWA and Handheld UE, the MIMO correlation are still on discussion for 6Rx UE without interference scenario as mentioned in [5]. Therefore, it is reasonable to follow the same conclusion for MIMO correlation when there is conclusion for 6Rx FWA and Handheld UE.

Proposal 16: For MIMO correlation of 6Rx intra-cell inter-user interference scenario, it is reasonable to follow the same conclusion when there is conclusion for 6Rx FWA and Handheld UE in Rel-19 6Rx discussion.

MCS value

For MCS value of the target UE, we prefer to align with Rel-19 MU-MIMO requirements i.e., MCS 17 (Table 1) for 2+2 case, and consider MCS19 (Table 1) for 1+1 case.

Proposal 17: For 6Rx intra-cell inter-user interference scenario, MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case.

  - Applicability rule and release independent

Test applicability

In order to balance the test coverage and test workload, we prefer to define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 18: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, prefer to define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Release independent

Since the release independent issue about 6Rx is still on discussion on Main session, it is reasonable to wait the conclusion and use the same conclusion for 6Rx demodulation requirements with interference.

Proposal 19: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the release independent should follow the same conclusion about 6Rx in NR_ENDC_RF_Ph4 WI.

- Conclusion

In this contribution, we provide analysis and views on the 6Rx UE demodulation performance with interference objective of NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Observation 1: Expected bands for 6Rx are all FR1 TDD bands, and RAN4 agreed to define 6Rx demodulation performance requirements without interference for FR1 TDD only in RAN4#114bis meeting.

Proposal 1: Prefer only to define 6Rx demodulation performance requirements with interference for FR1 TDD according the expected bands for 6Rx.

Proposal 2: Typical configuration SCS as 30kHz and Bandwidth as 40MHz is sufficient.

Proposal 3: For the TDD UL-DL configuration, use 7DS2U with S: 6D+4G+4U configuration in FR1.30-1.

Proposal 4: Reuse the same PDSCH mapping type configuration for Rel-17 2/4Rx and Rel-19 8Rx UE with interference, i.e., PDSCH mapping type A only.

Proposal 5: Set the number of antennas as 4Tx and 6Rx for Rel-20 6Rx demodulation requirements with interference.

Proposal 6: For Interference modelling and power levels, reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx.

Proposal 7: For propagation condition of 6Rx UE with inter-cell interference, reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell.

Proposal 8: It’s no doubt MIMO rank of severing cell could be 2 for inter-cell interference scenario, and whether to introduce demodulation requirements for 4 MIMO layers of serving cell based on simulation results.

Proposal 9: For the serving cell MCS values of 6Rx UE with inter-cell interference, prefer to use MCS17 (Table 1) for serving cell Rank 2 case, and MCS13 (Table 1) for serving cell Rank 4 case.

Proposal 10: for the MIMO correlation of 6Rx, it is reasonable to follow the same conclusion when there is conclusion for 6Rx FWA and Handheld UE in Rel-19 6Rx discussion.

Proposal 11: For other parameters of 6Rx UE test assumptions with inter-cell interference, reuse the same configuration for Rel-17 2/4Rx as the starting point.

Proposal 12: For 6Rx intra-cell inter-user interference scenario, reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4, and consider 1 target UE + 1 co-UE configuration.

Proposal 13: For 6Rx intra-cell inter-user interference scenario, reuse the same propagation condition as Rel-17 is a suitable selection, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Proposal 14: For 6Rx intra-cell inter-user interference scenario, reuse the same precoding method as Rel-17 MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

Proposal 15: For 6Rx intra-cell inter-user interference scenario, the rank assumptions could be Rank 1+1 and Rank 2+2 for the target UE and co-scheduled UE separately.

Proposal 16: For MIMO correlation of 6Rx intra-cell inter-user interference scenario, it is reasonable to follow the same conclusion when there is conclusion for 6Rx FWA and Handheld UE in Rel-19 6Rx discussion.

Proposal 17: For 6Rx intra-cell inter-user interference scenario, MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case.

Proposal 18: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, prefer to define a test applicability rule to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 19: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the release independent should follow the same conclusion about 6Rx in NR_ENDC_RF_Ph4 WI.

- Reference

[1] RP-252957, WID: NR demodulation performance: Phase 6, RAN#109 meeting, Apple

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#114bis meeting, Huawei, HiSilicon

[4] R4-2508727, Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#115 meeting, Huawei, HiSilicon

[5] R4-2512617, Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#116 meeting, Huawei, HiSilicon