3GPP TSG-RAN WG4 Meeting #116-bis														R4-2513878

Prague, Czech, 13th – 17th October, 2025

Agenda item:			4.13.1.3

Source:	Samsung

Title:	Discussion on HARQ and weighting for dynamic MIMO OTA

Document for:	Discussion

1	Introduction

The core part for dynamic MIMO OTA was completed in last RAN4 meeting, with two main issues remained in maintenance phase:

- Maximum number of HARQ transmission

- The weighting factors value for CMT

In this contribution we share our views on how to move forward for above open issues.

2	Discussion

As indicated in the latest TR 38.762 [1], the maximum number of HARQ transmission parameter for dynamic link adaptation is still FFS.

Table 9.3-1: CSI configuration

| Parameter |  | Unit | Test |
| --- | --- | --- | --- |
| TDD Slot Configuration (Note 1) |  |  | FR1.30-1 |
| SNR |  | dB | 22 |
| Beamforming Model |  |  | As defined in Clause B.4.1 in TS 38.101-4 [8] |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig-periodicity and offset | slot | 10/1 |
| ReportConfigType |  |  | Periodic |
| CQI-table (Note 3) |  |  | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | 10/9 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | 2-1 |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00001111 for follow RI |
| Physical channel for CSI report |  |  | PUCCH |
| CQI/RI/PMI delay (Note 2) |  | ms | 9.5 |
| Maximum number of HARQ transmission |  |  | FFS |
| Note 1: FR1.30-1 is defined in Clause A.1.2 of TS 38.101-4 [8] Note:2: Let CQI/RI/PMI delay corresponds to ‘k’ slots. Then, if the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-k/2)], this reported PMI cannot be applied at the gNB downlink before slot#[(n+k/2)]. Note 3: CQI table to use for CQI calculation (see clause 5.2.2.1 of TS 38.214 [9]) |  |  |  |

In previous meeting, a value of 4 is considered as starting point while measurement comparison is expected between the value 1 and 4, as indicated in the WF of RAN4#112bis meeting [2, R4-2416579]:

Issue 1-2-1: The maximum number of HARQ transmissions Agreements: - Take the following test parameters as the starting point and further check the feasibility and potential impact on dynamic MIMO OTA testing. - Maximum number of HARQ transmission: [4] - Number of HARQ processes for TDD bands: [8] - Number of HARQ processes for FDD bands: [4] - Measurements can be performed to compare the measurement results with Maximum Number of HARQ Transmission = 1 vs. 4.

So the determination of this parameter relies on practical measurement. The weighting factors for performance metric seem also relying on practical measurement. In last meeting, the performance metric for CTMT is as updated as following [1]:

# 5 Performance metrics Let the CDF of the Total Measured Throughput (CTMT) be the figure of merit for the MIMO OTA tests with link adaptation and dynamic channel models. The TP shall be recorded every 200 ms during the emulation of the route separately for each of the M DUT orientations and for each of the L rotations per orientation, defined in Clause B.2. The Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e., TMTUMi=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMi and TMTUMa=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMa with m = 1 to M=4 and l = 1 to L=4. Subsequently, the TMTs from both channel models are combined into a single one TMTUMi&UMa=TMTUMi, TMTUMa Three separate CTMT values shall be determined from the TMT values, i.e., the 10%, 50%, and 90%-ile CDF - CTMTUMi&UMa,10%=CDF10%TMTUMi&UMa - CTMTUMi&UMa,50%=CDF50%TMTUMi&UMa - CTMTUMi&UMa,90%=CDF90%(TMTUMi&UMa) The final CTMT metric is the weighted average of the three CTMTs, i.e., CTMTUMi&UMa=w10CTMTUMi&UMa,10%+w50CTMTUMi&UMa,50%+w90CTMTUMi&UMa,90% with the weights w10, w50, w90 FFS.

Observation 1:	Both the HARQ parameter and the CTMT weighting factors are probably relying on practical measurement.

It would be natural to configure both 1 and 4 for the Maximum Number of HARQ Transmission parameter in the practical measurement for derivation of weighting factors w10, w50, w90 if going with measurement approach.

Proposal 1:	configure both 1 and 4 for the Maximum Number of HARQ Transmission parameter in the practical measurement for derivation of weighting factors w10, w50, w90 if going with measurement approach.

Besides, the sum of weighting factors should equal to 1.

Proposal 2:	the sum of weighting factors w10, w50, w90 should equal to 1.

3	Conclusion

Observation 1:	Both the HARQ parameter and the CTMT weighting factors are probably relying on practical measurement.

Proposal 1:	configure both 1 and 4 for the Maximum Number of HARQ Transmission parameter in the practical measurement for derivation of weighting factors w10, w50, w90 if going with measurement approach.

Proposal 2:	the sum of weighting factors w10, w50, w90 should equal to 1.

Reference

[1] RP-252875	TR 38.762 v1.1.1		Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs (Release 19)	RAN#109

[2] R4-2416579    WF for [112bis][324] MIMO_OTA_Ph3, CAICT