3GPP TSG-RAN WG4 Meeting #116bis			                     	R4-2513892

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	 6.2.2

Source:        ZTE Corporation, Sanechips

Title:           Discussion on performance for intra-band non-collocated EN-DC and NR-CA

Document for:  Approval

Introduction

According to the WID [1], RAN4 shoud specify the corresponding performance requirements for intra-band non-collocated EN-DC/NR-CA for type 4a/4b capable FWA UE. Meanwhile, the performance part should be started from the this meeting. Hence, this contribution paper provides our views on which test shall be specified.

The performance part of the work item includes: - Specify the corresponding performance requirements for type 4a/4b capable FWA UE. - RRM performance requirements - PDSCH demodulation performance requirements

Discussion

As per the WID in Rel-19, RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption have been updated to support type 4 capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC.

In previous meeting, we have finished the update of core requirement based on the legacy requirements for type2 UE which has been supported in Rel-18. That’s to say, the enhancements made to R19 are all based on the requirements of R18, except for the introduction of new signaling to indicate UE capabilities and types. Therefore, for performance part, the agreements for type 2 UE that achieved in Rel-18 can be used as baseline.

Observation 1: The enhancements made to the core part in Rel-19 are all based on the requirements of Rel-18 Type 2 UE, except for the introduction of new signaling to indicate UE capabilities and UE types.

In pervious meeting, an agreement was achieved that no new RRM test case is needed for non-collocated FR1 intra-band NR-CA, and further discuss whether to modify the exiting interruption test case by considering the new RRC signaling for UE capable of type 2 requirements. Finally, the test requirements for interruptions during measurements on deactivated NR SCC in FR1 in A.6.5.2.1.2 need to be revised to cover type 2 UE.

Similar, for Rel-19, there is no new RRM test case is needed for FR1 intra-band non-collocated EN/DC/NR-CA.

RAN4#108bis Issue 2-1: whether to introduce new RRM test case for non-collocated FR1 intra-band NRCA Agreement: - No new RRM test case is needed for non-collocated FR1 intra-band NR-CA. - FFS whether to modify the exiting interruption test case by considering the new RRC signaling for UE capable of type 2 requirements. RAN4#110 Agreement: - The test requirements for interruptions during measurements on deactivated NR SCC in FR1 in A.6.5.2.1.2 need to be revised to cover type 2 UE. Further discuss on the wording based on the CR.

Proposal 1: Similar to Rel-18, no new RRM test case is needed for non-collocated FR1 intra-band EN-DC/NR-CA in Rel-19.

Proposal 2: RAN4 should consider whether any updates are needed to existing test cases to cover type 4 UE.

Conclusions

In this contribution, we provide our views on the performance part for R19 non-collocated EN-DC/NR-CA and the proposals are:

Observation 1: The enhancements made to the core part in Rel-19 are all based on the requirements of Rel-18 Type 2 UE, except for the introduction of new signaling to indicate UE capabilities and UE types.

Proposal 1: Similar to Rel-18, no new RRM test case is needed for non-collocated FR1 intra-band EN-DC/NR-CA in Rel-19.

Proposal 2: RAN4 should consider whether any updates are needed to existing test cases to cover type 4 UE.

References

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA, Moderator (Apple)