3GPP TSG-RAN WG4 Meeting #116bis		                		   R4-2513893

Prague, Czech Republic, Oct. 13-17, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on impacts for measurement event prediction

Agenda Item:	7.12.4

Document for:	Approval

- Introduction

In RAN plenary #109 meeting, a revised WID[1] for Rel-20 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this WID are listed as follows.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In this document, we will provide our views on measurement event prediction of AI/ML for mobility from the RAN4 perspective.

- Discussion

RAN2 has discussed measurement event prediction use-cases and focused on discussing simulation assumptions and results based on measurement event prediction in previous meetings.

During the discussion on SI phase, indirect measurement event prediction and direct measurement event prediction are both within the scope. Indirect measurement event prediction means that intermediate results are needed. That is to say, the prediction of RRM measurement is first performed to obtain the predicted RSRP, and then based on these prediction results, the calculation results of measurement events are further obtained. Correspondingly, direct measurement event prediction does not require to obtain the prediction results of RSRP.

During the discussion of the requirements for indirect event prediction, RAN2 has an agreement that the simulation assumption of RRM measurement prediction can be reused unless otherwise specified, so we can apply the analysis results of the simulation parameters for RRM measurement prediction to measurement event prediction. In the discussion on RRM measurement prediction, we have considered the measurement error in high level that RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy, and the error models has been agreed in the last meeting [2].

Issue 2-1: Impacts of measurement error on prediction accuracy Agreement: RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy: - As a baseline: - For BB error, use link level simulation to generate L3-RSRP difference as baseband error. - For RF error model, use a truncated Gaussian distribution. - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

On the basic, measurement error should be considered into indirect measurement event perdiction when we discuss the prediction accuracy, and error model(s) of RRM measurement prediction can be reused.

Proposal 1: Measurement error should be considered into indirect measurement event prediction use case, and error model of RRM measurement prediction can be reused.

For indirect event measurement prediction, measurement result(s) is predicted by a RRM measurement prediction model first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Therefore, the RRM impact on RSRP difference prediction accuracy in RRM measurement prediction use case can be directly reused for indirect event measurement prediction. In previous meetings, we have discussed the performance metric for event prediction use case and reached consensus blew [3].

Issue 3-1: Performance metric for event prediction use case Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP. Issue 3-1: Performance metric for indirect measurement event prediction Agreement: - On top of the agreement made in RAN4 #115, RAN4 to discuss whether/how to define additional performance metrics for indirect measurement event prediction in WI phase. - Capture the candidate proposals in the WF for information purpose.

As to the issue whether need to define requirement for the predicted RSRPs or not if the report includes the predicted RSRPs, we have reached consensus that RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP if the report includes the predicted RSRPs. In RAN2#131 meeting, they have an agreement that RRM measurements and Time-related information about the predicted event would be include in the event prediction report.

Agreements in RAN2#131 Aim for a single framework for event prediction and performance monitoring. From RAN2 point of view indirect event prediction, RSRP differences can be used as the performance metric for monitoring. Capture in the TR that there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction. Agreements in RAN2#131 1 For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters) 2 For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence) 3 For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

As to the metric of indirect event prediction, RAN2 has consiered the RSRP differences between predicated results and ground truth. To ensure the accuracy of the measurement, RAN4 should define performance metric for event prediction use case based on RAN2’s progress. In our understanding, the requirements for RRM measurement prediction can be used as baseline for intermediate output of indirect event prediction.

Observation 1: According to RAN2’s design, RRM measurements and time-related information about the predicted event would be included in the event prediction report. And RSRP differences can be used as the performance metric for monitoring from RAN2 point of view on indirect event prediction.

Proposal 2: For indirect event prediction, RAN4 to reuse the accuracy requirement for RRM measurement prediction.

In the last meeting, the issue about prediction report delay has reached consensus in AI BM, the overall prediction reporting delay at least includes measurement delay plus reporting delay for aperiodic report, where the reporting delay includes the inference delay. Meanwhile, for semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion.

Agreement in RAN4#116 Issue 2-2: Prediction report delay Agreement: For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay -The reporting delay includes the inference delay For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

For event prediction, UE behavior is different from the legacy procedure, and the predicted report based on event prediction is generally earlier than the measurement reported after the actual event occurs. Therefore, legacy event triggered measurement report delay maybe not suitable for event prediction report delay, and RAN4 should study the impact on event prediction report delay between the point at which the UE initiate the event prediction and the point at which the UE transmits the corresponding event prediction report over the air interface.

Proposal 3: For indirect event prediction, the impact on event prediction report delay should be studied.

- Conclusion

In this contribution, we have the following proposals for the AI/ML mobility on measurement event prediction:

Proposal 1: Measurement error should be considered into indirect measurement event prediction use case, and error model of RRM measurement prediction can be reused.

Observation 1: According to RAN2’s design, RRM measurements and time-related information about the predicted event would be included in the event prediction report. And RSRP differences can be used as the performance metric for monitoring from RAN2 point of view on indirect event prediction.

Proposal 2: For indirect event prediction, RAN4 to reuse the accuracy requirement for RRM measurement prediction.

Proposal 3: For indirect event prediction, the impact on event prediction report delay should be studied.

- Reference

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital

- R4-2508268, WF on FS_NR_AIML_Mob_Part1, Nokia

- R4-2512127, WF on FS_NR_AIML_Mob_Part1, Nokia