3GPP TSG-RAN WG4 Meeting #116bis		                		       R4-2513895

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	4.23.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Event triggered L1 measurement reporting

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

Discussion

In the last meeting, the core part of R19 mobility has been closed. And there are some issues which do not impact the WI core part completion can be further discussed in maintenance phase.

According to the WID, RAN4 should discuss the impacts on RRM requirements depend on the RAN1/2 specification introduced for event triggered L1 measurement reporting. In pervious meetings, we have reached consensus on Event-triggered L1 reporting delay requirement and accuracy requirement. However, there is no consensus on whether to introduce/update requirements for reporting criteria per measurement category [2].

- Issue 2-2: whether to introduce/update requirements for reporting criteria per measurement category Agreement: - No consensus on the necessity to introduce/update requirements for reporting criteria per measurement category. - The issue does not impact the WI core part completion, and interested companies can further discuss this issue in maintenance phase based on contribution driven.

We have discussed this issue many times and it is hard to derive a conclusion on the necessity to introduce/update requirements for reporting criteria per measurement category.

- Option 1: define new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM (CATT, Nokia, CTC, OPPO, Xiaomi, MTK, CMCC, vivo, ZTE, E///)

- Option 2: extend existing Ecat to support LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4. (OPPO, MTK)

    - Concern: CMCC, E///, Nokia, ZTE

- Option 3: not introduce/update requirements for reporting criteria per measurement category

    - Concern: MTK, vivo, E///

For this issue, some companies proposed that requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell in pervious meeting. In our understanding, it is necessary to introduce/update the requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133, which is similar as legacy event triggered L3 measurement reporting. It’s commom understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. Base on this understanding, event triggering L1 reporting for LTM maybe not support if there is no update on requirements for reporting criteria per measurement category.

Our initial viewpoint is to introduce UE capability for triggering and reporting L1 events in LTM, while supporting L1 event triggering and reporting without affecting the original requirements of L3. However, during the last meeting, the specific values reported by the company regarding the L1 incident were discussed, and it seems difficult to unify the relevant values. Through offline discussions, the relevant values focused on two options [3].

Baseline for further discussion:

Reporting criteria for LTM event triggered reporting is [5/9] * 39-3-3 (Number of total cells UE can measure).

The different between two values is the number of event that can be supported per cell, and both of these values are okay for us. If we cannot reach a consensus on the value in this meeting, we can compromise to not introduce/update requirements for reporting criteria per measurement category. As far as we know, MIMO is also handled related issue in this way.

Proposal 1: Define new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM if the number of reporting criteria for LTM event triggered reporting can be unified. Otherwise, not introduce/update requirements for reporting criteria per measurement category.

Conclusions

In this contribution, we put forward the following proposal on R19 L1/L2 triggered mobility.

Proposal 1: Define new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM if the number of reporting criteria for LTM event triggered reporting can be unified. Otherwise, not introduce/update requirements for reporting criteria per measurement category.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2512140, Ad-hoc minutes for NR_Mob_Ph4, Apple