3GPP TSG-RAN WG4 Meeting #116bis		                	  	       R4-2513896

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.17.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on performance requirements for CLTM

Document for:	Approval

Introduction

The RRM core requirement on Mobility work item has been summarized, which include

- Event triggered L1 measurement reporting

- CSI-RS measurements for LTM procedures

- (subsequent) conditional LTM

This contribution provides our view on performance requirements for CLTM.

Discussion

- For (subsequent) conditional LTM, we should define test cases based on R18 LTM. In R18 LTM, we defined the test cases for RACH-based/RACH-less, Intra-frequency/inter-frequency, PCell/PSCell switch from FR1 to FR1 or FR2 to FR2. During Release 18, we have done the down selection when designed the test cases. So, we can use the test list in R18 as baseline when we discuss the test cases for conditional LTM.

| A.6.3.4.1 | RACH-based Intra-frequency PCell switch from FR1 to FR1 |
| --- | --- |
| A.6.3.4.2 | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 |
| A.6.3.4.3 | RACH-less Intra-frequency PCell switch from FR1 to FR1 |
| A.6.3.4.4 | RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement |
| A.6.3.5.1 | RACH-based intra-frequency LTM PSCell switch from FR1 to FR1 |
| A.7.3.4.1 | RACH based Intra-frequency cell switch from FR2 to FR2 |
| A.7.3.4.2 | RACH-less Intra-frequency PCell switch from FR2 to FR2 |
| A.7.3.4.3 | RACH-less Inter-frequency LTM PCell switch from FR2 to FR2 |
| A.7.3.5.1 | Intra-frequency RACH-based LTM PSCell switch from FR2 to FR2 |
| A.6.3.2.4 | LTM PDCCH-order Random Access |
| A.7.3.2.4 | LTM PDCCH-order Random Access |

- According to the discussion of R19 CLTM, the requirement of LTM can be applied to CLTM, so the test case introduced by LTM earlier can serve as the starting point for CLTM test cases. For R19 CLTM, introduce test cases for conditional LTM and test cases defined for R18 LTM cell switch can be used as baseline. In addition, for PDCCH-order random access, we have an agreement in RAN4#113 meeting that the related R18 RAN4 requirements are applicable if no new PDCCH ordered RACH procedure will be introduced in RAN2, which means the same procedure is used for LTM and CLTM. Therefore, for UEs that can pass R18 related tests, they meet the R19 random access requirements and do not require repeated testing.

In the last meeting, we have an agreement that CLTM test at least includes FR1 to FR1 and FR2 to FR2, which is same as LTM. After checking, unlike LTM which can be applied to PCell switch and PSCell switch, CLTM only considers PCell switch, so PSCell switch does not need to be considered. Except for PScell switch, other parts supported by LTM are also supported by CLTM, so they should also be considered.

Observation 1: Conditional PScell switch is not considered in R19 CLTM.

Based on the analysis above, follow aspects should be considered:

- RACH-based & RACH-less

- Intra-f & inter-f

- With & without L1-RSRP measurement for FR1

Proposal 1: Define test cases for CLTM and test cases defined for LTM cell switch can be used as baseline. Follow aspects should be considered:

- RACH-based & RACH-less

- Intra-f & inter-f

- With & without L1-RSRP measurement for FR1

For subsequent CLTM, the main difference compared to CLTM is that it does not require receiving RRC configuration. Therefore, the testing design for subsequent CLTM can be based on CLTM.

Proposal 2: Define test cases for Subsequent CLTM and test cases defined for CLTM can be used as baseline. Follow aspects should be considered:

- FR1 to FR1& FR2 to FR2

- RACH-based & RACH-less

- Intra-f & inter-f

- With & without L1-RSRP measurement for FR1

For UE which can pass subsequent CLTM, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure.

Proposal 3: For UE which can pass subsequent CLTM, test case for CLTM can be skipped.

Conclusions

In this contribution, we put forward the following proposal on performance part for mobility enhancements.

Observation 1: Conditional PScell switch is not considered in R19 CLTM.

Proposal 1: Define test cases for CLTM and test cases defined for LTM cell switch can be used as baseline. Follow aspects should be considered:

- RACH-based & RACH-less

- Intra-f & inter-f

- With & without L1-RSRP measurement for FR1

Proposal 2: Define test cases for Subsequent CLTM and test cases defined for CLTM can be used as baseline. Follow aspects should be considered:

- FR1 to FR1& FR2 to FR2

- RACH-based & RACH-less

- Intra-f & inter-f

- With & without L1-RSRP measurement for FR1

Proposal 3: For UE which can pass subsequent CLTM, test case for CLTM can be skipped.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple