3GPP TSG-RAN WG4 Meeting #116bis		                		     R4-2513901

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.4.3.2

Source: 	ZTE Corporation

Title: 	Discussion on RRM requirements for (e)RedCap

Document for:	Approval

Introduction

In RAN#109 meeting, the WID on NR Radio Resource Management (RRM) Phase 6 has been approved [1]. Core part objectives are shown below for reference:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

In this contribution, we will provide our initial views on the RRM requirements for (e)RedCap UE with Relaxed RLM/BFD and/or less than 5MHz channel bandwidth.

Discussion

From the description of WI, there are two bullets need to be discussed in R20. First, power saving is important for (e)RedCap UE and existing Rel-17 RLM/BFD relaxation for leagcy UE can be applied to (e)RedCap UE for power saving. Meanwhile, to reduce complexity and cost, it is beneficial for (e)RedCap UE to support less than 5 MHz channel bandwidth.

2.1 Relaxed RLM/BFD RRM requirements

Before discussing how to apply relaxed requirements to RedCap UE, let's first review the descriptions of RedCap UE related requirements in existing specification.

For legcay UE, both SSB based and CSI-RS based RLM/BFD are considered for relax measurement. Meanwhile, for redcap UE, Requirements for SSB based radio link monitoring and CSI-RS based radio link monitoring are defined. Based on this, both SSB based and CSI-RS based RLM/BFD relax should be considered for RedCap UE.

|  | SSB based RLM/BFD | CSI-RS based RLM/BFD | SSB based RLM/BFD relaxation | CSI-RS based RLM/BFD relaxation |
| --- | --- | --- | --- | --- |
| Legcay UE | 8.1.2.2/8.5.2.2 | 8.1.3.2/8.5.3.2 | 8.1.2.4/8.5.2.4 | 8.1.3.4/8.5.3.4 |
| RedCap UE | 8.1B.2.2/8.5B.2.2 | 8.1B.3.2/8.5B.3.2 | TBD | TBD |

Proposal 1: Both SSB based and CSI-RS based RLM/BFD relaxation should be considered for (e)RedCap UE.

Taking SSB based RLM as example, we observed that hypothetical PDCCH transmission parameters listed in table are different for 1Rx and 2Rx, such as Aggregation level (CCE) and Bandwidth (PRBs), which will impact RRM requirement.

By comparison, it can be seen that hypothetical PDCCH transmission parameters for the 2Rx Redcap UE and legacy UE are the same. Therefore, the RLM related requirements for Legacy UE can also be applied to the Redcap UE of 2Rx.

Table 8.1.2.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4 dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 8.1B.2.1-1: PDCCH transmission parameters for out-of-sync evaluation for RedCap UE

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4 dB |
| Bandwidth (PRBs) | 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches. |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| NOTE: SCS = 60kHz is not applicable for FR1. |  |

Table 8.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Observation 1: Hypothetical PDCCH transmission parameters are same for legacy UE and 2Rx (e)RedCap UE, and for 1Rx (e)RedCap UE, Aggregation level and Bandwidth are different from legacy UE.

Proposal 2: Define relaxed RLM/BFD measurement requirement for (e)RedCap UE. For 2Rx (e)RedCap UE, the evaluation period TEvaluate_out_SSB_Relax for legacy UE can be reused. For 1Rx (e)RedCap UE, the evaluation period should be extended based on legacy UE.

2.2 Support of less than 5 MHz channel bandwidth

From the WID, we should discuss RRM requirements for RedCap UE when less than 5 MHz channel bandwidth is supported. And R18 3MHz requirements for non-RedCap UE should be uesed as a baseline.

In R18, only SSB based measurement has been considered for 3 MHz requirements for non-RedCap UE. Before the discussion of requirements, we should make the work scope clear and decide whether to study CSI-RS based measurement for RedCap UE.

Observation 2: For R18 non-redcap UE, only SSB based measurements for less than 5MHz were considered, without taking CSI-RS based measurement into account.

Proposal 3: For (e)RedCap UE, SSB based measurement should be considered and the requirements for legacy UE can be used as baseline. Meanwhile, RAN4 should discuss whether to study CSI-RS based measurement for less than 5MHz in Rel-20.

In Rel-18, the impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz has been discussed for single carrier and relative RRM requirements were updated. For the case that CBW equals to 3MHz, PBCH DMRS detection and PBCH decoding maybe impact due to the transmission bandwidth for PBCH is smaller.

According to the current protocol, we have summarized the impact of R18 less than 5MHz on the core requirements of legacy UE, as shown in the table below.

Impact of R18 less than 5MHz on the core requirements of legacy UE

| NR FR1 - NR FR1 Handover | T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. |
| --- | --- |
| PDCCH transmission parameters of RLM | Number of control OFDM symbols Aggregation level (CCE) Bandwidth (PRBs) Mapping from REG to CCE |
| Time period for time index detection | In Rel-18, SSB index identification delay Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples |

Based on the progress of R18, at least the following issues need to be discussed for redcap UE:

- The value of T∆ in handover

- PDCCH transmission parameters of RLM

- Time period for time index detection

Proposal 4: For (e)RedCap UE, Study the following issues for less than 5 MHz:

  - The value of T∆ in handover

  - PDCCH transmission parameters for RLM/BFD

  - Time period for time index detection

  - Other aspects are not preclude

Conclusions

In this contribution, we provide our views on the RRM impacts for (e)RedCap UE with Relaxed RLM/BFD and/or less than 5MHz channel bandwidth.

Proposal 1: Both SSB based and CSI-RS based RLM/BFD relaxation should be considered for (e)RedCap UE.

Observation 1: Hypothetical PDCCH transmission parameters are same for legacy UE and 2Rx (e)RedCap UE, and for 1Rx (e)RedCap UE, Aggregation level and Bandwidth are different from legacy UE.

Proposal 2: Define relaxed RLM/BFD measurement requirement for (e)RedCap UE. For 2Rx (e)RedCap UE, the evaluation period TEvaluate_out_SSB_Relax for legacy UE can be reused. For 1Rx (e)RedCap UE, the evaluation period should be extended based on legacy UE.

Observation 2: For R18 non-redcap UE, only SSB based measurements for less than 5MHz were considered, without taking CSI-RS based measurement into account.

Proposal 3: For (e)RedCap UE, SSB based measurement should be considered and the requirements for legacy UE can be used as baseline. Meanwhile, RAN4 should discuss whether to study CSI-RS based measurement for less than 5MHz in Rel-20.

Proposal 4: For (e)RedCap UE, Study the following issues for less than 5 MHz:

  - The value of T∆ in handover

  - PDCCH transmission parameters for RLM/BFD

  - Time period for time index detection

  - Other aspects are not preclude

References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple.