3GPP TSG-RAN WG4 Meeting #116bis                                        R4-2513917

Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Huawei, HiSilicon

Title: 	Co-existence study for HPUE with maximum output power up to 32dBm

Agenda Item:	7.1.3

Document for:	Approval

Introduction

The new WID on UE RF enhancements for NR FR1 Phase 5 was approved at RAN#109 [1]. The objectives were agreed as below:

The core part of WI: - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations: - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class

This document provides our initial consideration on coexistence study to evaluate the ACLR requirements based on the assumption of max 32dBm output power with a 4x26 dBm PA configuration.

Discussion

Simulation scenarios

Table 1 summarizes the simulation scenarios to be considered. Regarding the example bands in the WI, bands n41, n77/n78, and n79 could be considered as a group with a representative simulation frequency of 3.5 GHz. As for band n104, due to the larger frequency gap with other bands in the WI, thus the simulation frequency is considered as 7GHz. While a number of channel bandwidth would be specified for band n41, n77/n78 and n79, 100 MHz is considered as a representative channel bandwidth.

Table 1 Summary of considered scenario

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 3.5 GHz | Urban macro |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Urban macro |

Proposal 1: The simulation assumptions corresponding to the n41, n77/n78, and n79 remain consistent, with the simulation frequency defined as 3.5 GHz. While for n104, 7GHz is considered as the simulation frequency.

simulation assumptions

Network layout model

Details on urban macro network layout model are listed in table 2 and table 3.

The network layout model for 7GHz aligns with the co-existence simulation assumption provided in TR 38.921[2]. Due to more favorable propagation conditions for n41, tentatively we could consider a larger cell radius for the representative frequency compared to the 7 GHz band.

Table 2 Single operator layout for urban macro

| Parameters |  | 3.5 GHz | 7 GHz |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.6 km/0.45 km | 0.6 km/0.45 km |
| BS antenna height |  | 20 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |

Table 3 Multi operators’ layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator’s layout | coordinated operation (0% Grid Shift) and un-coordinated operation (100% Grid Shift) | RAN4 has long been using un-coordinated operation in below 6 GHz coexistence simulation |

Propagation model

Table 4 lists the propagation model used in this document.

Table 4 Propagation model

| Propagation loss | 3.5 GHz | 7 GHz |
| --- | --- | --- |
| Path loss | TR 38.901 UMa |  |
| LOS probability | TR 38.901 UMa |  |
| Penetration loss | TR 38.901 O2I building penetration loss |  |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |  |

Antenna and beam forming pattern modelling

Table 4 lists the antenna radiation patterns and array antenna parameters for BS, based on TR 38.921 and TR 38.922 [3]. UE antenna is modelled using isotropic antenna pattern.

Table 5 BS Antenna array parameters

| Parameter | Symbol | 3.5 GHz | 7 GHz |
| --- | --- | --- | --- |
| Antenna pattern | Table 7.1.3-1 in TR 38.922 |  |  |
| Front to back ratio | Am | 30 dB | 30 dB |
| Side lobe suppression | SLAv | 30 dB | 30 dB |
| Horizontal HPBW | θ3dB | 90 deg. | 90 deg. |
| Vertical HPBW | θ3dB | 65 deg. | 90 deg. |
| Element peak gain | GE,max | 6.4 dBi | 5.5 dBi |
| Number of element rows in sub-array | Msub | 3 | N/A |
| Vertical element separation in sub-array | dv,sub | 0.7λm | N/A |
| Pre-set sub-array down-tilt | θsubtilt | 3 deg. | N/A |
| Number of radiating elements rows | M | 4 | 16 |
| Number of radiating elements columns | N | 8 | 8 |
| Horizontal element separation | dh | 0.5λm | 0.5λm |
| Vertical element separation | dv | 2.1λm | 0.5λm |
| Electrical down-tilt angle | θetilt | 90-100 deg. | 90-120 deg. |
| Electrical scan angle | φescan | ±60 deg. | ±60 deg. |
| Mechanical downtilt | θmech | 6 deg. | 10 deg. |

Proposal 2: RAN4 shall decide simulation assumptions firstly. Parameters in Table 2~5 could be considered as starting point in coexistence study for evaluating the ACLR requirements with 32dBm maximum output power.

Simulation results

Based on the current specifications and the assumptions above, we set 46dB ACS for BS and 30dB ACLR for UE as the baseline (DeltaACIR=0). The corresponding UL ACIR is about 29.9dB. When we simulate the scenario, different delta-ACIR X values were considered in order to find the reasonable requirements.

The urban macro simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 3.5GHz centre frequency in table 6.

Table 6 The urban macro simulation results of the uplink throughput loss at 3.5GHz centre frequency

| ACIR offset =X [dB] | -1 | 0 | 1 |
| --- | --- | --- | --- |
| Average throughput loss (3.5GHz) | 1.11% | 0.86% | 0.89% |
| 5%-tile throughput loss (3.5GHz) | 8.10% | 5.17% | 4.49% |

Observation 1: The required ACIR for 3.5 GHz is ~30 dB under the assumption of max 32dBm output power for urban macro scenario.

The urban macro simulation results of the uplink throughput loss of the victim BS with different delta-ACIR values are shown at 7GHz centre frequency in table 7.

Table 7 The urban macro simulation results of the uplink throughput loss at 7GHz centre frequency

| ACIR offset =X [dB] | 0 | -1 | -2 | -3 | -4 |
| --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 0.47% | 0.52% | 0.76% | 0.72% | 0.79% |
| 5%-tile throughput loss (7GHz) | 2.70% | 3.41% | 4.56% | 5.67% | 7.20% |

Observation 2: The required ACIR for 7 GHz is ~27 dB under the assumption of a max 32dBm output power for urban macro scenario.

Conclusion

In this contribution, we provide assumptions and preliminary simulation results of coexistence study for evaluating the ACLR requirements, based on the assumption of a 4x26 dBm PA configuration.

The observations and proposals made in this contribution are summarized below:

Proposal 1: The simulation assumptions corresponding to the n41, n77/n78, and n79 remain consistent, with the simulation frequency defined as 3.5 GHz. While for n104, 7GHz is considered as the simulation frequency.

Proposal 2: RAN4 shall decide simulation assumptions firstly. Parameters in Table 2~5 could be considered as starting point in coexistence study for evaluating the ACLR requirements with 32dBm maximum output power.

Observation 1: The required ACIR for 3.5 GHz is ~30 dB under the assumption of max 32dBm output power for urban macro scenario.

Observation 2: The required ACIR for 7 GHz is ~27 dB under the assumption of a max 32dBm output power for urban macro scenario.

- References

[1] RP-252952, New WID: UE RF enhancements for NR FR1, Phase 5, Huawei, HiSilicon, T-Mobile

[2] TR 38.921 v18.0.1. Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz (Release 18)

[3] TR 38.922 v19.1.0. Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz (Release 19)

[4] ITU WP 5D/792. Working document on characteristics of terrestrial component of IMT sharing and compatibility studies in preparation for WRC-27(Annex 4.32 to Working Party 5D Chair’s Report)