# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513930

# Prague, CZ, 13th-17th Oct., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on co-existence for UAV

# Agenda Item:   7.7.3

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WID to enable UAV with higher power than R18 NR_UAV WID was approved [1]. The objective of this WID is:

The core part of WI: - Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected

As the maximum output power of UAV is improved, co-existence issues arise again. In this contribution, we provide our views on the co-existence issue of high power UAV.

- Discussion

The high power UAV UE with up to 33dBm maximum conductive output power will cause more output RF spectrum emissions than the PC3 Aerial UE (UAV) with 23dBm transmit power introduced in Rel-18. The interference between high power UAV network and terrestrial network in synchronized scenarios is non-negligible and should be addressed.

With the introduction of high power UAV transmission, interference between UAV network and TN network exists in both UL transmission and DL transmission. The possible interference scenarios for co-existence and our analysis are listed in Table 1.

Table 1  Interference scenarios for co-existence between UAV network ans TN

| Scenario | Interference type | Analysis |
| --- | --- | --- |
| Scenario 1 UL |  | Interference comes from TN UE and impacts UL transmission of UAV, where the interference level is same as legacy Rel-18 co-existence scenarios. As the output power of UAV is increased up to 33dBm, which is much higher than Rel-18 PC3 UAV, the impact of interference is weakened and legacy BS ACS requirements can be considered to apply for UAV BS. |
| Scenario 2 UL |  | Interference comes from UAV and impacts UL transmission of TN UE, where the interference level is higher than legacy Rel-18 co-existence scenarios. This case can be seen as the worst co-existence scenario, and the ACLR requirements for UAV with both omni-directional antenna and directional antenna should be re-evaluated. |
| Scenario 3 DL |  | Interference comes from UAV BS and impacts DL transmission of TN UE, where the UAV UL signal steers from UAV BS to UAV in the air. Thus, the interference to TN UE is less than other normal TN aggressor BSs. Legacy BS ACLR requirements can be considered to apply for UAV BS. |
| Scenario 4 DL |  | Interference comes from TN BS and impacts DL transmission of UAV, where the TN UL signal steers from TN BS to TN UE on the ground. As the output power of UAV is increased up to 33dBm, which is much higher than Rel-18 PC3 UAV, the impact of interference is weakened, and legacy UE ACS requirements can be considered to apply for UAV. |

As we analyzed above, Scenario 2 where the interference from high power UAV impacts UL transmission of TN UE can be seen as the worst scenario, which should be studied as first priority. The ACLR of high power UAV needs to be re-evaluated. In Scenario 2, if the UAV is equipped with omni-directional antenna, the worst case is that the TN is directly placed underneath the UAV, where the TN BS may suffer the most serious co-existence interference. If the UAV is equipped a directional antenna array, then whether TN placed close to UAV BS or TN placed underneath the UAV is the worst case needs to be evaluated.

Proposal 1: Scenario 2 where interference from high power UAV impacts UL transmission of TN UE is the worst co-existence scenario, which should be studied as first priority.

Proposal 2: In Scenario 2, the ACLR requirements of high power UAV with both omni-directional antenna and directional antenna need to be re-evaluated.

In remaining scenarios, as the interference level is no more than legacy scenarios, we think RAN4 can consider reusing legacy requirements for UAV and UAV BS, which could be treated as second priority.

Proposal 3: The remaining scenarios are second priority, where RAN4 can consider reusing legacy requirements.

For co-existence evaluation, first of all, RAN4 needs to confirm whether analysis-based method or simulation-based method is to be used for UAV co-existence evaluation. Analysis-based method utilizes specific scenario and assumptions to calculate how much ACLR and ACS are needed, or whether legacy requirements are enough for UAV co-existence. This is not accurate enough but can save a lot of works in RAN4. Simulation-based method needs to align simulation models and parameters and spend much time on doing co-existence simulation and aligning simulation results.

Proposal 4: RAN4 to confirm whether analysis-based method or simulation-based method is to be used for UAV co-existence evaluation.

Considering the similar electromagnetic environment in the air, similar propagation characteristics are shared between ATG network and UAV network. As we proposed in another contribution [2], the high power UAV should report its maximum output power just like that for ATG UE. The main differences between ATG UE and UAV are the flying height, flying distance and flying speed. Some co-existence parameters and evaluation methods in ATG network can be referred to, such as TN network layout, TN BS/UE parameters and antenna models, propagation model, etc..

Proposal 5: Some co-existence parameters and evaluation methods in ATG network can be referred to, such as TN network layout, TN BS and UE parameters, propagation model, etc..

Specifically, the antenna model should be discussed for UAV, i.e. whether omni-directional antenna or directional antenna array or both can be used for UAV transmitting and receiving.

Proposal 6: The antenna model should be discussed for UAV, i.e. whether omni-directional antenna or directional antenna array or both can be used for UAV transmitting and receiving.

- Conclusion

Proposal 1: Scenario 2 where interference from high power UAV impacts UL transmission of TN UE is the worst co-existence scenario, which should be studied as first priority.

Proposal 2: In Scenario 2, the ACLR requirements of high power UAV with both omni-directional antenna and directional antenna need to be re-evaluated.

Proposal 3: The remaining scenarios are second priority, where RAN4 can consider reusing legacy requirements.

Proposal 4: RAN4 to confirm whether analysis-based method or simulation-based method is to be used for UAV co-existence evaluation.

Proposal 5: Some co-existence parameters and evaluation methods in ATG network can be referred to, such as TN network layout, TN BS and UE parameters, propagation model, etc..

Proposal 6: The antenna model should be discussed for UAV, i.e. whether omni-directional antenna or directional antenna array or both can be used for UAV transmitting and receiving.

- Reference

- RP-252954 New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), RAN#109 Beijing, China, Sept. 2025,.

- R4-2513931 Discussion on UE RF requirements for UAV, ZTE, RAN4#116bis, Prague, CZ, Oct. 2025.