# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513931

# Prague, CZ, 13th-17th Oct., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on UE RF requirements for UAV

# Agenda Item:   7.7.4

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WID to enable UAV with higher power than R18 NR_UAV WID was approved [1]. The objective of this WID is:

The core part of WI: - Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected

In this contribution, we provide our views on UE RF requirements of high power UAV.

- Discussion

## 2.1 General Aspects

### Definition of high power UAV

RAN4 has defined Aerial UE feature in Rel-18 as:

where PC3 UAV followed this definition and the additional requirement clauses with suffix K were defined. In our understanding, the main difference between UAV specified in Rel-18 and the UAV introduced in this WID is the output power. The high power UAV in this WID should also follow this definition. However, legacy system parameters and RF requirements may not apply for high power UAV, which needs further discussion in RAN4. Therefore, the related requirements of the high power UAV in this WID need to be differentiated from legacy PC3 UAV requirements and some wording on the requirements of legacy PC3 UAV need to be revised in the specification and new requirements for high power UAV will be added if needed.

Proposal 1: The high power UAV in this WID follows the definition on Aerial UE in Rel-18.

Proposal 2: Wording revision is needed to differentiate the requirements for legacy PC3 UAV and the requirements for high power UAV in this WID.

### Operating bands

In Rel-18, PC3 UAV can operate in all the NR operating bands as defined in Table 5.2-1 of TS38.101-1.

In the WID, some example bands are listed, for example, TDD bands n41, n77, n78, n79 and FDD band n3. Operators should declare whether high power UAV can also operate in all the NR operating bands defined in Table 5.2-1 of TS38.101-1.

Proposal 3: It should be declared clearly whether high power UAV operates in all the NR operating bands defined in Table 5.2-1 of TS38.101-1 or only the listed example bands.

### UL MIMO

It is not clear whether UL MIMO is supported for high power UAV in the WID. This should be declared clearly.

Proposal 4: Whether UL MIMO is supported for high power UAV needs to be declared.

## 2.2 UE RF Requirements

### Transmitter power

In Rel-18, the UAV is defined as PC3 only. But in this WID, the conductive maximum output power (MOP) of UAV is up to 33dBm, which does not match the MOP of any power class in current specifications. To avoid the large workload to introduce a new power class that only apples to UAV at current stage, we suggest to follow the method of ATG as shown below, i.e., to introduce the new capability for UAV to report the rated maximum output power at maximum modulation order and full PRB configurations and its capability report granularity as 1dB. The lower limit of conductive MOP of high power UAV could be 23dBm, and the upper limit of conductive MOP of high power UAV could be 33dBm.

Proposal 5: Introduce the new capability for high power UAV to indicate the rated maximum output power.

If the rated maximum output power is reported by UAV, no MPR or A-MPR requirements are needed since the UAV can declare a lower power if power back-off is needed. Then the UAV can set its configured maximum output power based on NW indication and UAV capability.

Proposal 6: If the rated maximum output power is reported by UAV, no MPR or A-MPR requirements are needed.

### Output dynamic range

Output dynamic range requirements that need to be considered when introducing high power UAV and our views are listed in Table 1.

Table 1  Output dynamic range requirements to introduce high power UAV

| Requirements | ZTE’s views |
| --- | --- |
| Minimum output power | RAN4 needs to evaluate the minimum output power for high power UAV equipped with omni-directional antenna and directional antenna array, considering different center frequency, pass loss and antenna gain. |
| Transmit OFF power | Legacy requirements can apply for high power UAV. |
| Transmit ON/OFF time mask | Legacy requirements can apply for high power UAV. |
| Power control | Legacy requirements can apply for high power UAV. |

Proposal 7: RAN4 needs to evaluate the minimum output power for high power UAV equipped with omni-directional antenna and directional antenna array, considering different center frequency, pass loss and antenna gain.

### Transmit signal quality

Transmit signal quality requirements that need to be considered when introducing high power UAV and our views are listed in Table 2.

Table 2  Transmit signal quality requirements to introduce high power UAV

| Requirements | ZTE’s views |
| --- | --- |
| Frequency error | Considering that the speed of high power UAV is usually not that high and even sometimes it is static, legacy frequency error can be applied. |
| Transmit modulation quality | Legacy requirements should also apply for high power UAV at each transmit antenna connector for UAV with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UAV with antenna array. |

### Output RF spectrum emissions

Output RF spectrum emission requirements that need to be considered when introducing high power UAV and our views are listed in Table 3.

Table 3  Output RF spectrum emission requirements to introduce high power UAV

| Requirements | ZTE’s views |
| --- | --- |
| Occupied bandwidth | Legacy requirements can apply for high power UAV. |
| SEM requirements | It is observed that current spec specified the same SEM levels for all UEs up to PC1, i.e. 31dBm MOP, and scaling factor equal to the actual transmission power minus 31dBm is used for more than 31dBm transmit power. The discussion on SEM can be hold on until ACLR is concluded. |
| ACLR requirements | RAN4 needs to evaluate ACLR for high power UAV based on co-existence discussion. |
| Spurious emissions | Legacy requirements can apply for high power UAV. |
| Spurious emissions for UE co-existence | Legacy requirements can apply for high power UAV. |
| Transmit intermodulation | It is observed that no transmitted intermodulation requirements were designed for either Rel-18 UAV or ATG UE. Terrestrial interference scenario is different from scenarios over the air. This requirement is not needed for high power UAV either. |

Proposal 8: RAN4 needs to evaluate ACLR for high power UAV based on co-existence discussion. The discussion on SEM can be hold on until ACLR is concluded.

Proposal 9: Transmit intermodulation is not needed for high power UAV.

### Receiver characteristics

It is not clear yet whether omni-directional antenna or directional antenna is used for UAV receiving, which may impact the REFSENS of UAV. Besides, for UAV operating in FDD bands, legacy duplexer is used for 23dBm transmit power. When the transit power is improved to up to 33dBm, the self-interference from duplexer leakage needs to be further studied and REFSENS may be impacted. Except REFSENS, other Rx requirements may be reused for high power UAV.

Proposal 10: Whether omni-directional antenna or directional antenna is used for UAV receiving should be declared first. Then REFSENS of the high power UAV needs to be discussed.

- Conclusion

Proposal 1: The high power UAV in this WID follows the definition on Aerial UE in Rel-18.

Proposal 2: Wording revision is needed to differentiate the requirements for legacy PC3 UAV and the requirements for high power UAV in this WID.

Proposal 3: It should be declared clearly whether high power UAV operates in all the NR operating bands defined in Table 5.2-1 of TS38.101-1 or only the listed example bands.

Proposal 4: Whether UL MIMO is supported for high power UAV needs to be declared.

Proposal 5: Introduce the new capability for high power UAV to indicate the rated maximum output power.

Proposal 6: If the rated maximum output power is reported by UAV, no MPR or A-MPR requirements are needed.

Proposal 7: RAN4 needs to evaluate the minimum output power for high power UAV equipped with omni-directional antenna and directional antenna array, considering different center frequency, pass loss and antenna gain.

Proposal 8: RAN4 needs to evaluate ACLR for high power UAV based on co-existence discussion. The discussion on SEM can be hold on until ACLR is concluded.

Proposal 9: Transmit intermodulation is not needed for high power UAV.

Proposal 10: Whether omni-directional antenna or directional antenna is used for UAV receiving should be declared first. Then REFSENS of the high power UAV needs to be discussed.

- Reference

- RP-252954 New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) RAN#109, Sept. 2025, Beijing, China.