3GPP TSG-RAN4 Meeting #116bis	R4-2513935

# Prague, CZ, 13th Oct. 2025 - 17th Oct. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-1 on 3Tx MIMO Rx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The RF requirements for 3Tx feature were introduced in the big CR R4-2511773. However, this big CR only includes the changes for Tx requirements since the discussion mainly focus on the Tx parts. Due to the descriptions shall includes two, three and four antenna connectors, the related Rx requirements need to be updated to keep alignment with Tx parts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the descriptions with Tx parts to include 3 transmitter antenna connectors in receiver characteristics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3Tx requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3D, 7.4D, 7.5D, 7.6D, 7.7D, 7.8D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

7.3D	Reference sensitivity for UL MIMO

For UE with multiple transmitter antenna connectors up to a maximum of four in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.3 shall be met with the UL MIMO configurations described in clause 6.2D.1 and clause 6.2F.1D for shared spectrum access operation, and the reference measurement channels as specified in Annex A.2.2 for CP-OFDM waveforms shall apply. For UL MIMO, the parameter PUMAX is the total transmitter power over all transmit antenna connectors.

# << Next change >>

7.4D	Maximum input level for UL MIMO

For UE with multiple transmitter antenna connectors up to a maximum of four in closed-loop spatial multiplexing, the minimum requirements specified in clause 7.4 shall be met with the UL MIMO configurations described in clause 6.2D.1 and clause 6.2F.1D for shared spectrum access operation. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over all transmit antenna connectors.

# << Next change >>

7.5D	Adjacent channel selectivity for UL MIMO

For UE(s) with multiple transmitter antenna connectors up to a maximum of four in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.5 shall be met with the UL MIMO configurations described in clause 6.2D.1 and clause 6.2F.1D for shared spectrum access operation. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over all transmit antenna connectors.

# << Next change >>

7.6D	Blocking characteristics for UL MIMO

For UE with multiple transmitter antenna connectors up to a maximum of four in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.6 shall be met with the UL MIMO configurations described in clause 6.2D.1 and in clause 6.2F.1D for shared spectrum access operation. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over all transmit antenna connectors.

# << Next change >>

7.7D	Spurious response for UL MIMO

For UE with multiple transmitter antenna connectors up to a maximum of four in closed-loop spatial multiplexing scheme, the minimum requirements specified in clause 7.7 shall be met with the UL MIMO configurations described in clause 6.2D.1 and in clause 6.2F.1D for shared spectrum access operation. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over all transmit antenna connectors.

# << Next change >>

7.8D	Intermodulation characteristics for UL MIMO

For UE(s) with multiple transmitter antenna connectors up to a maximum of four in closed-loop spatial multiplexing scheme, the minimum requirements in clause 7.8 shall be met with the UL MIMO configurations described in clause 6.2D.1 and in clause 6.2F.1D for shared spectrum access operation. For UL MIMO, the parameter PCMAX_L is defined as the total transmitter power over all transmit antenna connectors.

# << End of change >>