3GPP TSG-RAN WG4 Meeting #116bis				                   R4-2513937

Prague, Czech, Oct. 13-17, 2025

Source:	Sony

Title:	Coexistence assumptions for outdoor AIoT system

Agenda item:	7.11.3

Document for:	Approval

# Background

The SI of supporting outdoor ambient IoT in Rel-20 has been approved [1], and the RAN4 work focuses on the coexistence aspect

In this contribution, we provide our initial view on the coexistence scenario to be considered in the RAN4 study.

# Deployment scenarios

The target of Rel-20 AIoT SI is to study the AIoT system design that can support outdoor deployment with active devices. In particular, deployment scenario 4 with topology 1 with no external carrier wave is the focus, where the corresponding scenario is illustrated below:

Fig. 1. Illustration of topology 1 and deployment scenario 4 in 38.848.

More specifically, the SI targets A-IoT BS readers are deployed on the same sites as existing outdoor NR macro-BSs. In our view, this is crucial for a potentially successful commercial deployment in an outdoor usage scenario, and thus, the coexistence study in RAN4 should cover such a scenario.

Observation 1: The SI targets A-IoT BS readers that are deployed on the same sites as existing outdoor NR macro-BSs, which is an essential aspect to ensure a potentially commercially successful design for outdoor scenarios.

Therefore, to be in line with the objective of the SI, it is proposed that the RAN4 coexistence study focuses on a macro cell layout where the AIoT reader is collocated with the NR BS. In 5G NR, the inter-site distance (ISD) for urban macro cells is assumed to be up to 1732m as in TR37.910; thus, it is proposed to use ISD = 1732m for the coexistence study as well. The number of cells can remain the same as in legacy coexistence studies, including Rel-19 AIoT, with 19 cells in total.

Proposal 1: For the coexistence study, RAN4 adopts the macro cell deployment scenario with 19 sites and ISD = 1732m, collocated between the NR BS and the AIoT reader.

Meanwhile, it can be expected that the device can be distributed both indoors and outdoors. For example, in the inventory use cases, the device should be able to connect to the network both when it is outdoors on the road and also when it reaches inside an inventory house. Therefore, it is proposed to consider AIoT devices in both indoor and outdoor settings for the coexistence study, with 80% outdoor and 20% indoor, which is similar to the assumption adopted for the NR coexistence study and can be taken as a starting point.

Proposal 2:  Considering AIoT devices can be both indoors and outdoors in the coexistence study, and adopt 80% outdoor and 20 % indoors as a starting point.

For the coexistence study cases, the following cases need to be considered under the coexistence study of outdoor AIoT in Rel-20:

| case | direction | Aggressor | victim | notes |
| --- | --- | --- | --- | --- |
| 1 | DL (R2D) | A-IoT reader | NR UE | Derive the AIoT reader ACLR, which can be down-prioritized if it is assumed to be similar to NR BS |
| 2 | DL (R2D) | NR BS | AIoT device | Derive the AIoT device ACS |
| 3 | UL (D2R) | AIoT device | NR BS | Derive the AIoT device ACLR |
| 4 | UL (D2R) | NR UE | A-IoT reader | Derive the AIoT reader ACS |

Cases 1 and 4 are to be studied to derive the required ACLR and ACS requirements of AIoT readers, respectively. However, if we assume the AIoT reader would perform similarly to a NR BS in terms of out of band emission level and it is also transmits with an OFDM waveform (e.g., OOK waveform generated by OFDM modulator), this case can be down prioritized since it will be basically the same as intra NR system coexistence and the coexistence can be guaranteed by defining the SEM and ACLR requirements for AIoT reader to be the same as NR BS.

Observation 2: If the outdoor AIoT reader is assumed to have a similar out-of-band emission level as the NR BS, there is no coexistence issue in the case where the AIoT reader is an aggressor to the NR UE.

In Rel-19, no filtering capability is assumed for AIoT device 1 in both DL and UL directions, and thus, no conclusion was drawn on device ACLR/ACS based on the coexistence study. However, for active devices, filters are considered (see Fig. 2), and a similar coexistence study is needed to derive the ACLR/ACS of the active AIoT devices, similar to legacy IoT devices. Case 2(NR BS  AIoT device) and case 3 (AIoT device  NR BS ) will be needed to derive the ACS and ACLR of AIoT devices, respectively.

Fig. 2. The schematic of AIoT device 2b based on the Rel-19 AIoT study.

Observation 3: Similar to legacy IoT devices, the ACS and ACLR requirements of Rel-19 AIoT devices need to be derived based on a coexistence study.

Proposal 3: Consider the following cases in the coexistence study for Rel-20 outdoor AIoT study:

| case | direction | Aggressor | victim | notes |
| --- | --- | --- | --- | --- |
| 1 | DL (R2D) | A-IoT reader | NR UE | Derive AIoT reader ACLR, can be down prioritized if it is assumed to be the similar as NR BS |
| 2 | DL (R2D) | NR BS | AIoT device | Derive AIoT device ACS |
| 3 | UL (D2R) | AIoT device | NR BS | Derive AIoT device ACLR |
| 4 | UL (D2R) | NR UE | A-IoT reader | Derive AIoT reader ACS |

# AIoT device assumption

In this study, two outdoor device types are considered, where device C has an output power of -3 to 5 dBm, and device 2B has an output power of -20 to -10 dBm.

To fulfill the coverage of co-site deployment with NR BS, it can be intuitively understood that it will be very challenging to operate with an output power below 0 dBm while maintaining a reasonable data rate and spectral efficiency. In addition, from the coexistence perspective, higher output power would be a more challenging case than low output power scenarios. Therefore, to ensure the system design of outdoor AIoT and reduce the overall scope of the coexistence study in RAN4, it is proposed that RAN4 focus on device type C with output power above 0 dBm in this study.

Proposal 4: To ensure the system design of outdoor AIoT and reduce the overall scope of the coexistence study in RAN4, it is proposed that RAN4 focus on device type C with output power above 0 dBm in this study.

Additionally, CFO performance of AIoT device is another factor that may impact the coexistence performance of the AIoT system with NR. Currently, RAN1 assumes:

|  | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| SFO after clock sync/calibration | 10^2 (O), 10^3 (M) ppm | [10] ppm |
| Initial CFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| CFO after clock sync/calibration | 100 (M), 200 (O), 50 (O) ppm | [10] ppm |

Considering the extremely large CFO that is currently assumed in device 2b, which is about 100 ppm even after calibration, the frequency shift due to the CFO would be up to 90 kHz at 900 MHz, which is quite significant considering the transmission BW of D2R is assumed to be very small, like 15 kHz. Therefore, this aspect needs to be considered when determining the feasibility of device types and deriving proper requirements for AIoT devices in Rel-20. Therefore, it is proposed to consider the CFO after the clock calibration is taken into account in the coexistence study.

Proposal 5: It is proposed to consider the CFO after the clock calibration is taken into account in the coexistence study

In addition, the major difference between device type C/2b and device 1 in Rel-19 is that these are active devices, which can generate the signal and waveform by itself. In this case, depending on the assumptions about power consumption and storage capability, it might even be feasible for the device to generate an OFDM-based waveform, similar to the reader, which can significantly improve spectrum efficiency compared to a raw MSK signal.

Proposal 6: RAN4 investigates whether it is feasible for AIoT devices to adopt OFDM transmission (e.g., single-tone OFDM transmission) in D2R, with possible OOK generated by the OFDM modulator.

Pending the waveform of the AIoT device in D2R direction, RAN4 can further determine the power leakage model in the coexistence simulation.

For the DL filter selectivity model, it is proposed to consider a flat model with a constant attenuation factor as the starting point to perform the coexistence simulation.

Proposal 7: RAN4 determines the power leakage model of AIoT devices based on the waveform in the D2R direction, while adopting a flat model for the R2D interference attenuation.

# Conclusion

In this contribution, we share our intial views on some key assumptions for co-existence study for Rel-20 AIoT SI, where the following observations and proposals are given:

Observation 1: The SI targets A-IoT BS readers that are deployed on the same sites as existing outdoor NR macro-BSs, which is an essential aspect to ensure a potentially commercially successful design for outdoor scenarios.

Observation 2: If the outdoor AIoT reader is assumed to have a similar out-of-band emission level as the NR BS, there is no coexistence issue in the case where the AIoT reader is an aggressor to the NR UE.

Observation 3: Similar to legacy IoT devices, the ACS and ACLR requirements of Rel-19 AIoT devices need to be derived based on a coexistence study.

Proposal 1: For the coexistence study, RAN4 adopts the macro cell deployment scenario with 19 sites and ISD = 1732m, collocated between the NR BS and the AIoT reader.

Proposal 2:  Considering AIoT devices can be both indoors and outdoors in the coexistence study, and adopt 80% outdoor and 20 % indoors as a starting point.

Proposal 3: Consider the following cases in the coexistence study for Rel-20 outdoor AIoT study:

| case | direction | Aggressor | victim | notes |
| --- | --- | --- | --- | --- |
| 1 | DL (R2D) | A-IoT reader | NR UE | Derive AIoT reader ACLR, can be down prioritized if it is assumed to be the similar as NR BS |
| 2 | DL (R2D) | NR BS | AIoT device | Derive AIoT device ACS |
| 3 | UL (D2R) | AIoT device | NR BS | Derive AIoT device ACLR |
| 4 | UL (D2R) | NR UE | A-IoT reader | Derive AIoT reader ACS |

Proposal 4: To ensure the system design of outdoor AIoT and reduce the overall scope of the coexistence study in RAN4, it is proposed that RAN4 focus on device type C with output power above 0 dBm in this study.

Proposal 5: It is proposed to consider the CFO after the clock calibration is taken into account in the coexistence study

Proposal 6: RAN4 investigates whether it is feasible for AIoT devices to adopt OFDM transmission (e.g., single-tone OFDM transmission) in D2R, with possible OOK generated by the OFDM modulator.

Proposal 7: RAN4 determines the power leakage model of AIoT devices based on the waveform in the D2R direction, while adopting a flat model for the R2D interference attenuation.

# References

- RP-252964	Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices	LG Electronics Inc.