3GPP TSG-RAN WG4 Meeting #116bis	R4-2513955

Prague, Czech, 13th – 17th October 2025

Agenda Item:	7.5.2

Source:	Ericsson

Title:	On general issues for performance enhancement phase 6

Document for:	Discussion

# 1	Introduction

In RAN#109 meeting, a new WID on NR demodulation performance: Phase 6 was approved [1]. According to the WID, following are the objectives for UE part:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered. - Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

In this contribution, we provided our views on the general issues that need to be considered before going to the details of simulation assumptions for each objective.

# 2	CDL model for UE demodulation

As WID mentioned, there are some remaining issues from Rel-19 on model generalization, PMI test bias by one fixed model and impact checking by latest version of 38.901. We think it might be reasonable to consider these general issues before corresponding requirement definition.

As for eType-II PMI requirements, companies’ simulation results with rCDL-C1 are not aligned in Rel-19, it would be necessary to get alignment at first and then consider adding more angle variants to have robust requirements. Another potential work on PMI part is to derive new CDL model with smaller delay spread, such as rCDL-A with 30 ns rms delay, to compare performance.

According to potential effort and complexity, we suggest RAN4 discuss these issues together with WI objectives as following order:

- New version 38.901 impact

- Model generalization.

- New rCDL model derivation if necessary.

- Simulation results alignment with new derived rCDL model for SU-MIMO PDSCH and introduce requirements.

- Simulation results alignment with new derived rCDL model for eType-II PMI

- Study on methodology of adding angular variants for PMI tests.

Proposal 1	RAN4 consider following discussion order for Rel-20 CDL model WI

- New version 38.901 impact

- Model generalization.

- New rCDL model derivation if necessary.

- Simulation results alignment with new derived rCDL model for SU-MIMO PDSCH and introduce requirements.

- Simulation results alignment with new derived rCDL model for eType-II PMI

- Study on methodology of adding angular variants for PMI tests.

# 3	Interference mitigation with 6Rx

In this section, we discuss the general issues for introducing PDSCH demodulation requirements for the scenario of inter-cell interference and intra-cell inter-user interference with 6Rx reception.

Following the similar approach of introducing Rel-17 interference mitigation requirements, we propose to discuss the interference model for the two scenarios. For PDSCH requirements with inter-cell interference, following aspects are proposed to start with:

- Number of interfering cell

- Interference profile

  - Whether to reuse the DIP method and INR level

- Transmission rank

- Time/frequency offset

- SSB configuration

Proposal 2: Propose RAN4 to first discuss and reach agreements on the following aspects for inter-cell interference scenario

- Number of interfering cell

- Interference profile

  - Whether to reuse the DIP method and INR level

- Transmission rank

- Time/frequency offset

- SSB configuration

As for the scenario of intra-cell inter-user interference, it is expected to introduce PDSCH demodulation requirement. Similarly, we propose to discuss the interference model first with the following aspects to be considered:

- Number of paired UE

- Rank for target and interference

- Precoder codebook type

- PMI selection and precoding

- MCS for interference

- DMRS configurations

Proposal 3: Propose RAN4 to first discuss and reach agreements on the following aspects for intra-cell inter-user interference scenario

- Number of paired UE

- Rank for target and interference

- Codebook type

- PMI selection and precoding

- MCS for interference

- DMRS configurations

After reaching agreements on the interference model and profile and before RAN4 going to the detailed parameter assumptions, it is also suggested to discuss the following aspects to have a whole picture of the test case structure:

- Whether to define one case for each scenario to cover two different UE types

- Whether to differentiate UE types when discussing the rank combination for intra-cell inter-user interference scenario

- And more

Test applicability rule discussion can be postponed and further discussed after having agreements on test case design.

# 4	Summary

In this contribution, we shared our views on general issues for introducing demodulation requirements for SCM and interference mitigation with 6Rx reception.

Proposal 1	RAN4 consider following discussion order for Rel-20 CDL model WI

- New version 38.901 impact

- Model generalization.

- New rCDL model derivation if necessary.

- Simulation results alignment with new derived rCDL model for SU-MIMO PDSCH and introduce requirements.

- Simulation results alignment with new derived rCDL model for eType-II PMI

- Study on methodology of adding angular variants for PMI tests.

Proposal 2: Propose RAN4 to first discuss and reach agreements on the following aspects for inter-cell interference scenario

- Number of interfering cell

- Interference profile

  - Whether to reuse the DIP method and INR level

- Transmission rank

- Time/frequency offset

- SSB configuration

Proposal 3: Propose RAN4 to first discuss and reach agreements on the following aspects for intra-cell inter-user interference scenario

- Number of paired UE

- Rank for target and interference

- Codebook type

- PMI selection and precoding

- MCS for interference

- DMRS configurations

# References

[1]	RP-252957: WID on NR demodulation performance: Phase 6, China Telecom, BT, RAN#109, Beijing, China

[2]	TS38.101-4

[3]	TS38.214