3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513967

Prague, CZ, October 13th – 17th 2025

Agenda Item:	6.13.2.2

Source:	Ericsson

Title:	Impact on SBFD BS RF TX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on conformance testing of BS RF TX requirements for SBFD.

Discussion on conformance testing of BS RF RX requirements for SBFD and new requirements for SBFD are detailed in separate submissions [1-2].

# 2	Discussion

Conformance testing for both SBFD and non-SBFD slots

It is important to ensure that conformance testing is performed for both SBFD and non-SBFD slot. Even for requirements that remain unchanged, the transmitter and receiver operate under different conditions in SBFD, where simultaneous uplink and downlink transmission leads to different hardware behaviour. Therefore, separate conformance testing is necessary for both cases to capture any deviations in RF performance.

- The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

Testing applicability for supported SBFD configuration

##### Issue 5-1-4: Testing applicability for supported SBFD configuration Way forward: - Further study on the following proposals on test configuration: - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. For the restriction on guardband size of SBFD - Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations.

The background of this issue is that, if the SSB symbol is overlapped with SBFD symbol, further study is required to define appropriate conformance test configurations. In our view, conformance testing is necessary for both DU/UD and DUD subband configurations even if the SSB symbol is overlapped with SBFD symbol, as different analog filters may be used depending on the specific subband configuration. Moreover, SSB symbol is not part of standard conformance test models.

- SSB symbol is not part of standard conformance test models.

- Different analog filters may be used for DU/UD and DUD subband configurations.

Conformance testing is necessary for both DU/UD and DUD subband configurations even if the SSB symbol is overlapped with SBFD symbol.

RF channels to be tested for SBFD operation

For conformance testing of SBFD slots, since the SBFD-capable BS rely on analogue components, e.g., subband-specific analog filters, or interference cancellation circuits, which are frequency-selective. As a result, it becomes necessary to test the performance of these implementations not only in a single frequency location, but potentially across multiple points in the band, which means that conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are required.

For conformance testing of SBFD slots, since the SBFD-capable BS rely on analog components, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are required.

Receiver status during conformance testing of transmitter

When testing both conducted and OTA transmitter requirements for an SBFD-capable BS, the receiver should always be turned on. This ensures that the testing accurately reflects real-world operational conditions. The receiver’s presence during testing is critical, as it accounts for potential interactions between the transmitter and receiver, such as signal interference, leakage, and system stability. Additionally, keeping the receiver on during testing helps to simulate normal operating conditions, where the receiver continuously monitors and reacts to the transmitted signals. This approach ensures that the SBFD-capable BS is thoroughly evaluated for both transmitter and receiver performance under realistic scenarios.

When testing both conducted and OTA transmitter requirements for an SBFD-capable BS, the receiver should always be turned on, to ensure the transmitter is evaluated under realistic scenarios.

Test model

In RAN4#113, it was agreed to reuse the existing FRCs for conformance testing of the SBFD UL subband. While reusing the existing FRCs for UL subbands is a logical starting point, DL test models must be designed for scalability. Current DL test models do not consider flexible UL subband configurations, which are essential for SBFD operation. Therefore, a scalable test model for the DL subband, which is parameterized by the UL subband bandwidth, UL subband center frequency or its relative offset from the DL, etc., is needed to accurately evaluate SBFD performance under scalable conditions.

- The test model for SBFD DL subbband needs to be scalable.

- New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

In addition, an important technical concern is the impact of scalable test models on signal statistics. As DL subbands are scaled (e.g., by varying bandwidth or power), their signal statistics may change, potentially affecting the PA testing. To ensure accurate and comprehensive testing, quantitative assessment of signal statistics variations under different subband configurations/DL subband sizes and power levels.

- Signal statistics may change for SBFD test models and need to be investigated. The impact of DL subband on signal statistics needs to be analyzed.

RAN4 use the TP below as a starting point for discussion on SBFD test model.

---------------------------------------------- Start of Text Proposal-----------------------------------------------------------

#### 4.9.2.2	FR1 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-1 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For FR1-TDD without NB-IoT operation in NR in-band, test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [19].

Table 4.9.2.2-1: Configurations of TDD for BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Common physical channel parameters for all NR FR1 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 and table 4.9.2.2-4 for PDSCH. Specific physical channel parameters for NR FR1 test models are described in clauses 4.9.2.2.1 to 4.9.2.2.8.

For DU:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For UD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For DUD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL,1 – 3 + NRB,SBFD,DL,2 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| Mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | pos2 |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 1 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

For DU:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

For UD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| Number of PRBs | 3 |

For DUD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |
| PDSCH nRNTI=3 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

##### 4.9.2.2.1	FR1 test model 1.1 (NR-FR1-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Transmit ON/OFF power

-	TAE

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

-	Receiver spurious emissions

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM1.1 are defined in table 4.9.2.2.1-1.

For DU:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

For UD:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 + NRB,SBFD,UL + NRB,SBFD,GB |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 + NRB,SBFD,UL + NRB,SBFD,GB |

For DUD:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| # of PRBs PDSCH nRNTI=3 | NRB,SBFD,DL,2 |
| Modulation PDSCH nRNTI=3 | QPSK |
| Starting RB location of PDSCH nRNTI=3 | NRB,SBFD,DL,1 + NRB,SBFD,UL + 2 * NRB,SBFD,GB |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 + NRB,SBFD,UL + NRB,SBFD,GB |

------------------------------------- End of Text Proposal--------------------------------------------------------------

Test signal

### Issue 13: BS conformance testing - Test signal - Way forward - FFS in next meeting - define the test signal for SBFD conformance testing in the following way: - Table 4.7.2-1: Signal to be used to build NR SBFD TCs Operating Band characteristics; Band n39; Other FR1 bands. TC signal; BWchannel; 20 MHz (Note 1); 40 MHz (Note 1). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing. NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used.. - Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O Operating band characteristics; FDL_high – FDL_low ≤ 3250 MHz. TC signal; BWchannel; 100 MHz (Note 1, Note 2). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing declared per operating band (D.7). NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used..

In the approved WF [3] in RAN4#116, for BS conformance testing, the test signal requires FFS. In RAN4#115 and RAN4#116, it was agreed that all declared channel bandwidths should be tested. For configuration-limited type of SBFD BS, all subband configurations should be tested, and for configuration-flexible type of SBFD BS, only the worst cases should be tested. Based on this, the test signal for SBFD conformance testing is defined as shown in Table 1 for both FR1 and FR2-1.

Table 1: Signal to be used to build NR SBFD TCs

| SBFD BS type |  | Configuration-limited | Configuration-flexible |  |
| --- | --- | --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |  |  |
|  | Subcarrier spacing | 30 kHz for FR1 60 kHz for FR2-1 |  |  |
|  | Subband configuration | All declared subband configurations | DUD | DU/UD |
|  |  |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2. | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL. |

RAN4 adopt the following table for test signals to be used to build NR SBFD TCs.

Table 1: Signal to be used to build NR SBFD TCs

| SBFD BS type |  | Configuration-limited | Configuration-flexible |  |
| --- | --- | --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |  |  |
|  | Subcarrier spacing | 30 kHz for FR1 60 kHz for FR2-1 |  |  |
|  | Subband configuration | All declared subband configurations | DUD | DU/UD |
|  |  |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2. | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL. |

# Conclusion

In the previous sections we made the following observations:

Observation 1	The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Observation 2	SSB symbol is not part of standard conformance test models.

Observation 3	Different analog filters may be used for DU/UD and DUD subband configurations.

Observation 4	The test model for SBFD DL subbband needs to be scalable.

Observation 5	New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

Observation 6	Signal statistics may change for SBFD test models and need to be investigated. The impact of DL subband on signal statistics needs to be analyzed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

Proposal 2	Conformance testing is necessary for both DU/UD and DUD subband configurations even if the SSB symbol is overlapped with SBFD symbol.

Proposal 3	For conformance testing of SBFD slots, since the SBFD-capable BS rely on analog components, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are required.

Proposal 4	When testing both conducted and OTA transmitter requirements for an SBFD-capable BS, the receiver should always be turned on, to ensure the transmitter is evaluated under realistic scenarios.

Proposal 5	RAN4 use the TP below as a starting point for discussion on SBFD test model.

---------------------------------------------- Start of Text Proposal-----------------------------------------------------------

#### 4.9.2.2	FR1 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-1 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For FR1-TDD without NB-IoT operation in NR in-band, test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon as defined in TS 38.331 [19].

Table 4.9.2.2-1: Configurations of TDD for BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Common physical channel parameters for all NR FR1 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 and table 4.9.2.2-4 for PDSCH. Specific physical channel parameters for NR FR1 test models are described in clauses 4.9.2.2.1 to 4.9.2.2.8.

For DU:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For UD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For DUD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL,1 – 3 + NRB,SBFD,DL,2 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| Mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | pos2 |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 1 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

For DU:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

For UD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| Number of PRBs | 3 |

For DUD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |
| PDSCH nRNTI=3 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

##### 4.9.2.2.1	FR1 test model 1.1 (NR-FR1-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Transmit ON/OFF power

-	TAE

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

-	Receiver spurious emissions

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM1.1 are defined in table 4.9.2.2.1-1.

For DU:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

For UD:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 + NRB,SBFD,UL + NRB,SBFD,GB |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 + NRB,SBFD,UL + NRB,SBFD,GB |

For DUD:

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| # of PRBs PDSCH nRNTI=3 | NRB,SBFD,DL,2 |
| Modulation PDSCH nRNTI=3 | QPSK |
| Starting RB location of PDSCH nRNTI=3 | NRB,SBFD,DL,1 + NRB,SBFD,UL + 2 * NRB,SBFD,GB |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 + NRB,SBFD,UL + NRB,SBFD,GB |

------------------------------------- End of Text Proposal--------------------------------------------------------------

Proposal 6	RAN4 adopt the following table for test signals to be used to build NR SBFD TCs.

Table 1: Signal to be used to build NR SBFD TCs

| SBFD BS type |  | Configuration-limited | Configuration-flexible |  |
| --- | --- | --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |  |  |
|  | Subcarrier spacing | 30 kHz for FR1 60 kHz for FR2-1 |  |  |
|  | Subband configuration | All declared subband configurations | DUD | DU/UD |
|  |  |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2. | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL. |

# References

R4-2513968, Impact on SBFD BS RF RX requirements, Ericsson

R4-2513966, New SBFD BS RF requirements, Ericsson

R4-2512680, Way Forward for [116][307] NR_duplex_evo_BSRF, Huawei