3GPP TSG-RAN WG4 Meeting #116bis	R4-2513979

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.16.4

Source:	Ericsson

Title:	UE demodulation requirements for LP-WUS/WUR

Document for:	Discussion

# 1	Introduction

RAN4#116 agreed with the way forward on LP-WUS demodulation requirements [1]. This contribution discusses the open issues on the UE demodulation requirements for LP-WUS/WUR.

# 2	Discussion

## 2.1	Scope of requirements

False alarm rate (FAR) for wake-up signal reception

RAN4 agreed to specify the wake-up signal reception performance requirements, where RAN4 has agreed to use missed detection rate (MDR) as a test metric. RAN4 also discussed whether to use false alarm rate (FAR) as an additional test metric.

Issue 2-1-2: Whether to introduce FAR test Proposals - Option 1: Yes - Option 2: No Issue 2-1-3: Test metric for FAR test (if introduced) Proposals - Option 1: 1% FAR - Option 2: 0.1% FAR

False alarm is the case where the receiver incorrectly detects a signal when nothing is transmitted. Higher false alarm rate (FAR) of the wake-up signals wakes up the main receiver (MR) unnecessarily and results in increasing the UE power consumption.

RAN4 has the FAR requirements for PUCCH and PRACH, where it is called ‘DTX to ACK probability requirements’ and ‘PRACH false alarm probability’, respectively. According to TS 38.104, the false alarm rates shall not exceed 1% and 0.1% for PUCCH and PRACH, respectively. It is important to ensure minimizing the false alarm rates of PUCCH and PRACH, otherwise gNB may allocate unnecessary DL resources, e.g., random access response.

Although the false alarm of DL wake-up signal reception could increase the UE power consumption, it does not increase the UL interference. We think it is sufficient to define missed detection rate requirements for wake-up signals, because it does not affect the network performance.

Proposal 1: RAN4 does not specify the Wake-up signal demodulation requirements with false alarm rate (FAR).

## 2.2	Test parameters

RAN4 agreed to define the wake-up signal missed detection requirements, but RAN4 has not decided the detailed test parameters. Table 1 summarizes the current parameter options for the wake-up signal reception requirements.

Table 1	Simulation assumption for the wake-up signal reception requirements.

| Parameters | Mwus=1 | Mwus=4 |
| --- | --- | --- |
| MWUS | 1 | 4 |
| Wake-up information length (K) | 2 bits | 5 bits |
| Number of sequences (Nseq) | 1 | 1 |
| Number of root sequence numbers (Nroot) | 1 | 1 |
| Number of bits after line coding (G0 = Nos x MWUS, in TS 38.212 7.4.3) | Option 1: 24 bits (1*24) Option 2: 32 bits (1*32) | Option 1: 64 bits (4*16) |
| Code rate (K/G0) | TBD | TBD |
| Reception time offset | 5us | 1us |
| Reception frequency offset | 10ppm for OOK receiver 5ppm for OFDM receiver | 10ppm for OOK receiver 5ppm for OFDM receiver |
| Channel model | Option 1: TDLA30-10 (FR1 and FR2) Option 2: TDLC300-100 (FR1 only) | Option 1: TDLA30-10 Option 2: TDLC300-100 |
| Frequency range and SCS | FR1: SCS=15kHz FR1: SCS=30kHz FR2 SCS=120kHz | FR1 SCS=15kHz FR1 SCS=30kHz |
| Antenna configuration | 1x1 | 1x1 |
| LP-SS | Configured | Configured |

Table 2	Summary of WUS reception simulation results.

| SCS | Mwus | Information bits | Number of bits after Manchester coding | Number of OFDM symbols | Channel model | SNR to achieve 1% missed detection rate (MDR) |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 1 | 2 | 24 | 24 | TDLA30-10 | -7.4 |
|  |  |  |  |  | TDLC300-100 | -10.2 |
|  |  |  | 32 | 32 | TDLA30-10 | -8.2 |
|  |  |  |  |  | TDLC300-100 | -11.0 |
|  | 4 | 5 | 64 | 16 | TDLA30-10 | 3.5 |
|  |  |  |  |  | TDLC300-100 | -2.4 |
|  |  |  | 24 | 6 | TDLA30-10 | 7.3 |
|  |  |  |  |  | TDLC300-100 | 0.5 |
|  |  |  | 32 | 8 | TDLA30-10 | 6.0 |
|  |  |  |  |  | TDLC300-100 | -0.3 |
|  |  |  | 48 | 12 | TDLA30-10 | 4.6 |
|  |  |  |  |  | TDLC300-100 | -1.4 |

Table 2 shows our initial simulation result of WUS reception for FDD SCS=15kHz, where we assumed OOK receiver. Although the WF assumes 64 bits for the number bits after Manchester coding (line coding), we have also run other values, i.e., 24, 32, and 48 bits.

From the simulation results for Mwus=1, the required SNRs to achieve 1% MDR are very low, less than -7dB. We propose to set 24 bits for the number of bits after line coding and set TDLA30-10 for channel model considering the relatively higher SNR test point.

Proposal 2: For Mwus=1, set 24 bits for the number of bits after line coding, and set TDLA30-10 for channel model.

Regarding Mwus=4, the required SNR for 1% MDR is 3.5dB and -2.4dB for TDLA30-10 and TDLC300-100, respectively, when the number of bits after line coding is set to 64. We think the SNR is high enough. One concern on this setting is that this configuration requires 16 OFDM symbols, which requires two slots. Snice we prefer to transmit WUS signal within one slot, we propose to set the number of bits after line coding to 32 bits corresponding to 12 OFDM symbols. Regarding the channel model, we propose using TDLC300-100, because we propose using TDLA30-10 for Mwus=1. This configuration can ensure the reception performance with TDLA30-10 and TDLC300-100.

Proposal 3: For Mwus=4, set 32 bits for the number of bits after line coding, and set TDLC300-100 for channel model.

FR2 related parameters

Issue 2-1-12: Carrier frequency Proposals - Option 1: Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD as starting point - FFS for FR2 TDD Issue 2-1-13: Antenna configuration Agreement: - 1T1R for FR1 - FFS: 1T1R for FR2

According to RAN4 RF discussion, RAN4 agreed that LP-WUS feature is applicable for all the FR2 bands except for n263 [2]. On the other hand, RAN4 also assumed 24.3GHz as an example band for FR1 [3]. We therefore propose to assume 24300 MHz.

Observation 1: RAN4 RF assumes n258 as an example band for LP-WUS in FR2.

Proposal 4: For FR2 TDD, assume 24300 MHz as the carrier frequency.

Regarding the FR2 WUR antenna configuration, RAN4 RF set receiver characteristics requirements as follows [2]:

Unless otherwise specified, the requirements apply to all polarizations of DL. The minimum requirements on sensitivity apply to two measurements, corresponding to DL signals in orthogonal polarizations.

This is the same assumption as FR2 NR MR receiver characteristics requirements. We propose to configure 1T2R for FR2, following RF assumption.

Observation 2: RAN4 RF sets FR2 WUR receiver characteristics requirements with 2Rx in TS 38.101-2.

Proposal 5: For FR2 TDD, assume 1T2R for antenna configuration.

Receiver assumption

Issue 2-1-21: Whether to define single requirement for Type 1 and Type 2 receivers Proposals - Option 1: RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined. - Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

RAN4 RF agreed to define two sets of WUR REFSENS requirements in FR1: Type1 LR and Type 2 LR [4]. On the other hand, in FR2, RAN4 RF defined WUR REFSENS requirement only for Type 2 LR (Ref A.4M.1 in [3]). We propose evaluating the WUS detection performance with both the receiver types according to the RF agreements.

Observation 3: RAN4 RF has defined two WUS receiver types for FR1: OOK-WUR and OFDM-WUR.

Proposal 6: Study the performance of both receiver types (Type 1: OOK-WUR and Type 2: OFDM-WUR) for FR1.

Observation 4: RAN4 RF has defined one receiver type for FR2: OFDM-WUR.

Proposal 7: Assume Type 2 (OFDM-WUR) for WUS demodulation requirements in FR2.

Considering the OFDM-WUR receiver, we also need to configure the number of overlaid OFDM sequence per chip to carry information, denoted as L1 in TS 38.212 7.4.2.2. When this parameter is more than one, we can transmit information bits by using different Zadoff-Chu (ZC) sequence as specified in TS 38.211 7.4.4.1.1. According to TS 38.331 (lpwus-OverlaidSeqNum-r19), L1 = {1, 2, 4, 8, 16} for FR1 Mwus=1 and L1 = {1, 2, 4} for FR1 Mwus=4. For FR2, L1 = {1, 2}.

Since RF RMC configures L1 as follows (Ref: Table A.3M.1-2 in [3]), we propose to reuse these configurations for demodulation requirements.

- L1=4 for FR1 SCS=15kHz and 30kHz (Ref Table A.3M.1-1/2 in [4])

- L1=2 for FR2 SCS=120kHz.

Proposal 8: Configure the number of overlaid OFDM sequence per chip to carry information (L1, lpwus-OverlaidSeqNum-r19) to 4 for FR1 SCS=15/30kHz and to 2 for FR2 SCS=120kHz.

TS 38.211 7.4.4.1.1 also requires configuring ‘root sequence number’ q to generate the ZC sequences, where q∈{1,…,NZC-1}. RAN1 has agreed to use two root sequence numbers, configured by RRC signaling (roor1/root2). We propose configuring these parameters to avoid different configurations by TE vendors.

Proposal 9: Configure the root sequence numbers q’s (root1 and root2 in TS 38.331) to generate the ZC sequences. For simplicity, configure root1=1 and root2=1.

# 3	Summary

Observations:

Observation 1: RAN4 RF assumes n258 as an example band for LP-WUS in FR2.

Observation 2: RAN4 RF sets FR2 WUR receiver characteristics requirements with 2Rx in TS 38.101-2.

Observation 3: RAN4 RF has defined two WUS receiver types for FR1: OOK-WUR and OFDM-WUR.

Observation 4: RAN4 RF has defined one receiver type for FR2: OFDM-WUR.

Proposals:

Proposal 1: RAN4 does not specify the Wake-up signal demodulation requirements with false alarm rate (FAR).

Proposal 2: For Mwus=1, set 24 bits for the number of bits after line coding, and set TDLA30-10 for channel model.

Proposal 3: For Mwus=4, set 32 bits for the number of bits after line coding, and set TDLC300-100 for channel model.

Proposal 4: For FR2 TDD, assume 24300 MHz as the carrier frequency.

Proposal 5: For FR2 TDD, assume 1T2R for antenna configuration.

Proposal 6: Study the performance of both receiver types (Type 1: OOK-WUR and Type 2: OFDM-WUR) for FR1.

Proposal 7: Assume Type 2 (OFDM-WUR) for WUS demodulation requirements in FR2.

Proposal 8: Configure the number of overlaid OFDM sequence per chip to carry information (L1, lpwus-OverlaidSeqNum-r19) to 4 for FR1 SCS=15/30kHz and to 2 for FR2 SCS=120kHz.

Proposal 9: Configure the root sequence numbers q’s (root1 and root2 in TS 38.331) to generate the ZC sequences. For simplicity, configure root1=1 and root2=1.

# References

- R4-2512526, “WF for [116][330] NR_LPWUS_Demod”, Moderator (Qualcomm).

- R4-2511858, “dCR to 38.101-2, LPWUS feature CR”, Qualcomm.

- R4-2511873, “TP to TR 38.774 on FR2 UE RF of LP-WUR”, Samsung.

- R4-2511852, “Big CR for LP-WUS UE RF requirements in Rel-19”, vivo, Huawei, HiSilicon, Qualcomm, Ericsson, Nokia, Murata, CATT, ZTE Corporation, Sanechip, Sony, CMCC, Samsung, LG Electronics, OPPO, Apple.