3GPP TSG RAN WG4 Meeting #116bis	 															R4-2513989

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.12.2.2

Source: 	Ericsson

Title:	On RRM performance aspects and test cases for MIMO Ph 5

Document for:	Discussion

# Introduction

In RAN4#116 the following WF for RRM requirements regarding the performance part of the WI NR MIMO Phase 5 was approved [1]. The performance part was listed as topic #2.

… < Topic #2: RRM performance requirements for Rel-19 MIMO > Sub-topic 2-1: Performance requirements for UE-initiated/event-driven beam management #### Issue 2-1-1: Test scope <Way forward > QC; Apple; Samsung; CMCC; Huawei; ZTE; Nokia; Ericsson; MediaTek. High level: Define TCs to verify UEIBM; Y; Y; Y; Y; Y; Y; Y; Y; Y. Whether to cover Event-2/Event-1/Event-7?; Event-2: Y Event-1/Event-7: FFS; Event-2: Y Event-1/Event-7: No; Event-2: Y Event-1: limited TC Event-7: No; All; Event-2 FFS on others; All; Event-2; Event-2&7; Event-2. Whether to cover Mode A and Mode B?; Either A or B; Both; Both; Both; Mode A&B as Test configurations. Can be skip depends on UE capability; Mode A&B as Test configurations. Can be skip depends on UE capability. Event-trigger for Intra-cell BM for SSB; Y; N; Y; Y; Y; Y; Y; Y; Y. Event-trigger for Intra-cell BM for CSI-RS; Y; Y; Y; Y; Y; Y; Y; Y; Y. Event-trigger for Inter-cell BM for SSB; Y; Y; Y; Y; FFS; Y; Y; ?; Y. Event-trigger for Inter-cell BM for CSI-RS; N; N; N; Y; Y; N; N. Whether to cover no time window and eventInstanceCount-r19 > 1?; N; N; Y; Y; Y; Y; Y. Frequency Range – FR2?; Y. No FR1; Y. FR2; Y. FR2. Beam switching delay for TCI states switching; Y Both known and unknown TCI; N; FFS on core req.; FFS on core req.; No MAC CE-based TCI state switching FFS on DCI-based TCI state switching. L1 measurement reporting delay; Y. CMI; N. #### Issue 2-1-3: Whether to add requirement to verify the performance of CMI? <Way forward > - Option 1: (Xiaomi) - RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report. - RAN4 to discuss how to define performance metric for event triggered reporting - Option 2: - Others are not precluded ### Sub-topic 2-2: Performance requirements for CJTC reporting #### Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? Agreement: - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P. #### Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? Agreement: - RAN4 to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, further discuss the accuracy and SNR in the performance part. #### Issue 2-2-3: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Nokia; Ericsson; MTK. Whether to define TCs to verify CJTC-Dd, measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-f，measurement period; Y; FFS; Y; Y; Y; Y; Y. Whether to define TCs to verify CJTC-P， (To be verify?); N; N; N. Whether to define TCs to verify scheduling restriction; Y (if core req.). Whether to define TCs to verify CJTC-Dd, CJTC-f，accuracy; Y; Y. ### Sub-topic 2-3: Performance requirements for enhancement for Asymmetric DL sTRP / UL mTRP #### Issue 2-3-1: Test scope <Way forward > High level:; QC; Apple; Samsung; CMCC; Huawei; Nokia; MTK. Whether to define TCs to verify timing requirements?; Y; Y; Y; Y; Y. Whether to define TCs to verify TCI state switching requirements?; Y; N. FR1 and/or FR2?; both. Whether to cover Scenario#1 and Scenario#2?; Both for FR1. Scenario#2 for FR2; Both; Both; Scenarios#1; both; Scenarios#1. …

In this contribution we discuss and state proposals related to the performance part of the WF.

# Discussion

## CJTC measurement performance requirements

### Reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P (Issue 2-2-1)

The agreement that RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P is fine, but we need all the quantization steps defined by RAN1. The defined number of quantization steps are based on RAN1 and RAN4 analytical calculations and simulations.

Observation 1: We are not against report mapping as such but believe that we need all the quantization steps defined by RAN1. The defined number of quantization steps are based on RAN1 and RAN4 analytical calculations and simulations.

Proposal 1: Any defined report table should reflect all quantization steps.

### Accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P (Issue 2-2-2)

In Ericsson’s contribution to RAN4#116 [2], we had the following proposal which we repeat here:

Proposal 2: For defining measurement accuracy for delay and frequency offset, we suggest a 50° total phase error budget for CJT coherent combining, with 30° allocated to UE measurement error and the remainder to other contributing factors.

Frequency offset Δf could be derived as measured relative change of phase (∆φslot) over a slot duration Δtslot.

Δφslot=360⋅Δf⋅Δtslot

Δf= ∆φslot360⋅Δtslot

However, it is a general challenge to accurate measure small Δf over a short Δtslot since UE has to be able to measure small ∆φslot  e.g. with Δf= 2 Hz and Δtslot = 1 ms ⇒ ∆φslot= 0,72 degrees. This is shown in Figure 1.

Assume such measurement cannot measure ∆φslot  better than within  ± 15 degrees ⇒ error in Δ𝑓 ~ ± 42 Hz. Averaging of such single measurements would improve to some extent but not compensate for single measurement constraints.

Figure 1: Frequency offset measurement based on change in delta phase (∆φslot) from TRS measurements within a slot

One notes that FO estimation errors cause incorrect compensation between regular periodic phase reports/compensations, assuming a Phase Coherency Time (PCT) of 30 ms in total for such regulation loop would mean that a frequency offset compensation error of  28 Hz will result in large phase misalignment (~300 degrees in the end of the PCT).

Observation 2: It is a general challenge to accurately measure small frequency offset Δf over a slot duration Δtslot.

Observation 3: One note that FO estimation errors cause incorrect compensation between regular periodic phase reports/compensations, assuming a Phase Coherency Time (PCT) of 30 ms in total for such regulation loop would mean that a large frequency offset compensation error will result in large phase misalignment at the end of the PCT.

We can work with the assumed 30° UE phase error allocation. The issue is the short measurement period. If also adding and measure change of relative phase over longer period than a slot, i.e., a TRS period (ΔφTRSperiod= 20 ms), then assuming still cannot measure   ΔφTRSperiod better than ± 15 degrees, like before, it would benefit from the longer observation period and improve result ⇒ error in Δf= ∆φslot360⋅ΔφTRSperiod=15 degrees360 degrees⋅20 ms ≈ ± 2.1 Hz.

This means that we recover the 2 Hz frequency offset error (we started with) for a much larger allowed UE phase error. This is shown in Figure 2.

Figure 2: Frequency offset measurement based on change in delta phase (ΔφTRSperiod) during a TRS period

The challenge would be to detect and compensate for potential integer phase wrap arounds occurring for larger frequency offsets during such longer observation time , also using coarse measurements over the shorter slot duration could be one potential approach for such detection and compensation.

Observation 4: A longer observation period (ΔφTRSperiod=20 ms) improves frequency offset accuracy to ± 2.1 Hz for the same assumed phase inaccuracy in the measurement (± 15° = 30° total error).

Proposal 3: Assume longer observation periods of at least a TRS period when executing the frequency offset measurement and derive the error from that.

## Asymmetric DL sTRP/UL mTRP

In the core part of the requirements, we specified the requirements for following two scenarios.

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

Core requirements specified timing requirements for the Scenarios 1 and 2. In this section we discuss the need of the test cases for them.

Timing requirements related test cases:

In NR, RAN4 has introduced the following tests to verify the timing requirements for the UE:

- UE Transmit Timing Test

- Timing Advance Adjustment Accuracy

In this section, we examine the purpose and methodology of these tests in the legacy context to determine their relevance for Asymmetric DL sTRP/UL mTRP.

UE Transmit Timing Test:

Purpose and Method: The purpose of this test is to verify that the UE can follow the frame timing changes of the connected gNB. It ensures that the UE's initial transmit timing accuracy, the maximum amount of timing change in one adjustment, and the minimum and maximum adjustment rates are within the specified limits. This test verifies the requirements outlined in clause 7.1.2.

Applicability to Asymmetric DL sTRP/UL mTRP: Although the UE supports two Timing Advances (TAs) for PUCCH/PUSCH/SRS, only a single DL reference timing is supported. For the UL-only TRP case, this test does not add significant value, as the UE only needs to track the frame timing changes of the reference TRP.

Timing Advance Adjustment Accuracy:

Purpose and Method: Timing advance commands are sent to the UE and Sounding Reference Signals (SRS) are transmitted from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be assessed.

Applicability to Asymmetric DL sTRP/UL mTRP: We believe that timing advance adjustment accuracy can be tested for this feature. However, there is nothing particularly unique about UE supporting UL-only TRP in this context of the test we will be verifying. Therefore, we do not see significant benefits in conducting this test specifically for UL-only TRP.

From a timing requirements perspective, Scenario 2 aligns with Rel-18 multi-TRP featuring two TAs. Some UEs may support Rel-19 Asymmetric DL sTRP/UL mTRP but not Rel-18 multi-TRP with two TAs. Therefore, we propose that the timing test cases agreed upon in Rel-18 be applied to Rel-19 Asymmetric DL sTRP/UL mTRP for Scenario 2. If a UE supports both features, testing either one should be sufficient, as the timing tests are essentially equivalent. Below are the test cases introduced in Rel-18 for timing requirements, which we believe are reasonable to specify for Scenario 2 in Rel-19 as well.

| TC: FR1 UE transmit timing from two TRPs. | A.4.x for EN-DC A.6.4.x for SA |
| --- | --- |
| TC: FR2 UE transmit timing from two TRPs. | A.5.4.x for EN-DC A.7.4.x for SA |

Proposal 4:

- For Scenario 2, we propose to specify following test cases for Rel-19 Asymmetric DL sTRP/UL mTRP. If the UE supports both Rel-18 two TA and Rel-19 Asymmetric DL sTRP/UL mTRP features, testing one should be sufficient.

  - TC: FR1 UE transmit timing from two TRPs.

  - TC: FR2 UE transmit timing from two TRPs.

- Scenario 1 does not introduce new testable functionality; hence, no new test cases are needed.

# Summary

The following have been proposed in this contribution.

Observation 1: We are not against report mapping as such but believe that we need all the quantization steps defined by RAN1. The defined number of quantization steps are based on RAN1 and RAN4 analytical calculations and simulations.

Proposal 1: Any defined report table should reflect all quantization steps.

Proposal 2: For defining measurement accuracy for delay and frequency offset, we suggest a 50° total phase error budget for CJT coherent combining, with 30° allocated to UE measurement error and the remainder to other contributing factors.

Observation 2: It is a general challenge to accurately measure small frequency offset Δf over a slot duration Δtslot.

Observation 3: One note that FO estimation errors cause incorrect compensation between regular periodic phase reports/compensations, assuming a Phase Coherency Time (PCT) of 30 ms in total for such regulation loop would mean that a large frequency offset compensation error will result in large phase misalignment at the end of the PCT.

Observation 4: A longer observation period (ΔφTRSperiod=20 ms) improves frequency offset accuracy to ± 2.1 Hz for the same assumed phase inaccuracy in the measurement (± 15° = 30° total error).

Proposal 3: Assume longer observation periods of at least a TRS period when executing the frequency offset measurement and derive the error from that.

Proposal 4:

- For Scenario 2, we propose to specify following test cases for Rel-19 Asymmetric DL sTRP/UL mTRP. If the UE supports both Rel-18 two TA and Rel-19 Asymmetric DL sTRP/UL mTRP features, testing one should be sufficient.

  - TC: FR1 UE transmit timing from two TRPs.

  - TC: FR2 UE transmit timing from two TRPs.

- Scenario 1 does not introduce new testable functionality; hence, no new test cases are needed.

# References

[1] 		R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[2]		R4-2510771, On RRM performance aspects and test cases for MIMO ph5. Ericsson