3GPP TSG-RAN WG4 Meeting #116bis		R4-2513993
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	Discussion on R20 low NR band carrier aggregation via switching

Source:	OPPO

Agenda item:	7.8.3

Document for:	Approval

Introduction

In Rel-20, the enhancement of low NR band carrier aggregation via switching was approved in [1]. This paper shares our view on this aspect.

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Discussion

In Rel-19, it had introduced the switching between {case 1, case 2} to enable low band carrier aggregation via switching between FDD band and SDL band.

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

The essence of Rel-19 switching between case 1 and case 2 is the switching between FDD band and SDL band. In Rel-20, switching between case 1 and case 3 is to switch between FDD band and FDD DL+SDL band.

Figure 1 Compared case 3 to case 2

As shown the example CA_n12A-n29A in Figure 1, at the time of T1, normal FDD band works on its only UL/DL; at the time of T2, case 2 and case 3 all only have DL carrier(s). therefore, there is no impact of Tx to Rx in case 2 and case 3.

There is no impact of Tx to Rx in both of case 2 and case 3.

In Rel-20, band n12 DL and n29 DL need receive simultaneously. It’s the same for another example band combinations CA_n28A-n67A. As shown in Figure 2, the spectrum of band n12 DL is nearby the spectrum of SDL band n29,the spectrum of band n28 DL is  adjacent with the spectrum of SDL band n67, therefore, one wider filter can be adopt to combine FDD DL spectrum with SDL spectrum for CA_n12A-n29A and CA_n28A-n67A, and it’s not needed to consider the in-gap interference due to there is no gap or only 1MHz gap between two DL carriers:

Figure 2 The spectrum of band n12 + band n29 and band n28+band n67

One wider filter can be adopted to combine FDD DL spectrum with SDL spectrum for CA_n12A-n29A and CA_n28A-n67A, and it’s not needed to consider the in-gap interference due to there is no gap or only 1MHz gap between two DL carriers.

Using one wider filter cover two bands DL spectrums for CA_n12A-n29A and CA_n28A-n67A.

Figure 3 The possible UE implementation for the switching between {case 1, case 3} and between {case 1, case 2}

Compared the possible UE implementation for the switching between {case 1, case 3} and between {case 1, case 2} as shown in Figure 3, the only different is the filter in the receiver for SDL or for FDD DL+ SDL.

The only difference of UE implementation between Rel-19 low-band CA via switching and R20 low-band CA via switching for CA_n12A-n29A and CA_n28A-n67A is the filter in the receiver for SDL or for FDD DL+ SDL.

R20 low-band CA via switching can reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching.

Since FDD DL and SDL in case 3 share one RF chain in the receiver, and there is no impact of Tx to Rx, the REFSENs of FDD band need re-evaluate based on new receiver architecture.

The REFSENs of FDD band in case 3 need re-evaluate based on new receiver architecture.

Conclusion

This paper discussed the switching between {case 1, case 3}, and got below observations and proposals.

Observation 1: 	 There is no impact of Tx to Rx in both of case 2 and case 3.

Observation 2: 	One wider filter can be adopted to combine FDD DL spectrum with SDL spectrum for CA_n12A-n29A and CA_n28A-n67A, and it’s not needed to consider the in-gap interference due to there is no gap or only 1MHz gap between two DL carriers.

Proposal 1: 	Using one wider filter cover two bands DL spectrums for CA_n12A-n29A and CA_n28A-n67A.

Observation 3: 	The only difference of UE implementation between Rel-19 low-band CA via switching and R20 low-band CA via switching for CA_n12A-n29A and CA_n28A-n67A is the filter in the receiver for SDL or for FDD DL+ SDL.

Proposal 2: 	R20 low-band CA via switching can reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching.

Proposal 3: 	The REFSENs of FDD band in case 3 need re-evaluate based on new receiver architecture.

Reference

RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, RAN #109.