3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514030

Prague, CZ, 13th – 17th October 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for (e)RedCap enh

# Agenda item:	7.4.3.1

# Document for:	Approval

# 1 Introduction

In RAN#109 meeting, a new Work Item [1] was approved on (e)RedCap UE enhancement with the following RAN4 objective. In this contribution, we want to share some initial views about (e)RedCap enhancement from UE RF requirements perspective.

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

# 2 Discussion

To support less than 5MHz channel bandwidth for (e)RedCap UEs, the following requirements have some impact.

  - 2.1 Channel bandwidth for (e)RedCap

Regarding section 5.3I Channel bandwidth for (e)RedCap, it specifies that 3MHz CBW is not applicable for (e)RedCap UE in the current release. In Rel-20 spec, this wording needs to be deleted.

Proposal 1: Delete “3MHz CBW is not applicable for (e)RedCap UE in the current release” in Rel-20 spec.

  - 2.2 Reference sensitivity for (e)RedCap

To support less than 5MHz channel bandwidth for (e)RedCap UEs, Reference sensitivity needs to be defined for HD-FDD with 1Rx and 2Rx.

REFSENS for HD-FDD with 2Rx

For HD-FDD (e)RedCap UE with 2Rx, there are two principles to define REFSENS for 3MHz.

Principle 1:

The first is to use similar tightening principle for HD-FDD (e)RedCap UE with 2Rx, in which REFSENS for HD-FDD is a little more tightened than FDD due to lack of Tx self-interference and less insertion loss in the front-end architecture. The rule of tightening value for 5MHz is shown in the following table.

Table 2.2-1: NR HD-FDD 5MHz REFSENS tightening for different NR bands

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

In our understanding, same tightening value can be reused for 3MHz. Based on the REFSENS for FDD bands n100, n106, n26, n28 and n85 as shown in Table 2.2-2, the tightening value is 0 dB for bands n28, n100 and 0.5 dB for bands n26, n85, n106 respectively. Therefore, REFSENS can be defined as in Table 2.2-3 for HD-FDD with 2Rx for 3MHz channel bandwidth.

Table 2.2-2: 3MHz REFSENS for FDD bands with 2Rx

| Operating Band | SCS kHz | 3 MHz (dBm) |
| --- | --- | --- |
| n26 | 15 | -99.7 |
| n28 | 15 | -100.2 |
| n85 | 15 | -99.2 |
| n100 | 15 | -102.2 |
| n106 | 15 | -99.2 |

Table 2.2-3: 3MHz REFSENS for HD-FDD bands with 2Rx

| Operating Band | SCS kHz | 3 MHz (dBm) |
| --- | --- | --- |
| n26 | 15 | -100.2 |
| n28 | 15 | -100.2 |
| n85 | 15 | -99.7 |
| n100 | 15 | -102.2 |
| n106 | 15 | -99.7 |

Principle 2:

The second is to scale from 5MHz REFSENS for HD-FDD (e)RedCap UE with 2Rx. REFSENS for 3MHz non-(e)RedCap UE is scaled from 5MHz REFSENS. The same scaling factor can be used to derive REFSENS for HD-FDD (e)RedCap UE, and it will be -1.7dB for band n28 and -2.2dB for bands n26, n85, n100. Based on the 5MHz REFSENS for FDD bands n26, n28 and n85 as shown in Table 2.2-4, REFSENS can be defined as in Table 2.2-5 for HD-FDD with 2Rx for 3MHz channel bandwidth. For bands n100 and n106, (e)RedCap is not supported before Rel-20. Regarding 3MHz REFSENS for bands n100 and n106, we make the proposals as in Table 2.2-5.

Table 2.2-4: 5MHz REFSENS for HD-FDD bands with 2Rx

| Operating Band | SCS kHz | 5 MHz (dBm) |
| --- | --- | --- |
| n26 | 15 | -98.3 |
| n28 | 15 | -99.3 |
| n85 | 15 | -97.8 |

Table 2.2-5: 3MHz REFSENS for HD-FDD bands with 2Rx

| Operating Band | SCS kHz | 3 MHz (dBm) |
| --- | --- | --- |
| n26 | 15 | -100.5 |
| n28 | 15 | -101 |
| n85 | 15 | -101.9 |
| n100 | 15 | -102.2 |
| n106 | 15 | -101.9 |

Proposal 2: We give two principles to derive REFSENS for 3MHz for HD-FDD (e)RedCap UE with 2Rx:

Principle 1: to use similar tightening principle for 5MHz for HD-FDD (e)RedCap UE with 2Rx, REFSENS can be defined as in Table 2.2-3 for HD-FDD with 2Rx for 3MHz channel bandwidth.

Principle 2: to scale from 5MHz REFSENS for HD-FDD (e)RedCap UE with 2Rx, REFSENS can be defined as in Table 2.2-5 for HD-FDD with 2Rx for 3MHz channel bandwidth.

REFSENS for HD-FDD with 1Rx

Regarding REFSENS for FDD with 1Rx, the following allowance was used compared with REFSENS for 2Rx. The 2.5 dB allowance for 5MHz comes from two parts: one is single Rx will need 3 dB more SNR than a 2Rx diversity receiver and the other is compromise of 0.5 dB tightening of the implementation margin for single Rx. For 3MHz REFSENS for HD-FDD with 1Rx, we propose to use 2.5dB allowance as a starting point.

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Proposal 3: For 3MHz REFSENS for HD-FDD with 1Rx, we propose to use 2.5dB allowance as a starting point.

  - 2.3 UE capabilities

During Rel-18 less than 5MHz discussion, some capabilities were introduced, and all specified that “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” . Additionally, in the WID, it states that “Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)”. Hence, only 15PRB transmission bandwidth configuration related capabilities will be supported in (e)RedCap, which is shown in the following:

support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only (except for the PRACH formats with 1.25kHz SCS). It applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band.

support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only (except for the PRACH formats with 1.25kHz SCS). It is applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL.

To support less than 5MHz channel bandwidth for (e)RedCap UEs, the following capabilities need to be updated:

support3MHz-ChannelBW-Asymmetric-r18

support3MHz-ChannelBW-Symmetric-r18

Proposal 4: To support less than 5MHz channel bandwidth for (e)RedCap UEs, the following capabilities need to be updated:

support3MHz-ChannelBW-Asymmetric-r18

support3MHz-ChannelBW-Symmetric-r18

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: Delete “3MHz CBW is not applicable for (e)RedCap UE in the current release” in Rel-20 spec.

Proposal 2: We give two principles to derive REFSENS for 3MHz for HD-FDD (e)RedCap UE with 2Rx:

Principle 1: to use similar tightening principle for 5MHz for HD-FDD (e)RedCap UE with 2Rx, REFSENS can be defined as in Table 2.2-3 for HD-FDD with 2Rx for 3MHz channel bandwidth.

Table 2.2-3: 3MHz REFSENS for HD-FDD bands with 2Rx

| Operating Band | SCS kHz | 3 MHz (dBm) |
| --- | --- | --- |
| n26 | 15 | -100.2 |
| n28 | 15 | -100.2 |
| n85 | 15 | -99.7 |
| n100 | 15 | -102.2 |
| n106 | 15 | -99.7 |

Principle 2: to scale from 5MHz REFSENS for HD-FDD (e)RedCap UE with 2Rx, REFSENS can be defined as in Table 2.2-5 for HD-FDD with 2Rx for 3MHz channel bandwidth.

Table 2.2-5: 3MHz REFSENS for HD-FDD bands with 2Rx

| Operating Band | SCS kHz | 3 MHz (dBm) |
| --- | --- | --- |
| n26 | 15 | -100.5 |
| n28 | 15 | -101 |
| n85 | 15 | -101.9 |
| n100 | 15 | -102.2 |
| n106 | 15 | -101.9 |

Proposal 3: For 3MHz REFSENS for HD-FDD with 1Rx, we propose to use 2.5dB allowance as a starting point.

Proposal 4: To support less than 5MHz channel bandwidth for (e)RedCap UEs, the following capabilities need to be updated:

support3MHz-ChannelBW-Asymmetric-r18

support3MHz-ChannelBW-Symmetric-r18

# 4 Reference

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple