3GPP TSG-RAN4 Meeting # 116bis	R4-2514031

Prague, CZ, 13th – 17th, Oct. 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on IoT-NTN PC1.5 single Tx

# Agenda item:	7.6.3

# Document for:	Approval

# 1 Introduction

In RAN#109 meeting, a new Work Item [1] was approved on introduction of HPUE for NTN with the following RAN4 objective. In this contribution, we want to share some initial views about IoT-NTN PC1.5 single Tx from UE RF requirements perspective.

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

# 2 Discussion

In Rel-19, PC1 and PC2 with 1Tx were introduced for NB-IoT based IoT-NTN for both handheld and non-handheld UEs. For feasibility, GSM PA and satellite PA e.g. Iridium system were tested and analyzed, and it concluded that PC1 1Tx can achieved. For PC1.5 with 1Tx for NB-IoT based IoT-NTN, same analysis can be reused.

  - 2.1 Tx requirements

Maximum output power

In the scope of this WID, the first thing is to introduce IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types in bands 256 and 255. For max output power for IoT-NTN PC1.5, it is proposed to reuse nominal MOP with +2/-3 tolerance.

Proposal 1: It is proposed to reuse nominal MOP with +2/-3 tolerance for IoT-NTN PC1.5.

MPR/A-MPR

During Rel-19 discussion, the following MPR/A-MPR assumption was agreed for PC1 and PC2 for NB-IoT.

- Agreement: - [3] dB post-PA insertion losses for calibration purpose - For PC2, PC3 NR PA can also be considered for reference architecture - Calibration point for PC2 (PC3 NR PA) (further update based on offline common view): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 dBc - CIM3: 60 dBc - NR ACLR: 30dBc, other number is not precluded - Calibration based on NB-IOT signal is not precluded. - For PC1, consider adding [-34] dBc IQ-image and LO-leakage and 60 dBc CIM3 as another option for evaluation - Calibration points for PC1 (based on NR PC2 PA) (further update based on offline common view): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 / [-34]dBc - CIM3: 60 dBc - NR ACLR: 31dBc, other number is not precluded - Calibration based on NB-IOT signal is not precluded.

Regarding MPR/A-MPR assumptions for IoT-NTN PC1.5 single Tx, we proposed to use a average value with 31dBc for IQ image.

Proposal 2: It is proposed to use the following MPR/A-MPR assumptions for IoT-NTN PC1.5 single Tx:

- Calibration point for PC1.5 (PC2 NR PA):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 31 dBc

      - CIM3: 60 dBc

      - NR ACLR: 31dB

ACLR

During Rel-19 IoT-NTN HPUE discussion, ACLR requirement was defined as 37 dB for PC1 and PC2 for both handheld and non-handheld UEs, which is reused UTRA ACLR. Since 37 dB ACLR can ensure the coexistence for PC1, for a lower power class, 37 dB ACLR also can be used. Therefore, we propose to reuse 37dB UTRA ACLR for IoT-NTN PC1.5 for both handheld and non-handheld UEs.

Proposal 3: It is proposed to reuse 37 dB UTRA ACLR for IoT-NTN PC1.5 for both handheld and non-handheld UEs.

  - 2.2 Rx requirements

REFSENS

For PC1 and PC2, no RSD is defined for NB-IoT based IoT-NTN since NB-IoT is half-duplex and Tx leakage would not impact Rx. With the same logic reused for PC1.5, it is proposed that no RSD needed for PC1.5 NB-IoT based IoT-NTN with 1Tx.

Proposal 4: No RSD needed for PC1.5 NB-IoT based IoT-NTN with 1Tx.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed to reuse nominal MOP with +2/-3 tolerance for IoT-NTN PC1.5.

Proposal 2: It is proposed to use the following MPR/A-MPR assumptions for IoT-NTN PC1.5 single Tx:

- Calibration point for PC1.5 (PC2 NR PA):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 31 dBc

      - CIM3: 60 dBc

      - NR ACLR: 31dB

Proposal 3: It is proposed to reuse 37 dB UTRA ACLR for IoT-NTN PC1.5 for both handheld and non-handheld UEs.

Proposal 4: No RSD needed for PC1.5 NB-IoT based IoT-NTN with 1Tx.

# 4 Reference

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)