3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514047

Prague, Czech Republic, 13rd – 17th October 2025

Source:	Nokia

Title:	Discussion on UE RF issues for US 4.9 GHz band

Agenda item:	5.11.4

Document for:	Approval

Introduction

RAN at #108 meeting agreed the WID on Introduction of NR TDD 4.9GHz band for US operation [1]. In RAN4#116, the initial study phase concluded that 3GPP would rely on the band manager described by FCC for the coexistence issues with incumbents and thus no specific 3GPP work would be required for them [2]. RAN4 is now expected to work on the core part of the WI in the following.

Following the completion of the study phase, the core objectives of this WI are:

-	Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz to support the following SCS and channel bandwidths:

| SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

- Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band.

- Address potential BS and UE co-existence issues, if any.

  - Specify band specific BS emissions requirements if needed

  - Specify UE additional emission limits and A-MPR if needed

- Specify the BS, repeater and IAB RF requirements.

- Specify the UE RF requirements for the new band. Reuse NR band n79 UE hardware assumption if possible.

- Update the related technical specifications to include support for the new band

- CRs shall not be approved until ongoing litigation is resolved to avoid specification rework and potential NBC changes

Note: The new NR band will be introduced in a REL-independent way starting from REL-15.

In this contribution, we discuss the UE RF aspects for the US 4.9GHz band for UE Power Class 3, as RAN considered that HPUE would be handled later in a basket WI [3].

Discussion

We submitted the summary of the existing regulatory requirements for US 4.9GHz band in RAN4#116 [4]. FCC Title 47 CFR §90.1213 specifies Band plan, §90.1215 Power limits and §90.210 Emission masks [5][6].

The center frequency, bandwidth and channel numbers are defined in §90.1213 (a). Aggregation up to 50 MHz bandwidth in any manner is allowed and therefore UE channel bandwidths listed in the WID (shown above) are aligned with regulations.

UE channel bandwidths in WID are consistent with regulations.

Power limits in §90.1215 (a)(1) specify the conducted output power (TRP) limit listed in Table 1.

Table 1: Maximum limits of transmitting power of stations operating in 4940-4990 MHz

| Channel bandwidth (MHz) | Low power maximum conducted output power (dBm) | High power maximum conducted output power (dBm) |
| --- | --- | --- |
| 1 | 7 | 20 |
| 5 | 14 | 27 |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

This TRP limit per channel bandwidth corresponds to PSD of 7 dBm/MHz for Low power and 20 dBm/MHz for High power for all the channel bandwidths.

Furthermore, a peak power spectrum density of 21 dBm/MHz and 8 dBm/MHz is specified separately in FCC Power limits for High power and Low power, respectively, in §90.1215 (a) and (b).

As specified in §90.1215 (a) and (b), High power (and Low power) devices using channel bandwidths other than those listed above are permitted; however, they are limited to peak power spectral density of 21 dBm/MHz (and 8 dBm/MHz). Therefore, TRP limit for 25 MHz channel bandwidth is derived only from the peak spectral density and is summarized in Table 2.

Table 2: Maximum transmitter power derived from the peak power spectral density

| Channel bandwidth (MHz) | Low power maximum conducted output power (dBm) | High power maximum conducted output power (dBm) |
| --- | --- | --- |
| 25 | 22.98 | 34.98 |

PSD limit 20 dBm/MHz for the entire channel bandwidth and 21 dBm/MHz for the peak density shall be applied to High power requirement for 10/20/30/40/50 MHz channel bandwidth.

PSD limit 7 dBm/MHz for the entire channel bandwidth and 8 dBm/MHz for the peak density shall be applied to Low power requirement for 10/20/30/40/50 MHz channel bandwidth.

For 25 MHz channel bandwidth, 8 dBm/MHz and 21 dBm/MHz peak spectrum density shall be applied to Low power and High power requirement, respectively.

UE power class 3 in 3GPP is 23 dBm as TRP for the entire channel bandwidth. PSD limit 7 dBm/MHz for the Low power requirement is restrictive for channel bandwidth 30 MHz or less when the uplink PRB allocation is narrower than the channel bandwidth. In particular, the maximum output power for an edge PRB allocation would be quite limited if 7 dBm/MHz is applied. Therefore, it is proposed to prioritize studying the PSD limit 21 dBm/MHz for UE power class 3.

It is proposed to confirm that TRP 23 dBm/channel bandwidth (MHz) and PSD 21 dBm/MHz are applied to UE power class 3.

EIRP limit is also applicable in FCC Power limits. If antenna gain is more than 9 dBi, the excess must be reduced from the conducted power limit. As high gain antenna is not considered for UE Power Class 3, the power reduction due to EIRP limit is not required.

Power reduction due to EIRP limit does not need to be taken into account for UE power class 3.

As WID [1] suggests that we reused NR band n79 UE hardware assumption, if possible, it is proposed that UE Power Class 3 is defined with the maximum output power 23 dBm with +2/-3 dB tolerance, reused from n79.

UE maximum output power for PC3 is 23 dBm with +2/-3 dB tolerance, reused from n79.

Emission mask M is applicable for high power transmitter (greater than 20 dBm) operating in 4940-4990 MHz frequency band. The power spectral density of the emissions must be attenuated below the output power of the transmitter as follows:

(1) On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.

(2) On any frequency removed from the assigned frequency between 45-50% of the authorized bandwidth: 568 log (% of (BW)/45) dB.

(3) On any frequency removed from the assigned frequency between 50-55% of the authorized bandwidth: 26 + 145 log (% of BW/50) dB.

(4) On any frequency removed from the assigned frequency between 55-100% of the authorized bandwidth: 32 + 31 log (% of (BW)/55) dB.

(5) On any frequency removed from the assigned frequency between 100-150% of the authorized bandwidth: 40 + 57 log (% of (BW)/100) dB.

(6) On any frequency removed from the assigned frequency between above 150% of the authorized bandwidth: 50 dB or 55 + 10 log (P) dB, whichever is the lesser attenuation.

(7) The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. The power spectral density is the power measured within the resolution bandwidth of the measurement device divided by the resolution bandwidth of the measurement device. Emission levels are also based on the use of measurement instrumentation employing a resolution bandwidth of at least one percent of the occupied bandwidth.

The resolution bandwidth is at least 1% of the bandwidth in FCC which are consistent with 3GPP spectrum emission masks specified in TS 38.101-1 clause 6.5.2.2, except for 50 MHz channel bandwidth. 30 kHz is used for the first 1 MHz in 3GPP for 50 MHz channel bandwidth.

Figure 1 shows the comparison of FCC masks with 3GPP masks (SEM and Spurious (for above BWchannel+5MHz)) when the requirement is scaled in dBm/MHz for 10/20/40/50 MHz channel bandwidths. FCC masks are more stringent than 3GPP spectrum emission masks for out-of-band. Further, the mask is defined also in-band, i.e., within the channel bandwidth.

Figure 1: Comparison of FCC and 3GPP emission masks for PC3

Emission masks in FCC for US 4.9 GHz band are more stringent than 3GPP general UE emissions masks for out-of-band emissions. Further, the FCC mask is also appliable within the channel bandwidth.

The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission and a video bandwidth of 30 kHz. To avoid the mask from being unnecessarily stringent, it is proposed to assume the channel bandwidth to measure the 0 dB reference.

0 dB reference of the emission mask is defined with TRP 23 dBm/channel bandwidth (MHz) for UE power class 3.

FCC mask is 0 dB for 0-45% of the authorized bandwidth and then attenuation is already required for 45-50% within the authorized bandwidth. 3GPP does not have a mask for the outer 5% bandwidth within the channel bandwidth. It is proposed to discuss how to evaluate this mask for A-MPR simulations.

It is proposed to discuss how to evaluate the mask for outer 5% bandwidth within the channel bandwidth.

One possible way to handle PSD limits and emissions masks together is to apply A-MPR. Therefore, it is proposed to study if we can incorporate both FCC PSD limits and Emission masks in Network Signaling mechanism such that A-MPR is applied to meet these requirements.

It is proposed to study if we can incorporate FCC PSD limits and Emission masks in Network Signaling mechanism such that A-MPR is applied to meet these requirements.

Conclusion

The following Observations and Proposals were made:

Observation 1: UE channel bandwidths in WID are consistent with regulations.

Observation 2: PSD limit 20 dBm/MHz for the entire channel bandwidth and 21 dBm/MHz for the peak density shall be applied to High power requirement for 10/20/30/40/50 MHz channel bandwidth.

Observation 3: PSD limit 7 dBm/MHz for the entire channel bandwidth and 8 dBm/MHz for the peak density shall be applied to Low power requirement for 10/20/30/40/50 MHz channel bandwidth.

Observation 4: For 25 MHz channel bandwidth, 8 dBm/MHz and 21 dBm/MHz peak spectrum density shall be applied to Low power and High power requirement, respectively.

Proposal 1: It is proposed to confirm that TRP 23 dBm/channel bandwidth (MHz) and PSD 21 dBm/MHz are applied to UE power class 3.

Observation 5: Power reduction due to EIRP limit does not need to be taken into account for UE power class 3.

Proposal 2: UE maximum output power for PC3 is 23 dBm with +2/-3 dB tolerance, reused from n79.

Observation 6: Emission masks in FCC for US 4.9 GHz band are more stringent than 3GPP general UE emissions masks for out-of-band emissions. Further, the FCC mask is also appliable within the channel bandwidth.

Proposal 3: 0 dB reference of the emission mask is defined with TRP 23 dBm/channel bandwidth (MHz) for UE power class 3.

Proposal 4: It is proposed to discuss how to evaluate the mask for outer 5% bandwidth within the channel bandwidth.

Proposal 5: It is proposed to study if we can incorporate FCC PSD limits and Emission masks in Network Signaling mechanism such that A-MPR is applied to meet these requirements.

- References

- RP-252929 Revised WID on Introduction of NR TDD 4.9GHz band for US operation, Fujitsu, RAN#109

- RP-252913 Status report for Introduction of NR TDD 4.9GHz band for US operation, Fujitsu, RAN#109

- Draft meeting report RAN #109

- R4-2509418 On 4940-4990 MHz band regulations for adjacent spectrum protection, Nokia, RAN4#116

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I